3GPP TSG-RAN WG3 #98	Tdoc R3-174782

Reno, Nevada, USA, 27 November - 01 December 2017

Agenda Item:	10.10.1.6

Source:	Ericsson

Title:	Content for UE mobility command messages

Document for:	pCR TS 38.473

# Introduction

This TP provides the changes needed to 38.473 to enable the solution described in R3-174781 [1].

# Discussion

During RAN3 #97bis the inter-gNB-DU mobility procedure was discussed and a new class 1 procedure was introduced, namely the UE Mobility Command procedure. In R3-174781 [1], we discuss in more detail the inter-gNB-DU handover procedure agreed in TS 38.401 V0.4.0 and make concrete proposals on identified issues. In this contribution we propose new IEs for the UE Mobility Command message and UE Mobility Command Acknowledge message in order to enable the solution proposed.

In the UE Mobility Command Acknowledge message, we propose the addition of the element PDU Delivery Status Transfer and provide also its definition. This IE will provide information about the downlink PDCP SN status during handover.

In Annex I we provide a text proposal for TS 38.473 to reflect our proposals above.

Proposal 1	RAN3 is kindly to agree with the TP in Annex I.

# Conclusion

In this contribution, we analysed the additions to the handover procedure. We conclude with the following proposal:

Proposal 1		RAN3 is kindly asked to agree with the TP in Annex I.

# References

[1]	R3-174781, Mobility procedures, Ericsson

# Annex I: TP for TS 38.473

Start of Text Proposal for TS 38.473

# 8	Overall procedures in gNB-CU/gNB-DU Architecture

*************** START OF MODIFIED SECTION***********

### 8.3.6	UE Mobility Command procedure

#### 8.3.6.1	General

The purpose of the UE Mobility Command procedure is to trigger inter-gNB-DU mobility procedure. The procedure uses UE associated signalling.

#### 8.3.6.2	Successful Operation

Figure 8.3.6.2-1: Handover Command procedure. Successful operation.

The gNB-CU initiates the procedure by sending a UE MOBILITY COMMAND message to gNB-DU. The gNB-DU responds with a UE MOBILITY COMMAND ACKNOWLEDGE message.

The gNB-CU may include the Handover Type IE in the UE MOBILITY COMMAND message. If the gNB-DU receives the Handover Type IE, it may take this information into account for further signalling decisions.

The gNB-DU may include the PDU Delivery Status Transfer IE in the UE MOBILITY COMMAND ACKNOWLEDGE message. If the gNB-CU receives the PDU Delivery Status Transfer IE, it may take this information into account for further signalling decisions.

#### 8.3.6.3	Abnormal Conditions

Not applicable.

***************END OF MODIFIED SECTION***************

### 9.2.2 	UE Context Management messages

*************** START OF MODIFIED SECTION***********

#### 9.2.2.12	UE MOBILITY COMMAND

This message is sent by the gNB-CU to trigger inter-gNB-DU mobility operation.

Direction: gNB-CU  gNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| RRC-Container | O |  | 9.3.1.6 |  |  |  |
| Handover Type | O |  | ENUMERATED (intragNBCU, intergNBCU, …) | Indicates if the handover is inter-gNB-CU or intra gNB-CU. |  |  |

#### 9.2.2.13	UE MOBILITY COMMAND ACKNOWLEDGE

This message is sent by the gNB-DU to gNB-CU that the UE MOBILITY COMMAND message has been successfully sent out.

Direction: gNB-DU  gNB-CU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| PDU Delivery Status Transfer | O |  | 9.3.1.X |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |

Editor’s note: Additional IE is FFS

***************END OF MODIFIED SECTION***************

## 9.3	Information Element Definitions

*************** START OF MODIFIED SECTION***********

9.3.1.X PDU Delivery Status Transfer

This element indicates to the gNB-CU the downlink PDCP SN status during a handover.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| DRBs Subject To Status Transfer List | 1 | 1.. <maxnoof Bearers> |  |  |
| >>DRB ID | M |  | 9.3.1.8 |  |
| >>DL Highest successfully delivered PDCP Sequence Number | M |  |  | The highest PDCP PDU sequence number successfully delivered in sequence to the UE among those PDCP PDUs received from the source gNB-CU. |

***************END OF MODIFIED SECTION***************

End of Text Proposal for TS 38.473