3GPP TSG RAN WG3 Meeting #98	Tdoc R3-174788

Reno, Nevada, USA, 27 November - 01 December 2017

Agenda Item:	10.10.2.2

Source:	Ericsson

Title:	Monitoring of inactivity

Document for:	pCR TS 38.473

# Introduction

RAN3 is currently working on the F1 Application Protocol (F1AP). The F1AP procedures that are needed for option 3 are expected to be finalized for the early drop of Release 15. In this contribution, we discuss a functionality that is currently missing in the F1AP specifications and that we believe is needed for option 3, as well as for other connectivity options, namely the monitoring of inactivity.

# Discussion

In LTE, the eNB constantly monitors the UE activity on a per-bearer basis. The UE activity monitoring is typically performed by the lower-layers using dedicated counters. The reason for monitoring in the lower-layers is that it allows for measurements with very fine granularity, as the lower-layers have detailed knowledge of the UE activity over the air interface (i.e., on a per-TTI basis). The lower-layers measurements can also be used for UE observability. The lower-layers inform the higher-layers whenever a bearer has been inactive for a given amount of time. The higher-layers can then take actions accordingly. For example, the higher-layers could decide to remove a bearer that has been inactive for a given amount of time or it can decide to change the UE state to RRC_IDLE (or suspend the UE connection) in case that all the corresponding bearers have been inactive for a given amount of time.

A similar functionality is needed also for NR radio, where according to the information from the lower-layers, the higher-layers can decide to remove data radio bearers (DRBs) and in NG-RAN the corresponding QoS-flows, or it can decide to change the UE state to RRC_IDLE or RRC_INACTIVE.

Observation 1	Similar as in LTE, it is expected that in NR the lower-layers will monitor the UE activity on a per-bearer basis and report to the higher layers. This allows the higher-layers to take actions toward the UE e.g., removing bearers or changing UE state.

In the disaggregated gNB architecture, the (en-)gNB is separated into gNB-DU and gNB-CU. The lower-layers reside in the gNB-DU and the higher-layers reside in the gNB-CU. Therefore, it is expected that the gNB-DU monitors the UE activity on a per-bearer basis and reports to the gNB-CU either when a bearer or a UE (i.e., all the bearers for the UE) have been inactive for a given amount of time. Accordingly, the gNB-CU can take actions, such as removing unused bearers or changing the UE state.

Observation 2	In the disaggregated gNB architecture, the gNB-DU monitors the UE activity and reports to the gNB-CU whenever a bearer or a UE has been inactive for a certain time. The gNB-CU can decide to remove bearers or change the UE state accordingly.

Currently, in the F1AP specifications a mechanism that allows the gNB-DU to report UE activity to the gNB-CU is missing. We believe that such mechanism should be introduced as part of the early drop of F1AP specifications. There are two possible options for introducing this mechanism.

- Introduce a new F1AP procedure: This involves defining a new UE-associated and gNB-DU initiated procedure. The procedure could be class 2. The gNB-DU executes the procedure by sending a message to the gNB-CU including information on whether the UE is active or not-active. For example, the gNB-DU could declare that the UE is not-active when all its bearer have not been used for data transmission for a given amount of time. The message can also include information about activity per-bearer. The coding of the message could be as shown in Table 1.

Note: we use the term “Not-Active” to clearly differentiate with respect to the RRC_INACTIVE state.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| UE Activity |  |  | ENUMERATED (Active, Not-active) | The UE is considered Not-active if all its bearers have not been used for data transmission for a predefined amount of time. |  |  |
| DRB Activity List |  | 0.. 1 |  |  | YES | reject |
| >DRB Activity List Item |  | 1 .. <maxnoofDRBs> |  |  | EACH | ignore |
| >>DRB ID | M |  | 9.3.1.8 |  | - |  |
| >>DRB Activity | M |  | ENUMERATED (Active, Not-active) | The DRB is considered Not-active it has not been used for data transmission for a predefined amount of time. |  |  |

Table 1: Example of a new F1AP message to report inactivity from gNB-DU to gNB-CU.

Observation 3	One method to allow the gNB-DU to report the UE activity to the gNB-CU is to introduce a new UE-associated and gNB-DU initiated class 2 F1AP procedure.

- Employ the F1AP UE Context Modification Required procedure: Another option is to rely on the existing F1AP UE Context Modification Required procedure, which is a UE-associated and gNB-DU initiated procedure [1]. The UE Context Modification Required procedure is class 1. The gNB-DU initiates the procedure by sending the F1AP UE Context Modification Required messages, and the gNB-CU reports the successful update of the UE Context in the F1AP UE Context Modification Confirm message. The F1AP UE Context Modification Required message could be enhanced with a new IE that carries information about UE activity and optionally also information about DRBs activity (the structure can be similar as shown in Table 1).

Observation 4	Another method to allow the gNB-DU to report the UE activity to the gNB-CU is to introduce a new IE in the F1AP UE Context Modification Required message including information about UE and DRB activity.

In case that a UE is served by multiple gNB-DUs at the same time, each gNB-DU should monitor the UE activity on its own resources and report to the gNB-CU accordingly. For example, in case of split bearer each gNB-DU monitor only its own leg and reports inactivity to the gNB-CU about it. The gNB-CU can combine the reports from multiple gNB-DUs to take actions. For example, the gNB-CU can decide to change the UE state to RRC_IDLE only in case that all the gNB-DUs serving the UE report that the UE is not transmitting data for a given amount of time.

Observation 5	In case that a UE is served by multiple gNB-DUs, each gNB-DU report the UE activity only its own resources. The gNB-CU can combine the reports received by all the gNB-DUs serving the UE to take actions.

When comparing the two solutions above, solution 2 has the disadvantage that the gNB-CU always needs to send the F1AP UE Context Modification Confirm message in response to a UE activity indication from the gNB-DU. Even in case that the gNB-CU decides to take an action (e.g., removing a bearer) it should first send F1AP UE Context Modification Confirm message to the gNB-DU. This seems inefficient and consequently we prefer to rely on solution 1, i.e., introduce a new F1AP procedure to report UE activity from gNB-DU to gNB-CU.

Proposal 1	Introduce a new gNB-DU initiated, UE-associated, class 2 F1AP procedure to report UE activity from gNB-DU to gNB-CU.

Proposal 2	RAN3 is kindly asked to agree on the text proposal for TS 38.473 in Annex I.

# Conclusion

In this contribution, we discussed the monitoring of inactivity.

Proposal 1	Introduce a new gNB-DU initiated, UE-associated, class 2 F1AP procedure to report UE activity from gNB-DU to gNB-CU.

Proposal 2	RAN3 is kindly asked to agree on the text proposal for TS 38.473 in Annex I.

# References

3GPP TS 38.473 v0.4.0, F1 Application Protocol, October 2017.

# Annex 1: TP for TS 38.473

Start of Text Proposal for TS 38.473

Table 2: Class 2 procedures

| Elementary Procedure | Message |
| --- | --- |
| Error Indication | ERROR INDICATION |
| UE Context Release Request (gNB-DU initiated) | UE CONTEXT RELEASE REQUEST |
| Initial UL RRC Message Transfer | INITIAL UL RRC MESSAGE TRANSFER |
| DL RRC Message Transfer | DL RRC MESSAGE TRANSFER |
| UL RRC Message Transfer | UL RRC MESSAGE TRANSFER |
| System Information Delivery | SYSTEM INFORMATION DELIVERY COMMAND |
| Paging | PAGING |
| UE Activity Indication | UE ACTIVITY INDICATION |

***************Skip Unchanged Text***************

### 8.3.X	UE Activity Indication

#### 8.3.X.1	General

The purpose of the UE Activity Indication procedure is to enable the gNB-DU to send indication about the UE activity to the gNB-CU. The procedure uses UE-associated signalling.

#### 8.3.5.2	Successful Operation

UE   ACTIVITY INDICATION

gNB

-

DU

gNB

-

CU

Figure 8.3.X.2-1: UE Activity Indication procedure.

The gNB-DU executes the procedure by sending the UE ACTVITIY INDICATION message to the gNB-CU. The gNB-CU may use this indication to perform reconfigurations.

In case that an UE is served by multiple gNB-DUs at the same time, the gNB-CU may combine the indications received by all the gNB-DUs serving the UE to perform reconfigurations.

If the DRB Activity List Item IE is contained in the UE ACTVITIY INDICATION message, the gNB-CU may use this information to decide whether to perform reconfigurations, such as removing bearers.

#### 8.3.X.3	Abnormal Conditions

Not applicable.

***************Skip Unchanged Text***************

#### 9.2.2.X	UE ACTIVITY INDICATION

This message is sent by the gNB-DU to provide information about the UE activity to the gNB-CU.

Direction: gNB-DU  gNB-CU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| UE Activity |  |  | ENUMERATED (Active, Not-active) | The UE is considered Not-active if all its bearers have not been used for data transmission for a predefined amount of time. |  |  |
| DRB Activity List |  | 0.. 1 |  |  | YES | reject |
| >DRB Activity List Item |  | 1 .. <maxnoofDRBs> |  |  | EACH | ignore |
| >>DRB ID | M |  | 9.3.1.8 |  | - |  |
| >>DRB Activity | M |  | ENUMERATED (Active, Not-active) | The DRB is considered Not-active it has not been used for data transmission for a predefined amount of time. |  |  |

End of Text Proposal for TS 38.473