3GPP TSG-RAN WG3 #98	Tdoc R3-174789

Reno, Nevada, USA, 27 November - 01 December 2017

Agenda Item:	10.10.2.2

Source:	Ericsson

Title:	RRC Container in UE Context Setup Request

Document for:	pCR TS 38.473

# Introduction

This text proposal provides the changes needed to TS 38.473 to enable performing step 9 in the UE Initial Access as shown in TS 38.401 V0.4.1 [1].

# Discussion

In TS 38.401 v0.4.1 in the UE Initial Access Procedure at step 9 it is stated that: “The gNB-CU sends UE CONTEXT SETUP REQUEST message to establish overall initial UE context in the gNB-DU. This message may also encapsulate the RRC SECURITY MODE COMMAND message.”

To enable this, an RRC container needs to be included in the F1AP UE CONTEXT SETUP REQUEST message. In the following we provide a text proposal for TS 38.473 to introduce this RRC container.

Proposal 1	RAN3 is kindly to agree with the TP in Annex I.

# Conclusion

In this contribution, we analysed the addition of an RRC container to the F1AP UE CONTEXT SETUP REQUEST message to carry RRC Security Mode Command. We conclude with the following proposal:

Proposal 1		RAN3 is kindly asked to agree with the TP in Annex I.

# References

[1]	TS 38.401 v0.4.1, NG-RAN Architecture Description.

# Annex I: TP for TS 38.473

Start of Text Proposal for TS 38.473

## 9.2	Message Functional Definition and Content

### 9.2.2 	UE Context Management messages

#### 9.2.2.1	UE CONTEXT SETUP REQUEST

This message is sent by the gNB-CU to request the setup of a UE context.

Direction: gNB-CU  gNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID (FFS) | O |  | 9.3.1.5 |  | FFS | FFS |
| C-RNTI (FFS) | O |  | FFS |  |  |  |
| Target cell ID (FFS) | FFS |  | FFS |  | FFS | FFS |
| SRB to Be Setup List |  |  |  |  |  |  |
| >SRB to Be Setup Item IEs |  | 1 .. <maxnoofSRBs> [FFS] |  |  |  |  |
| >SRB ID |  |  | 9.3.1.7 |  |  |  |
| DRB to Be Setup List |  | 1 |  |  | YES | reject |
| >DRB to Be Setup Item IEs |  | 1 .. <maxnoofDRBs> [FFS] |  |  | EACH | reject |
| >>DRB ID | M |  | 9.3.1.8 |  | - |  |
| >>DRB QoS profile (FFS) |  |  |  |  | - |  |
| >>Flow-level QoS profile (FFS) |  |  |  |  |  |  |
| >>Flow-to-DRB mapping (FFS) |  |  |  |  |  |  |
| >>E-UTRAN QoS | O |  |  | Used for EN-DC case to convey E-RAB Level QoS Parameters |  |  |
| >> Tunnels to be setup List (FFS) |  | 1 |  |  |  |  |
| >>>Tunnels to Be Setup Item IEs |  | 1 .. <maxnoofULTunnels> |  |  |  |  |
| >>>>UL GTP Tunnel Endpoint | M |  | GTP Tunnel Endpoint 9.3.2.1 | gNB-CU endpoint of the F1 transport bearer. For delivery of UL PDUs. | - | - |
| RRC-Container | M |  | 9.3.1.6 |  | YES | ignore |

Editor’s note: for intra-DU CA PDCP duplication, two F1-U tunnels can be set up to distinguish duplicated PDCP PDUs belonging to the same DRB. It is FFS whether multiple tunnels are needed or if a single tunnel is enough.

Editor’s Note: The presence of APID/CRNTI/SRB/DRB depends on if this message is used to set up the UE associated logical F1 connection. It is also FFS whether gNB-DU UE F1 AP ID will be included. The naming of the GTP tunnel endpoint is FFS.

| Range bound | Explanation |
| --- | --- |
| maxnoofSRBs | Maximum no. of SRB allowed towards one UE, the maximum value is FFS. |
| maxnoofDRBs | Maximum no. of DRB allowed towards one UE, the maximum value is FFS. |
| <maxnoofULTunnels> | Maximum no. of tunnels allowed towards one DRB, the maximum value is 2. |

End of Text Proposal for TS 38.473