3GPP TSG RAN WG3 Meeting #97b	Tdoc R3-174790

Prague, Czech Republic, 09-13 October 2017

Agenda Item:	10.10.2.2

Source:	Ericsson

Title:	RLC mode indication

Document for:	pCR 38.473

# Introduction

In the disaggregated gNB architecture, the gNB-CU decides which data radio bearers (DRBs) to setup and the QoS-flows to DRBs mapping. The gNB-CU should then inform the gNB-DU in the F1AP UE Context Setup Request (or F1AP UE Context Modification Request).

The gNB-CU can decide to serve an UE via multiple gNB-DUs using dual- or multi-connectivity. It can also decide to split a DRB among multiple gNB-DUs. In this case, it is very important that all the gNB-DUs that provide a radio leg for the same split DRB utilize the same RLC mode. Otherwise, there could be issues related for example on how to handle flow-control.

The gNB-DUs could autonomously derive the RLC mode to use for a DRB based on the QoS information received from the gNB-CU. However, there is no standardized way of mapping the QoS parameters to the RLC mode. Different gNB-DU implementations could potentially choose different RLC modes even if they receive the same QoS parameters.

To avoid this problem, the gNB-CU should signal to the gNB-DU the RLC mode to use for each DRB in the F1AP UE Context Setup Request and F1AP UE Context Modification Request messages.

Proposal 1	Include the RLC mode for each DRB in the F1AP UE Context Setup Request and F1AP UE Context Modification Request messages.

Proposal 2	RAN3 is kindly asked to agree with the text proposal in Annex I.

# Conclusion

In this contribution, we discussed how to select the RLC mode.

Proposal 1	Include the RLC mode for each DRB in the F1AP UE Context Setup Request and F1AP UE Context Modification Request messages.

Proposal 2	RAN3 is kindly asked to agree with the text proposal in Annex I.

# Annex I: TP for TS 38.473

Start of Text Proposal for TS 38.473

#### 9.2.2.1	UE CONTEXT SETUP REQUEST

This message is sent by the gNB-CU to request the setup of a UE context.

Direction: gNB-CU  gNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID (FFS) | O |  | 9.3.1.5 |  | FFS | FFS |
| C-RNTI (FFS) | O |  | FFS |  |  |  |
| Target cell ID (FFS) | FFS |  | FFS |  | FFS | FFS |
| SRB to Be Setup List |  |  |  |  |  |  |
| >SRB to Be Setup Item IEs |  | 1 .. <maxnoofSRBs> [FFS] |  |  |  |  |
| >SRB ID |  |  | 9.3.1.7 |  |  |  |
| DRB to Be Setup List |  | 1 |  |  | YES | reject |
| >DRB to Be Setup Item IEs |  | 1 .. <maxnoofDRBs> [FFS] |  |  | EACH | reject |
| >>DRB ID | M |  | 9.3.1.8 |  | - |  |
| >> RLC Mode | M |  | 9.3.1.x |  | - | - |
| >>DRB QoS profile (FFS) |  |  |  |  | - |  |
| >>Flow-level QoS profile (FFS) |  |  |  |  |  |  |
| >>Flow-to-DRB mapping (FFS) |  |  |  |  |  |  |
| >>E-UTRAN QoS | O |  |  | Used for EN-DC case to convey E-RAB Level QoS Parameters |  |  |
| >> Tunnels to be setup List (FFS) |  | 1 |  |  |  |  |
| >>>Tunnels to Be Setup Item IEs |  | 1 .. <maxnoofULTunnels> |  |  |  |  |
| >>>>UL GTP Tunnel Endpoint | M |  | GTP Tunnel Endpoint 9.3.2.1 | gNB-CU endpoint of the F1 transport bearer. For delivery of UL PDUs. | - | - |

***************Skip Unchanged Text***************

#### 9.2.2.7	UE CONTEXT MODIFICATION REQUEST

This message is sent by the gNB-CU to provide UE Context information changes to the gNB-DU.

Direction: gNB-CU  gNB-DU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| SRB to Be Added List |  |  |  |  |  |  |
| DRB to Be Added List |  |  |  |  | YES | reject |
| >DRB to Be Added Item IEs |  | 1 .. <maxnoofDRBs> [FFS] |  |  | EACH | reject |
| >>DRB ID | M |  | 9.3.1.8 |  | - |  |
| >>RLC Mode | M |  | 9.3.1.x |  | - | - |
| DRB to Be Modified List |  |  |  |  | YES | reject |
| DRB to Be Released List |  |  |  |  | YES | reject |

***************Skip Unchanged Text***************

#### 9.3.1.x RLC Mode

The RLC Mode IE indicates the RLC Mode used for a DRB.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| RLC Mode |  |  | ENUMERATED ( RLC-AM, RLC-UM,…) |  |

***************Skip Unchanged Text***************

### 9.4.4	PDU Definitions

-- **************************************************************

--

-- PDU definitions for F1AP.

--

-- **************************************************************

F1AP-PDU-Contents {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) f1ap (3) version1 (1) f1ap-PDU-Contents (1) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- **************************************************************

--

-- IE parameter types from other modules.

--

-- **************************************************************

IMPORTS

Cause,

CriticalityDiagnostics,

GNB-CU-F1AP-ID,

GNB-DU-F1AP-ID,

TimeToWait,

UE-associatedLogicalF1-ConnectionItem,

RRCContainer,

SRBID,

RLC-Mode

FROM F1AP-IEs

PrivateIE-Container{},

ProtocolExtensionContainer{},

ProtocolIE-Container{},

ProtocolIE-ContainerList{},

ProtocolIE-ContainerPair{},

ProtocolIE-ContainerPairList{},

ProtocolIE-SingleContainer{},

F1AP-PRIVATE-IES,

F1AP-PROTOCOL-EXTENSION,

F1AP-PROTOCOL-IES,

F1AP-PROTOCOL-IES-PAIR

FROM F1AP-Containers

id-Cause,

id-CriticalityDiagnostics,

id-gNB-CU-F1AP-ID,

id-gNB-DU-F1AP-ID,

id-ResetType,

id-TimeToWait,

id-UE-associatedLogicalF1-ConnectionItem,

id-UE-associatedLogicalF1-ConnectionListResAck,

id-RRCContainer,

id-SRBID,

id-RLC-Mode,

maxnoofErrors,

maxnoofIndividualF1ConnectionsToReset

FROM F1AP-Constants;

[Skipped unchanged text]

-- **************************************************************

--

-- UE CONTEXT SETUP REQUEST

--

-- **************************************************************

UEContextSetupRequest ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { UEContextSetupRequestIEs} },

...

}

UEContextSetupRequestIEs F1AP-PROTOCOL-IES ::= {

...

{ ID id-RLC-Mode		CRITICALITY reject	TYPE RLC-Mode				PRESENCE mandatory},

}

-- **************************************************************

--

[Skipped unchanged text]

-- **************************************************************

--

-- UE CONTEXT MODIFICATION REQUEST

--

-- **************************************************************

UEContextModificationRequest ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { UEContextModificationRequestIEs} },

...

}

UEContextModificationRequestIEs F1AP-PROTOCOL-IES ::= {

...

{ ID id-RLC-Mode		CRITICALITY reject	TYPE RLC-Mode				PRESENCE mandatory},

}

[Skipped unchanged text]

### 9.4.5	Information Element Definitions

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

F1AP-IEs {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) f1ap (3) version1 (1) f1ap-IEs (2) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

maxnoofErrors,

maxnoofIndividualF1ConnectionsToReset

FROM F1AP-Constants

Criticality,

ProcedureCode,

ProtocolIE-ID,

TriggeringMessage

FROM F1AP-CommonDataTypes

ProtocolExtensionContainer{},

F1AP-PROTOCOL-EXTENSION,

ProtocolIE-SingleContainer{},

F1AP-PROTOCOL-IES

FROM F1AP-Containers;

-- A

[skipped unchanges text]

-- R

RRCContainer ::= OCTET STRING

RLC-Mode ::= ENUMERATED {

RLC-AM,

RLC-UM,

...

}

[skipped unchanges text]

### 9.4.7	Constant Definitions

-- **************************************************************

--

-- Constant definitions

--

-- **************************************************************

F1AP-Constants {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) f1ap (3) version1 (1) f1ap-Constants (4) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- **************************************************************

--

-- IE parameter types from other modules.

--

-- **************************************************************

IMPORTS

ProcedureCode,

ProtocolIE-ID

FROM F1AP-CommonDataTypes;

-- **************************************************************

--

-- Elementary Procedures

--

-- **************************************************************

id-Reset									ProcedureCode ::= 0

id-F1Setup									ProcedureCode ::= 1

id-ErrorIndication							ProcedureCode ::= 2

id-GNBDUConfigurationUpdate					ProcedureCode ::= 3

id-GNBCUConfigurationUpdate					ProcedureCode ::= 4

id-UEContextSetup							ProcedureCode ::= 5

id-UEContextRelease							ProcedureCode ::= 6

id-UEContextModification						ProcedureCode ::= 7

id-UEContextModificationRequired				ProcedureCode ::= 8

id-UEMobilityCommand							ProcedureCode ::= 9

id-UEContextReleaseRequest					ProcedureCode ::= 10

id-InitialULRRCMessageTransfer				ProcedureCode ::= 11

id-DLRRCMessageTransfer						ProcedureCode ::= 12

id-ULRRCMessageTransfer						ProcedureCode ::= 13

id-SystemInformationDelivery					ProcedureCode ::= 14

id-Paging									ProcedureCode ::= 15

-- **************************************************************

--

-- Extension constants

--

-- **************************************************************

maxPrivateIEs							INTEGER ::= 65535

maxProtocolExtensions					INTEGER ::= 65535

maxProtocolIEs							INTEGER ::= 65535

-- **************************************************************

--

-- Lists

--

-- **************************************************************

maxnoofErrors							INTEGER ::= 256

maxnoofIndividualF1ConnectionsToReset	INTEGER ::= 256

-- **************************************************************

--

-- IEs

--

-- **************************************************************

id-Cause											ProtocolIE-ID ::= 0

id-CriticalityDiagnostics							ProtocolIE-ID ::= 1

id-gNB-DU-F1AP-ID									ProtocolIE-ID ::= 2

id-gNB-CU-F1AP-ID									ProtocolIE-ID ::= 3

id-ResetType											ProtocolIE-ID ::= 4

id-TimeToWait										ProtocolIE-ID ::= 5

id-UE-associatedLogicalF1-ConnectionItem 			ProtocolIE-ID ::= 6

id-UE-associatedLogicalF1-ConnectionListResAck		ProtocolIE-ID ::= 7

id-RRCContainer										ProtocolIE-ID ::= 8

id-SRBID												ProtocolIE-ID ::= 9

id-RLC-Mode											ProtocolIE-ID ::= xx

END

End of Text Proposal for TS 38.473