3GPP TSG RAN WG3 Meeting #98	Tdoc R3-174900

Reno, Nevada, USA, 27 November - 01 December 2017

Agenda Item:	10.10.1.4

Source:	Ericsson

Title:	SI over F1

Document for:	pCR TS 38.473

# Introduction

This a text proposal for SI exchange over F1-C. It is based on the following:

CB: # 19_SysInfo_Xch

- concentrate on F1 Setup Req / DU Config Update

- capture agreement

- ref. to 38.331

- add appropriate semantics desc if needed

- modify purpose of Sys Info delivery (Sec. 8.5.1.1) from 4881 if agreeable

# Annex 1: TP for TS 38.473

Start of Text Proposal for TS 38.473

### 8.5.1	System Information Delivery

#### 8.5.1.1	General

The purpose of the System Information Delivery procedure is to command  the gNB-DU to broadcast the requested Other SI. The procedure uses non-UE associated signalling.

#### 8.5.1.2	Successful Operation

Figure 8.5.1.2-1: System Information Delivery procedure. Successful operation.

The gNB-CU initiates the procedure by sending a SYSTEM INFORMATION DELIVERY COMMAND message to the gNB-DU.

Upon reception of the SYSTEM INFORMATION DELIVERY COMMAND message, the gNB-DU shall broadcast the Other SI for the time interval indicated in the Broadcast Time Interval IE.

Editor’s Note: It is FFS whether Broadcast Time Interval IE is needed.

***************Skip Unchanged Text***************

#### 9.2.1.4	F1 SETUP REQUEST

This message is sent by the gNB-DU to transfer information for a TNL association.

Direction: gNB-DU  gNB-CU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-DU ID | M |  | 9.3.1.9 |  | YES | reject |
| gNB-DU Name (FFS) | O |  | FFS | Human readable gNB-DU name (FFS) | YES | reject |
| gNB-DU Served Cells List |  | 1 |  | List of cells configured in the gNB-DU | YES | reject |
| >gNB-DU Served Cells Item |  | 1.. <maxCellingNBDU> |  |  | EACH | reject |
| >>Served Cell Information | M |  | 9.3.1.10 | Information about the cells configured in the gNB-DU | - | - |
| >>gNB-DU System Information | M |  | 9.3.1.X | RRC container with system information owned by gNB-DU | - | - |

Editor’s note: The gNB-DU Name IE definition and whether it will be introduced is FFS.

***************Skip Unchanged Text***************

#### 9.2.1.7	GNB-DU CONFIGURATION UPDATE

This message is sent by the gNB-DU to transfer updated information for a TNL association.

Direction: gNB-DU  gNB-CU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Served Cells To Add List |  | 0.. 1 |  | Complete list of added cells served by the gNB-DU | YES | reject |
| >Served Cells To Add Item |  | 1 .. <maxCellingNBDU> |  |  | EACH | reject |
| >>Served Cell Information | M |  | 9.3.1.10 | Information about the cells configured in the gNB-DU | - | - |
| >>gNB-DU System Information | M |  | 9.3.1.X | RRC container with system information owned by gNB-DU | - | - |
| Served Cells To Modify List |  | 0..1 |  | Complete list of modified cells served by the gNB-DU | YES | reject |
| >Served Cells To Modify Item |  | 1 .. <maxCellingNBDU> |  |  | EACH | reject |
| >>Old NR Cell ID | M |  | FFS | NR cell identifier (FFS) | - | - |
| >>Served Cell Information | M |  | 9.3.1.10 | Information about the cells configured in the gNB-DU | - | - |
| >>gNB-DU System Information | O |  | 9.3.1.X | RRC container with system information owned by gNB-DU | - | - |
| Served Cells To Delete List |  | 0..1 |  | Complete list of deleted cells served by the gNB-DU | YES | reject |
| >Served Cells To Delete Item |  | 0 .. <maxCellingNBDU> |  |  | EACH | reject |
| >>Old NR Cell ID | M |  | FFS | NR cell identifier (FFS) | - | - |

| Range bound | Explanation |
| --- | --- |
| maxCellingNBDU | Maximum no. cells that can be served by an gNB-DU. Value is FFS. |

***************Skip Unchanged Text***************

#### 9.3.1.X	gNB-DU System Information

This IE contains the system information generated by the gNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| MIB message | M |  | OCTET STRING | MIB message, as defined in TS 38.331 [XX]. |
| SIB1 message | M |  | OCTET STRING | SIB1 message, as defined in TS 38.331 [XX]. |

End of Text Proposal for TS 38.473