3GPP TSG RAN WG3 Meeting #98	Tdoc R3-174929

Reno, NV, USA, 27th Nov- 1st Dec 2017

Agenda Item:	10.10.1.1

Source:	Ericsson

Title:	Transaction ID

Document for:	pCR TS 38.473

# Introduction

In this contribution, we try to clarify the use-cases and benefits for adding F1AP transaction IDs. this is based on the following CB:

CB: # 17_TransIDs

- clarify use case

- clarify benefits

- should we add them now?

# Discussion

We provide a brief discussion to clarify the benefits of introducing a transaction ID in the non-UE associated messages.

- Uniquely identifies a procedure among all ongoing parallel procedures of the same type initiated by the same protocol peer, without the need to decode other IEs in the same message. This allows for more efficient implementations, especially when handling large numbers of parallel procedures.

- Error handling is improved, provided that when reporting criticality diagnostics for the non-comprehended message, its Transaction ID is also reported. This greatly simplifies error debugging.

- Handling of abnormal conditions at interface setup is also greatly improved. In case of e.g., multiple F1 setup requests from the same initiating gNB-DU (e.g., due to errors and/or undelivered messages), the gNB-CU is able to distinguish between them according to their respective Transaction IDs.

- It may make link quality diagnostics more feasible e.g., by looking at delay and/or non-delivery ratio for F1AP messages belonging to the same procedure. This may be advantageous in case of non-ideal transport networks.

In the following, we also provide a TP for TS 38.473.

# TP for 38.473

Start of Text Proposal 1 for TS 38.473

#### 9.2.1.1	RESET

This message is sent by both the gNB-CU and the gNB-DU and is used to request that the F1 interface, or parts of the F1 interface, to be reset.

Direction: gNB-CU  gNB-DU and gNB-DU  gNB-CU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  | 9.3.1.X |  | YES | reject |
| Cause | M |  | 9.3.1.2 |  | YES | ignore |
| CHOICE Reset Type | M |  |  |  | YES | reject |
| >F1 interface |  |  |  |  |  |  |
| >>Reset All | M |  | ENUMERATED (Reset all,…) |  | - |  |
| >Part of F1 interface |  |  |  |  |  |  |
| >>UE-associated logical F1-connection list |  | 1 |  |  | - |  |
| >>>UE-associated logical F1-connection Item |  | 1 .. <maxnoofIndividualF1ConnectionsToReset> |  |  | EACH | reject |
| >>>> gNB-CU UE F1AP ID | O |  | 9.3.1.4 |  | - |  |
| >>>> gNB-DU UE F1AP ID | O |  | 9.3.1.5 |  | - |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofIndividualF1ConnectionsToReset | Maximum no. of UE-associated logical F1-connections allowed to reset in one message. Value is 256. |

#### 9.2.1.2	RESET ACKNOWLEDGE

This message is sent by both the gNB-CU and the gNB-DU as a response to a RESET message.

Direction: gNB-DU  gNB-CU and gNB-CU  gNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  | 9.3.1.X |  | YES | reject |
| UE-associated logical F1-connection list |  | 0..1 |  |  | YES | ignore |
| >UE-associated logical F1-connection Item |  | 1 .. <maxnoofIndividualF1ConnectionsToReset> |  |  | EACH | ignore |
| >>gNB-CU UE F1AP ID | O |  | 9.3.1.4 |  | - |  |
| >>gNB-DU UE F1AP ID | O |  | 9.3.1.5 |  | - |  |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofIndividualF1ConnectionsToReset | Maximum no. of UE-associated logical F1-connections allowed to reset in one message. Value is 256. |

#### 9.2.1.3	ERROR INDICATION

This message is sent by both the gNB-CU and the gNB-DU and is used to indicate that some error has been detected in the node.

Direction: gNB-CU  gNB-DU and gNB-DU  gNB-CU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| Transaction ID | M |  | 9.3.1.X |  | YES | reject |
| gNB-CU UE F1AP ID | O |  | 9.3.1.4 |  | YES | ignore |
| gNB-DU UE F1AP ID | O |  | 9.3.1.5 |  | YES | ignore |
| Cause | O |  | 9.3.1.2 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |

#### 9.2.1.4	F1 SETUP REQUEST

This message is sent by the gNB-DU to transfer information for a TNL association.

Direction: gNB-DU  gNB-CU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  | 9.3.1.X |  | YES | reject |
| gNB-DU ID | M |  | 9.3.1.9 |  | YES | reject |
| gNB-DU Name (FFS) | O |  | FFS | Human readable gNB-DU name (FFS) | YES | reject |
| gNB-DU Served Cells List |  | 1 |  | List of cells configured in the gNB-DU |  |  |
| >gNB-DU Served Cells Item |  | 1.. <maxCellingNBDU> |  |  | YES | reject |
| >>Served Cell Information | M |  | 9.3.1.10 | Information about the cells configured in the gNB-DU | - | - |
| >>gNB-DU System Information | M |  | FFS | RRC container with system information owned by gNB-DU | - | - |

Editor’s note: The gNB-DU Name IE definition and whether it will be introduced is FFS.

#### 9.2.1.5	F1 SETUP RESPONSE

This message is sent by the gNB-CU to transfer information for a TNL association.

Direction: gNB-CU  gNB-DU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  | 9.3.1.X |  | YES | reject |
| gNB ID (FFS) | M |  |  |  |  |  |
| Cells to be Activated List |  | 0.. 1 |  |  |  |  |
| >Cells to be Activated List Item |  | 1.. <maxCellingNBDU> |  | List of cells to be activated | YES | reject |
| >>NR Cell ID | M |  | FFS | NR cell identifier (FFS) | - | - |
| >>gNB-CU System Information | M |  |  | RRC container with system information owned by gNB-CU | - | - |
| >>Broadcast PLMN (FFS) |  |  |  |  |  |  |
| >>TAI (FFS) |  |  |  |  |  |  |

Editor’s note: it is FFS whether the NR Cells ID will be: (1) local cell ID (NCI without gNB ID), (2) NCI or (3) PCI.

Editor’s note: it is FFS whether the PLMN and TAI will be included (1) in the F1 Setup Request in the Served Cells Information IE, or (2) in the F1 Setup Response.

#### 9.2.1.6	F1 SETUP FAILURE

This message is sent by the gNB-CU to indicate F1 Setup failure.

Direction: gNB-CU  gNB-DU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  | 9.3.1.X |  | YES | reject |
| Cause | M |  | 9.3.1.2 |  | YES | ignore |
| Time to wait | O |  | FFS |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |

#### 9.2.1.7	GNB-DU CONFIGURATION UPDATE

This message is sent by the gNB-DU to transfer updated information for a TNL association.

Direction: gNB-DU  gNB-CU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  | 9.3.1.X |  | YES | reject |
| Served Cells To Add List |  | 1 |  | Complete list of added cells served by the gNB-DU | YES | reject |
| >Served Cells To Add Item |  | 1 .. <maxCellingNBDU> |  |  | EACH | reject |
| >>Served Cell Information | M |  | 9.3.1.10 | Information about the cells configured in the gNB-DU | - | - |
| >>gNB-DU System Information | M |  | FFS | RRC container with system information owned by gNB-DU | - | - |
| Served Cells To Modify List |  | 0..1 |  | Complete list of modified cells served by the gNB-DU | YES | reject |
| >Served Cells To Modify Item |  | 1 .. <maxCellingNBDU> |  |  | EACH | reject |
| >>Old NR Cell ID | M |  | FFS | NR cell identifier (FFS) | - | - |
| >>Served Cell Information | M |  | 9.3.1.10 | Information about the cells configured in the gNB-DU | - | - |
| >>gNB-DU System Information | M |  | FFS | RRC container with system information owned by gNB-DU | - | - |
| Served Cells To Delete List |  | 0..1 |  | Complete list of deleted cells served by the gNB-DU | YES | reject |
| >Served Cells To Delete Item |  | 0 .. <maxCellingNBDU> |  |  | EACH | reject |
| >>Old NR Cell ID | M |  | FFS | NR cell identifier (FFS) | - | - |

| Range bound | Explanation |
| --- | --- |
| maxCellingNBDU | Maximum no. cells that can be served by an gNB-DU. Value is FFS. |

#### 9.2.1.8	GNB-DU CONFIGURATION UPDATE ACKNOWLEDGE

This message is sent by a gNB-CU to a gNB-DU to acknowledge update of information for a TNL association.

Direction: gNB-CU  gNB-DU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  | 9.3.1.X |  | YES | reject |
| Cells to be Activated List |  | 0.. 1 |  | List of cells to be activated | YES | reject |
| >Cells to be Activated List Item |  | 1.. <maxCellingNBDU> |  |  | EACH | reject |
| >>NR Cell ID | M |  | FFS | NR cell identifier (FFS) | - | - |
| >> gNB-CU System Information | M |  | FFS | RRC container with system information owned by gNB-DU | - | - |
| >>Broadcast PLMN (FFS) |  |  |  |  |  |  |
| >>TAI (FFS) |  |  |  |  |  |  |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |

Editor’s note: it is FFS whether the NR Cells ID will be: (1) local cell ID (NCI without gNB ID), (2) NCI or (3) PCI.

Editor’s note: it is FFS whether the PLMN and TAI will be included (1) in the GNB-DU GONFIGURATION UPDATE in the Served Cells Information IE, or (2) in the GNB-DU GONFIGURATION UPDATE ACKNOWLEDGE.

#### 9.2.1.9	GNB-DU CONFIGURATION UPDATE FAILURE

This message is sent by the gNB-CU to indicate gNB-DU Configuration Update failure.

Direction: gNB-CU  gNB-DU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  | 9.3.1.X |  | YES | reject |
| Cause | M |  | 9.3.1.2 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |

#### 9.2.1.10	GNB-CU CONFIGURATION UPDATE

This message is sent by the gNB-CU to transfer updated information for a TNL association.

Direction: gNB-CU  gNB-DU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  | 9.3.1.X |  | YES | reject |
| Cells to be Activated List |  | 0..1 |  | List of cells to be activated | YES | reject |
| >Cells to be Activated List Item |  | 1.. <maxCellingNBDU> |  |  | EACH | reject |
| >>NR Cell ID | M |  | FFS | NR cell identifier (FFS) | - | - |
| >> gNB-CU System Information | M |  | FFS | RRC container with system information owned by gNB-DUCU |  |  |
| >>Broadcast PLMN (FFS) |  |  |  |  |  |  |
| >>TAI (FFS) |  |  |  |  |  |  |
| Cells to be Deactivated List |  | 0..1 |  | List of cells to be deactivated | YES | reject |
| >Cells to be Deactivated List Item |  | 1.. <maxCellingNBDU> |  |  | EACH | reject |
| >>NR Cell ID | M |  | FFS | NR cell identifier (FFS) | - | - |

#### 9.2.1.11	GNB-CU CONFIGURATION UPDATE ACKNOWLEDGE

This message is sent by a gNB-DU to a gNB-CU to acknowledge update of information for a TNL association.

Direction: gNB-DU  gNB-CU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  | 9.3.1.X |  | YES | reject |
| Cells Failed to be Activated List |  | 0..1 |  | List of cells which are failed to be activated | YES | reject |
| >Cells Failed to be Activated Item |  | 1.. <maxCellingNBDU> |  |  | EACH | reject |
| >>NR Cell ID | M |  | FFS | NR cell identifier (FFS) | - | - |
| >>Cause | M |  | 9.3.1.2 |  | - | - |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxCellingNBDU | Maximum no. cells that can be served by an gNB-DU. Value is FFS. |

Editor’s note: it is FFS whether the NR Cell ID will be: (1) local cell ID (NCI without gNB ID), (2) NCI or (3) PCI.

Editor’s note: it is FFS whether the PLMN and TAI will be included (1) in the GNB-CU GONFIGURATION UPDATE in the Served Cells Information IE, or (2) in the GNB-CU GONFIGURATION UPDATE ACKNOWLEDGE.

#### 9.2.1.12	GNB-CU CONFIGURATION UPDATE FAILURE

This message is sent by the gNB-DU to indicate gNB-CU Configuration Update failure.

Direction: gNB-DU  gNB-CU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  | 9.3.1.X |  | YES | reject |
| Cause | M |  | 9.3.1.2 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |

SKIP UNCHANGED TEXT

#### 9.3.1.X	Transaction ID

The Transaction ID IE uniquely identifies a procedure among all ongoing parallel procedures of the same type initiated by the same protocol peer

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Transaction ID | M |  | INTEGER (0..255, …) |  |

End of Text Proposal 1 for TS 38.473