3GPP TSG-RAN WG3 #98	                 R3-174986

Reno, Nevada, USA, 27 November - 1 December 2017

Source: 	AT&T

Title:	Way Forward on CB: # 58_ LTE-NR_Coex

Agenda Item:	10.14.2

Document for:	Discussion and Decision

# Introduction

RAN1 sent RAN3 an LS identifying required signalling to support DL/UL sharing on overlapping and adjacent spectrum [1]:

Agreements:

- For LTE-NR coexistence in overlapping spectrum,

  - Send an LS to RAN3 to specify the Xn interface and enhanced X2 interface messages that enable coordination between LTE and NR, including

    - LTE cell on/off configuration with details up to RAN3

    - LTE MBSFN subframe configuration

    - DL and/or UL carrier center frequency (ARFCN)

    - Carrier bandwidth

    - Signaling related to timing synchronization and SFN

      - Note: this does not require the network to be synchronized and/or SFN aligned and/or radio frame boundary aligned

      - Note: It is up to RAN3 if this requires new procedures in addition to signaling support

    - Indication of semi-statically used resources (to avoid collisions with, e.g., CSI-RS, SRS, PRACH, PUCCH, DRS, PSS/SSS, PBCH, …)

    - Indication of slots/PRBs not intended for transmissions by the eNB and gNB, respectively

- For LTE-NR coexistence in adjacent spectrum,

  - Send an LS to RAN3 to specify the Xn interface and enhanced X2 interface messages that enable coordination between LTE and NR, including

    - Signaling related to timing synchronization and SFN

      - Note: this does not require the network to be synchronized and/or SFN aligned and/or radio frame boundary aligned

      - Note: It is up to RAN3 if this requires new procedures in addition to signaling support

    - TDD UL/DL configuration in case of LTE and NR and special subframe configuration in case of LTE

The following was captured from the RAN3#98 chairman’s notes:

CB: # 58_ LTE-NR_Coex

-  WF

- capture proposals (if agreeable)

- include comments from companies (if agreeable)

- non-UE-associated signaling (if agreeable)

- St2 (if feasible)

- St3 (if feasible)

(AT&T)

This document summarizes discussion during RAN3#98 and proposes a way forward for LTE-NR Coexistence on overlapping and adjacent spectrum.

# Discussion

This section addresses working assumptions and some of the open issues identified in [2]:

Scenario 1 (DL): Signalling of scheduling resources that are not used for PDSCH: down-selection between MBSFN Subframe Info IE (enhanced semantics + additional parameter), ABS Pattern Info IE (enhanced semantics), RNTP IE and Enhanced RNTP IE (both with RNTP threshold set to -∞).

Proposal 1: Signalling for the DL sharing scenario (Scenario 1) can support both indication of time/frequency resources (e.g. FDM/TDM patterns). Detailed Stage 2/3 CRs on the signalling details are encouraged for the next meeting, e.g. whether/how to indicate LTE/NR RS configurations/patterns vs. time/frequency bitmap(s).

Scenario 2 (UL)

- Working assumption: Scheduling resources not used for PUSCH to be signalled based on FDM solution, e.g. similarly to UL High Interference Indication IE.

Proposal 2: Signalling for the UL sharing scenario (Scenario 2) can support both indication of time/frequency resources (e.g. FDM/TDM patterns). Detailed Stage 2/3 CRs on the signalling details are encouraged for the next meeting.

- Working assumption: PRB-level granularity for frequency resources

- Working assumption: Slot-level granularity for time resources

Scenario 3 (adjacent spectrum)

- Working assumption: Subframe Assignment and Special Subframe Info IEs may be used.

Proposal 3: Continue discussion in the next meeting on the working assumption for the adjacent spectrum scenario (Scenario 3), especially on the need for indicating a DL/UL switching pattern.

Stage 2 Open Issues:

- Should bi-directional procedure be introduced, i.e. both eNB and gNB can initiate the coordination?

Proposal 4: A bi-directional procedure is introduced, i.e. both eNB and gNB can initiate the coordination.

- Existing X2 signalling is per cell granularity – would frequency carrier (EARFCN) granularity be sufficient?

Proposal 5: Cell-level granularity is the basis for signalling on X2/Xn.

# Remaining Open Issues

This section lists remaining open issues:

Remaining open issues – stage 2:

- Generic resource set indications bitmaps with RB and OFDM symbol granularity as agreed for NR in RAN1 may also be applicable/beneficial for indication of semi-statically used resources (to avoid collisions with, e.g., SRS, PRACH, PUCCH, …)

- How to support signalling related to timing synchronization and SFN.

- Can RAN3 assume availability of common time-of-day in eNB and gNB?

- Shall RAN3 consider support of NB-IoT in mentioned scenarios?

Remaining open issues – stage 3:

- How to support DRS configuration? (note: DRS is not scheduled in MBSFN subframes (FFS))

- How to support CSI-RS configuration? (note: CSI-RS is not scheduled in MBSFN subframes (FFS))

- How to support PBCH configuration? (note: PBCH is not scheduled in MBSFN subframes)

- How to support SRS configuration?

- How to support PUCCH configuration?

# Summary

The following are proposals and working assumptions for LTE-NR Coexistence in overlapping and adjacent spectrum:

Proposal 1: Signalling for the DL sharing scenario (Scenario 1) can support both indication of time/frequency resources (e.g. FDM/TDM patterns). Detailed Stage 2/3 CRs on the signalling details are encouraged for the next meeting, e.g. whether/how to indicate LTE/NR RS configurations/patterns vs. time/frequency bitmap(s).

Proposal 2: Signalling for the UL sharing scenario (Scenario 2) can support both indication of time/frequency resources (e.g. FDM/TDM patterns). Detailed Stage 2/3 CRs on the signalling details are encouraged for the next meeting.

- Working assumption: PRB-level granularity for frequency resources

- Working assumption: Slot-level granularity for time resources

Proposal 3: Continue discussion in the next meeting on the working assumption for the adjacent spectrum scenario (Scenario 3), especially on the need for indicating a DL/UL switching pattern.

Proposal 4: A bi-directional procedure is introduced, i.e. both eNB and gNB can initiate the coordination.

Proposal 5: Cell-level granularity is the basis for signalling on X2/Xn.

The following are remaining open issues to be resolved:

Remaining open issues – stage 2:

- Generic resource set indications bitmaps with RB and OFDM symbol granularity as agreed for NR in RAN1 may also be applicable/beneficial for indication of semi-statically used resources (to avoid collisions with, e.g., SRS, PRACH, PUCCH, …)

- How to support signalling related to timing synchronization and SFN.

- Can RAN3 assume availability of common time-of-day in eNB and gNB?

- Shall RAN3 consider support of NB-IoT in mentioned scenarios?

Remaining open issues – stage 3:

- How to support DRS configuration? (note: DRS is not scheduled in MBSFN subframes (FFS))

- How to support CSI-RS configuration? (note: CSI-RS is not scheduled in MBSFN subframes (FFS))

- How to support PBCH configuration? (note: PBCH is not scheduled in MBSFN subframes)

- How to support SRS configuration?

- How to support PUCCH configuration?

# References

- R3-173397_R1-1715041, LS on RAN3 signaling support for LTE-NR coexistence feature, AT&T

- R3-174150, WF on LTE-NR Coexistence, Nokia, Nokia Shanghai Bell