3GPP TSG-RAN WG3 #97bis	R3-175031

Prague, Czech Republic, 9th – 13th October 2017

Agenda Item:	10.8.3.3

Source:	Ericsson, Intel Corporation

Title:	Discard the duplicated transmissions of PDCP PDUs

Document for:	pCR

# Introduction

In the past meetings, the handling of PDCP duplication had been discussed. This paper provides our view on the flow control handling when it relates to discarding the duplicated/redundant PDCP packages.

# Discussion

RAN2 has agreed that the same PDCP packages can be sent on all the legs to increase robustness. In most of the cases, the PDCP packages are transmitted smoothly, so UE is responsible to handle the received duplicated packages.

However it would be desirable that as soon as the UE has received data via either node, all other copies, which are redundant, were cleared from the queues in the other nodes, in order to make room for transmission of data that UE has yet not received.

In our opinion, there could be two ways of discarding the redundant PDUs. One is to “flush”, i.e. to discard all the PDCP PDUs in queue up to a given PDCP PDU SN. Another is to discard the PDCP PDUs between a start and a stop range, when non-contiguous PDCP PDU ranges are given.

Proposal 1: RAN3 to agree the above discard mechanism in FC.

# Conclusions and Proposals

Proposal 1: RAN3 to agree the above discard mechanism in FC.

# Text Proposal to TS 38.425 v0.2.0

<<<<<<<<<<<<<<<<<<<< Begin Text Proposal >>>>>>>>>>>>>>>>>>>>

## 5.2	Xn user plane protocol layer services

Editor’s Note: All the text below is FFS.

The following functions are provided by the Xn UP protocol for dual connectivity:

-	Provision of Xn UP specific sequence number information for user data transferred from the M-NG-RAN node to the S-NG-RAN node for a specific data bearer configured with the split bearer option;

-	Information of successful in sequence delivery of PDCP PDUs to the UE from S-NG-RAN node for user data associated with a specific data bearer configured with the split bearer option;

-	Information of PDCP PDUs that were not delivered to the UE;

-	Information of PDCP PDUs transmitted to the lower layers for user data associated with a specific data bearer configured with the split bearer option;

-	Information of downlink PDCP PDUs to be discarded for user data associated with a specific data bearer configured with downlink PDCP duplication;

-	Information of the currently desired buffer size at the S-NG-RAN node for transmitting to the UE user data associated with a specific data bearer configured with the split bearer option;

-	Information of the currently minimum desired buffer size at the S-NG-RAN node for transmitting to the UE user data associated with all data bearers configured with the split bearer option.

The following functions are provided by the Xn UP protocol for the SCG split bearer for dual connectivity with NR in E-UTRAN:

-	Provision of Xn UP specific sequence number information for user data transferred from the S-NG-RAN node to the M-NG-RAN node for a specific data bearer configured with the SCG split bearer option;

-	Information of successful in sequence delivery of PDCP PDUs to the UE from M-NG-RAN node for user data associated with a specific data bearer configured with the SCG split bearer option;

-	Information of PDCP PDUs transmitted to the lower layers for user data associated with a specific data bearer configured with the SCG split bearer option;

-	Information of PDCP PDUs from the M-NG-RAN node that were not delivered to the UE;

-	Information of downlink PDCP PDUs to be discarded for user data associated with a specific data bearer configured with downlink PDCP duplication;

-	Information of the currently desired buffer size at the M-NG-RAN node for transmitting to the UE user data associated with a specific data bearer configured with the split bearer option;

-	Information of the currently minimum desired buffer size at the M-NG-RAN node for transmitting to the UE user data associated with all data bearers configured with the split bearer option.

---------------------------------

Skip unchanged text

- ----------------------------------

### 5.4.1	Transfer of Downlink User Data

#### 5.4.1.1	Successful operation

Editor’s Note: All the text below is FFS.

The purpose of the Transfer of Downlink User Data procedure is to provide Xn-U specific sequence number information at the transfer of user data carrying a DL PDCP PDU from the NG-RAN node hosting the PDCP entity to the corresponding NG-RAN node via the Xn-U interface.

An Xn user plane instance making use of the Transfer of Downlink User Data procedure is associated to a single data bearer only. The Transfer of Downlink User Data procedure is invoked whenever user data for that particular data bearer needs to be sent across the Xn-U interface.

The NG-RAN node hosting the PDCP entity shall assign consecutive Xn-U sequence numbers to each transferred Xn-U packet.

The corresponding NG-RAN node shall detect whether an Xn-U packet was lost and memorise the respective sequence number after it has declared the respective Xn-U packet as being "lost".

The corresponding NG-RAN node shall transfer the remaining PDCP PDUs towards the UE and memorise the highest PDCP PDU sequence number of the PDCP PDU that was successfully delivered in sequence towards the UE (in case RLC AM is used) and the highest PDCP PDU sequence number of the PDCP PDU that was transmitted to the lower layers.

The corresponding node shall send the DL DATA DELIVERY STATUS if the Polling Flag is set.

The NG-RAN node hosting the PDCP entity can indicate to the corresponding NG-RAN node to either discard all PDCP PDU’s up to a defined DL PDCP PDU SN or discard one or a number of blocks of downlink PDCP PDUs.

NOTE:	The Transfer of Downlink User Data procedure and the associated feedback of lost Xn-U packets assist the NG-RAN node hosting the PDCP entity in avoiding PDCP HFN de-synchronisation. If an E-UTRAN deployment decides to not use the Transfer of Downlink User Data procedure, PDCP HFN synchronization should be ensured by other means.

Figure 5.4.1.1-1: Successful Transfer of Downlink User Data

---------------------------------

Skip unchanged text

- ----------------------------------

### 5.5.2	Frame format for the Xn user plane protocol

#### 5.5.2.1	DL USER DATA (PDU Type 0)

Editor’s Note: All the text below is FFS.

This frame format is defined to allow the NG-RAN node that does not host the PDCP entity to detect lost Xn-U packets and is associated with the transfer of a Downlink PDCP PDU over the Xn-U interface.

The following shows the respective DL USER DATA frame.

| Bits |  |  |  |  |  |  |  | Number of Octets |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |  |
| PDU Type (=0) |  |  |  | spare | DL Discard Blocks | DL Flush | Report polling | 1 |
| Xn-U Sequence Number |  |  |  |  |  |  |  | 3 |
| DL PDCP PDU SN |  |  |  |  |  |  |  | 3 |
| DL Number of blocks |  |  |  |  |  |  |  | 1 |
| DL PDCP PDU SN start (first block) |  |  |  |  |  |  |  | 3 |
| Block size (first block) |  |  |  |  |  |  |  | 1 |
| … |  |  |  |  |  |  |  |  |
| DL PDCP PDU SN start (last block) |  |  |  |  |  |  |  | 3 |
| Block size (last block) |  |  |  |  |  |  |  | 1 |
| Spare extension |  |  |  |  |  |  |  | 0-4 |

Figure 5.5.2.1-1: DL USER DATA (PDU Type 0) Format

---------------------------------

Skip unchanged text

- ----------------------------------

### 5.5.3	Coding of information elements in frames

#### 5.5.3.1	PDU Type

Editor’s Note: All the text below is FFS.

Description: The PDU Type indicates the structure of the Xn UP frame. The field takes the value of the PDU Type it identifies; i.e. "0" for PDU Type 0. The PDU type is in bit 4 to bit 7 in the first octet of the frame.

Value range: {0=DL USER DATA, 1=DL DATA DELIVERY STATUS, 2-15=reserved for future PDU type extensions}

Field length: 4 bits

#### 5.5.3.2	Spare

Editor’s Note: All the text below is FFS.

Description: The spare field is set to "0" by the sender and should not be interpreted by the receiver. This field is reserved for later versions.

Value range: (0–2n-1).

Field Length: n bits.

#### 5.5.3.3	Xn-U Sequence Number

Editor’s Note: All the text below is FFS.

Description: This parameter indicates the Xn-U sequence number as assigned by the respective NG-RAN node.

Value range: {0..224-1}.

Field length: 3 octets.

#### 5.5.3.4	Lost Packet Report

Editor’s Note: All the text below is FFS.

Description: This parameter indicates the presence of a list of lost Xn-U packets in the respective Xn UP frame.

Value range: {0=Lost Frame List not present, 1=Lost Frame List present}.

Field length: 1 bit.

#### 5.5.3.5	Final Frame Indication

Editor’s Note: All the text below is FFS.

Description: This parameter indicates whether the frame is the last DL status report as described in clause 5.4.2.1.

Value range: {0=Frame is not final, 1= Frame is final}.

Field length: 1 bit.

#### 5.5.3.6	Highest successfully delivered PDCP Sequence Number

Editor’s Note: All the text below is FFS.

Description: This parameter indicates feedback about the in-sequence delivery status of PDCP PDUs at the corresponding NG-RAN node towards the UE.

Value range: {0..218-1}.

Field length: 3 octets.

#### 5.5.3.7	Desired buffer size for the data bearer

Editor’s Note: All the text below is FFS.

Description: This parameter indicates the desired buffer size for the concerned data bearer as specified in clause 5.4.2.1.

Value range: {0..232-1}.

Field length: 4 octets.

#### 5.5.3.8	Minimum desired buffer size for the UE

Editor’s Note: All the text below is FFS.

Description: This parameter indicates the minimum desired buffer size for all data bearers established for the UE as specified in clause 5.4.2.1.

Value range: {0..232-1}.

Field length: 4 octets.

#### 5.5.3.9	Number of lost Xn-U Sequence Number ranges reported

Editor’s Note: All the text below is FFS.

Description: This parameter indicates the number of Xn-U Sequence Number ranges reported to be lost.

Value range: {1..256}.

Field length: 1 octet.

#### 5.5.3.10	Start of lost Xn-U Sequence Number range

Editor’s Note: All the text below is FFS.

Description: This parameter indicates the start of an Xn-U sequence number range.

Value range: {0..224-1}.

Field length: 3 octets.

#### 5.5.3.11	End of lost Xn-U Sequence Number range

Editor’s Note: All the text below is FFS.

Description: This parameter indicates the end of an Xn-U sequence number range.

Value range: {0..224-1}.

Field length: 3 octets.

#### 5.5.3.x	DL Flush

Editor’s Note: All the text below is FFS.

Description: This parameter indicates the presence of DL PDCP PDU SN.

Value range: {0= DL PDCP PDU SN not present, 1= DL PDCP PDU SN present}.

Field length: 1 bit.

#### 5.5.3.x+1	DL PDCP PDU SN

Editor’s Note: All the text below is FFS.

Description: This parameter indicates the downlink PDCP PDU sequence number up to which all the PDCP PDUs should be discarded.

Value range: {0..218-1}.

Field length: 3 octets.

#### 5.5.3.y	DL Discard Blocks

Editor’s Note: All the text below is FFS.

Description: This parameter indicates the downlink PDCP PDU blocks which should be discarded from “DL PDCP PDU SN start” for the number of PDUs indicated by “Block size”. Repeat for “the DL Number of blocks”.

Value range: {0= No Discard, 1= Discard}.

Field length: 1 bit.

#### 5.5.3.z	DL PDCP PDU SN start

Editor’s Note: All the text below is FFS.

Description: This parameter indicates the starting SN of a downlink PDCP PDU block.

Value range: {0..218-1}.

Field length: 3 octets.

#### 5.5.3.T	DL Number of blocks

Editor’s Note: All the text below is FFS.

Description: This parameter indicates the number of PDCP PDU blocks.

Value range: {1..256}.

Field length: 1 octet.

#### 5.5.3.W	Block size

Editor’s Note: All the text below is FFS.

Description: This parameter indicates the number of PDCP PDUs counted from the starting SN.

Value range: {1..256}.

Field length: 1 octet.

#### 5.5.3.12	Spare extension

Editor’s Note: All the text below is FFS.

Description: The spare extension field shall not be sent. The receiver should be capable of receiving a spare extension. The spare extension should not be interpreted by the receiver, since in later versions of the present document additional new fields might be added in place of the spare extension. The spare extension can be an integer number of octets carrying new fields or additional information; the maximum length of the spare extension field (m) depends on the PDU type.

Value range: 0–2m*8-1.

Field Length: 0–m octets. For the PDU Types defined in the present document m=4.