3GPP TSG-RAN WG3 Meeting #98	R3-175077

# Reno, USA, 27th November - 1st  December 2017

Agenda Item:	10.8.1

Source:	Ericsson, Nokia, Nokia Shanghai Bell

Title:	RLC Mode

Document for:	pCR

# Text Proposal for NGAP TS 38.423 v0.4.0

<<<<<<<<<<<<<<<<<<<<<<<<<<<<<< Begin Text Proposal >>>>>>>>>>>>>>>>>>>>>>>>>>>>

### 8.3.3	M-NG-RAN node initiated S-NG-RAN node Modification Preparation

Editor’s Note: All the text below is FFS.

#### 8.3.3.1	General

This procedure is used to enable an M-NG-RAN node to request an S-NG-RAN node to modify the UE context at the S-NG-RAN node or to query the current SCG configuration for supporting delta signalling in M-NG-RAN node initiated S-NG-RAN node change.

The procedure uses UE-associated signalling.

#### 8.3.3.2	Successful Operation

Figure 8.3.3.2-1: M-NG-RAN node initiated S-NG-RAN node Modification Preparation, successful operation

The M-NG-RAN node initiates the procedure by sending the S-NODE MODIFICATION REQUEST message to the S-NG-RAN node.

When the M-NG-RAN node sends the S-NODE MODIFICATION REQUEST message, it shall start the timer TDCprep.

The S-NODE MODIFICATION REQUEST message may contain

-	within the UE Context Information IE;

-	PDU sessions to be added within the PDU sessions To Be Added Item IE;

-	PDU sessions to be modified within the PDU sessions To Be Modified Item IE;

-	PDU sessions to be released within the PDU sessions To Be Released Item IE;

-	the S-NG-RAN node UE Aggregate Maximum Bit Rate IE;

-	the M-NG-RAN node to S-NG-RAN node Container IE;

-	the SCG Change Indication IE;

-	the SCG Configuration Query IE.

If the S-NODE MODIFICATION REQUEST message contains the Serving PLMN IE, the S-NG-RAN node may use it for RRM purposes.

If the S-NODE MODIFICATION REQUEST message contains the Handover Restriction List IE, the S-NG-RAN node shall

- replace the previously provided Handover Restriction List by the received Handover Restriction List in the UE context;

- use this information to select an appropriate SCG.

If the S-NG-RAN node UE Aggregate Maximum Bit Rate IE is included in the S-NODE MODIFICATION REQUEST message, the S-NG-RAN node shall:

-	replace the previously provided S-NG-RAN node UE Aggregate Maximum Bit Rate by the received S-NG-RAN node UE Aggregate Maximum Bit Rate in the UE context;

-	use the received S-NG-RAN node UE Aggregate Maximum Bit Rate for non-GBR Bearers for the concerned UE as defined in TS 37.340 [8].

The allocation of resources according to the values of the Allocation and Retention Priority IE included in the PDU session Level QoS Parameters IE shall follow the principles described for the PDU session Setup procedure in TS 38.413 [5].

If at least one of the requested modifications is admitted by the S-NG-RAN node, the S-NG-RAN node shall modify the related part of the UE context accordingly and send the S-NODE MODIFICATION REQUEST ACKNOWLEDGE message back to the M-NG-RAN node.

The S-NG-RAN node shall include the PDU sessions for which resources have been either added or modified or released at the S-NG-RAN node either in the PDU sessions Admitted To Be Added List IE or the PDU sessions Admitted To Be Modified List IE or the PDU sessions Admitted To Be Released List IE. The S-NG-RAN node shall include the PDU sessions that have not been admitted in the PDU sessions Not Admitted List IE with an appropriate cause value.

If the PDU session level QoS parameter IE is included in the S-NODE MODIFICATION REQUEST message for an PDU session to be modified the S-NG-RAN node shall allocate respective resources and provide corresponding radio configuration information within the S-NG-RAN node to M-NG-RAN node Container IE as described in TS 37.340 [8].

If the S-NODE MODIFICATION REQUEST message contains for an PDU session to be modified which is configured with the SCG bearer option the NG UL GTP Tunnel Endpoint IE the S-NG-RAN node shall use it as the new UL NG-U address.

If the S-NODE MODIFICATION REQUEST message contains for an PDU session to be modified which is configured with the split bearer option the M-NG-RAN node GTP Tunnel Endpoint IE the S-NG-RAN node shall use it as the new UL Xn-U address.

For an PDU session to be modified which is configured with the SCG bearer option the S-NG-RAN node may include in the S-NODE MODIFICATION REQUEST ACKNOWLEDGE message the NG DL GTP Tunnel Endpoint IE.

For an PDU session to be modified which is configured with the split bearer option the S-NG-RAN node may include in the S-NODE MODIFICATION REQUEST ACKNOWLEDGE message the S-NG-RAN node GTP Tunnel Endpoint IE.

If the SCG Change Indication IE is included in the S-NODE MODIFICATION REQUEST message, the S-NG-RAN node shall act as specified in TS 37.340 [8].

Upon reception of the S-NODE MODIFICATION REQUEST ACKNOWLEDGE message the M-NG-RAN node shall stop the timer TDCprep. If the S-NODE MODIFICATION REQUEST ACKNOWLEDGE message has included the S-NG-RAN node to M-NG-RAN node Container IE the M-NG-RAN node is then defined to have a Prepared S-NG-RAN node Modification for that Xn UE-associated signalling.

If the SCG Configuration Query IE is included in the S-NODE MODIFICATION REQUEST message, the S-NG-RAN node shall provide corresponding radio configuration information within the SgNB to MeNB Container IE as described in TS 37.340 [8].

#### 8.3.1.3	Unsuccessful Operation

Figure 8.3.1.3-1: S-NG-RAN node Addition Preparation, unsuccessful operation

If the S-NG-RAN node is not able to accept any of the bearers or a failure occurs during the S-NG-RAN node Addition Preparation, the S-NG-RAN node sends the S-NODE ADDITION REQUEST REJECT message with an appropriate cause value to the M-NG-RAN node.

#### 8.3.1.4	Abnormal Conditions

Void.

#### 9.1.2.1	S-NODE ADDITION REQUEST

This message is sent by the M-NG-RAN node to the S-NG-RAN node to request the preparation of resources for dual connectivity operation for a specific UE.

Direction: M-NG-RAN node  S-NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | <reference> |  | YES | reject |
| M-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID <reference> | Allocated at the M-NG-RAN node | YES | reject |
| UE Security Capabilities | C- ifSCGorSCGsplitBearerOption |  | <reference> |  | YES | reject |
| SgNB Security Key | C-ifSCGorSCGsplitBearerOption |  | <reference> | The S-KgNB which is provided by the M-NG-RAN node, see xxx. Editor’s Note: terminology “S-KgNB” to be fixed with SA3 and RAN2 | YES | reject |
| S-NG-RAN node UE Aggregate Maximum Bit Rate | M |  | UE Aggregate Maximum Bit Rate <reference> | The UE Aggregate Maximum Bit Rate is split into M-NG-RAN node UE Aggregate Maximum Bit Rate and S-NG-RAN node UE Aggregate Maximum Bit Rate which are enforced by M-NG-RAN node and S-NG-RAN node respectively. | YES | reject |
| Serving PLMN | O |  | PLMN Identity <reference> | The serving PLMN of the SCG in the S-NG-RAN node. | YES | ignore |
| Handover Restriction List | O |  | <reference> |  | YES | ignore |
| PDU sessions To Be Added List |  | 1 |  |  | YES | reject |
| >PDU sessions To Be Added Item |  | 1 .. <maxnoofPDUsessions> |  |  | EACH | reject |
| >>CHOICE Bearer Option | M |  |  |  |  |  |
| >>>SCG Bearer |  |  |  |  |  |  |
| >>>>PDU session ID | M |  | <reference> |  | – | – |
| >>>>S-NSSAI | O |  | <reference> |  | – | – |
| >>>>PDU session Level QoS Parameters | M |  | <reference> | Includes necessary QoS parameters | – | – |
| >>>>DL Forwarding | O |  | <reference> |  | – | – |
| >>>>NG UL GTP Tunnel Endpoint | M |  | GTP Tunnel Endpoint <reference> | UPF endpoint of the NG transport bearer. For delivery of UL PDUs. | – | – |
| >>>>SIPTO Correlation ID | O |  | Correlation ID <reference> |  | – | – |
| >>>Split Bearer |  |  |  |  |  |  |
| >>>>PDU session ID | M |  | <reference> |  | – | – |
| >>>>S-NSSAI | O |  | <reference> |  | – | – |
| >>>>PDU session Level QoS Parameters | M |  | <reference> | Includes necessary QoS parameters | – | – |
| >>>>M-NG-RAN node GTP Tunnel Endpoint | M |  | GTP Tunnel Endpoint <reference> | M-NG-RAN node endpoint of the Xn transport bearer. For delivery of UL PDUs. | – | – |
| >>>>RLC Mode | M |  | RLC Mode 9.2.x |  | – | – |
| >>>SCG split Bearer |  |  |  |  |  |  |
| >>>>SCG split Bearer S-NG-RAN node Addition Item |  | 1 |  |  | YES | reject |
| >>>>>PDU session ID | M |  | <reference> |  | – | – |
| >>>>>S-NSSAI | O |  | <reference> |  | – | – |
| >>>>>Total PDU session Level QoS Parameters | M |  | PDU session Level QoS Parameters <reference> | Includes the PDU session Level QoS parameters as received on NG-AMF for the PDU session | – | – |
| >>>>>M-NG-RAN node PDU session Level QoS Parameters | O |  | PDU session Level QoS Parameters <reference> | Includes, if applicable, the share the M-NG-RAN node is offering to take | – | – |
| >>>>>DL Forwarding | O |  | <reference> |  | – | – |
| >>>>>M-NG-RAN node GTP Tunnel Endpoint | M |  | GTP Tunnel Endpoint <reference> | M-NG-RAN node endpoint of the Xn transport bearer. For delivery of DL PDUs. | – | – |
| >>>>>NG UL GTP Tunnel Endpoint | M |  | GTP Tunnel Endpoint <reference> | UPF endpoint of the NG transport bearer. For delivery of UL PDUs. | – | – |
| >>>>>SIPTO Correlation ID | O |  | Correlation ID <reference> |  | – | – |
| M-NG-RAN node to S-NG-RAN node Container | M |  | OCTET STRING | Includes the SCG-ConfigInfo message as defined in xxx Editor’s Note: to be checked with RAN2 | YES | reject |
| S-NG-RAN node UE XnAP ID | O |  | NG-RAN node UE XnAP ID <reference> | Allocated at the S-NG-RAN node | YES | reject |
| Expected UE Behaviour | O |  | <reference> |  | YES | ignore |
| Requested MCG split SRBs | O |  | ENUMERATED (srb1, srb2, srb1&2, ...) | Indicates that resources for MCG Split SRB are requested. | YES | reject |

| Range bound | Explanation |
| --- | --- |
| maxnoofPDUsessions | Maximum no. of PDU sessions. Value is 256 |

| Condition | Explanation |
| --- | --- |
| ifSCGorSCGsplitBearerOption | This IE shall be present if the Bearer Option IE is set to the value "SCG bearer" or "SCG split bearer". |

#### 9.1.2.2	S-NODE ADDITION REQUEST ACKNOWLEDGE

This message is sent by the S-NG-RAN node to confirm the M-NG-RAN node about the S-NG-RAN node addition preparation.

Direction: S-NG-RAN node  M-NG-RAN node.

Editor’s Note: The tabular below is FFS.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | <reference> |  | YES | reject |
| M-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID <reference> | Allocated at the M-NG-RAN node | YES | reject |
| S-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID <reference> | Allocated at the S-NG-RAN node | YES | reject |
| PDU sessions Admitted To Be Added List |  | 1 |  |  | YES | ignore |
| >PDU sessions Admitted To Be Added Item |  | 1 .. <maxnoofPDUsessions> |  |  | EACH | ignore |
| >>CHOICE Bearer Option | M |  |  |  |  |  |
| >>>SCG Bearer |  |  |  |  |  |  |
| >>>>PDU session ID | M |  | <reference> |  | – | – |
| >>>>NG DL GTP Tunnel Endpoint | M |  | GTP Tunnel Endpoint <reference> | S-NG-RAN node endpoint of the NG transport bearer. For delivery of DL PDUs. | – | – |
| >>>>DL Forwarding GTP Tunnel Endpoint | O |  | GTP Tunnel Endpoint <reference> | Identifies the Xn transport bearer used for forwarding of DL PDUs | – | – |
| >>>>UL Forwarding GTP Tunnel Endpoint | O |  | GTP Tunnel Endpoint <reference> | Identifies the Xn transport bearer used for forwarding of UL PDUs | – | – |
| >>>Split Bearer |  |  |  |  |  |  |
| >>>>PDU session ID | M |  | <reference> |  | – | – |
| >>>>S-NG-RAN node GTP Tunnel Endpoint | M |  | GTP Tunnel Endpoint <reference> | Endpoint of the Xn transport bearer at the S-NG-RAN node. | – | – |
| >>>SCG split Bearer |  |  |  |  |  |  |
| >>>>SCG split Bearer S-NG-RAN node Addition Item |  | 1 |  |  | YES | reject |
| >>>>>PDU session ID | M |  | <reference> |  | – | – |
| >>>>>NG DL GTP Tunnel Endpoint | M |  | GTP Tunnel Endpoint <reference> | S-NG-RAN node endpoint of the NG transport bearer. For delivery of DL PDUs. | – | – |
| >>>>>S-NG-RAN node GTP Tunnel Endpoint | M |  | GTP Tunnel Endpoint <reference> | S-NG-RAN node endpoint of the Xn transport bearer. For delivery of UL PDUs. | – | – |
| >>>>>DL Forwarding GTP Tunnel Endpoint | O |  | GTP Tunnel Endpoint <reference> | Identifies the Xn transport bearer used for forwarding of DL PDUs. | – | – |
| >>>>>UL Forwarding GTP Tunnel Endpoint | O |  | GTP Tunnel Endpoint <reference> | Identifies the Xn transport bearer used for forwarding of UL PDUs. | – | – |
| >>>>>RLC Mode | M |  | RLC Mode 9.2.x |  | – | – |
| >>QoS Flows Admitted List |  | 0..1 |  |  | - |  |
| >>>>QoS Flows List | O |  | 9.2.10 |  |  |  |
| >>QoS Flows not Admitted List |  | 0..1 |  |  |  |  |
| >>>>QoS Flows List | O |  | 9.2.10 |  |  |  |
| PDU sessions Not Admitted List | O |  | PDU session List <reference> | A value for PDU session ID shall only be present once in PDU sessions Admitted List IE and in PDU sessions Not Admitted List IE. | YES | ignore |
| S-NG-RAN node to M-NG-RAN node Container | M |  | OCTET STRING | Includes the SCG-Config message as defined in TS 36.331 [9]. Editor’s Note: To be checked with RAN2 if the same RRC message is applicable for SCG split bearer. | YES | reject |
| Criticality Diagnostics | O |  | <reference> |  | YES | ignore |
| SIPTO L-GW Transport Layer Address | O |  | BIT STRING (1..160, ...) | Indicating SIPTO L-GW Transport Layer Address. | YES | ignore |
| Admitted MCG split SRBs | O |  | ENUMERATED (srb1, srb2, srb1&2, ...) | Indicates admitted SRBs | YES | reject |

| Range bound | Explanation |
| --- | --- |
| maxnoofPDUsessions | Maximum no. of PDU sessions. Value is 256 |

------------------------------------------

Skip to next change

------------------------------------------

#### 9.1.2.5	S-NODE MODIFICATION REQUEST

This message is sent by the M-NG-RAN node to the S-NG-RAN node to request the preparation to modify S-NG-RAN node resources for a specific UE.

Direction: M-NG-RAN node  S-NG-RAN node.

Editor’s Note: The tabular below is FFS.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | <reference> |  | YES | reject |
| M-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID <reference> | Allocated at the M-NG-RAN node | YES | reject |
| S-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID <reference> | Allocated at the S-NG-RAN node | YES | reject |
| Cause | M |  | <reference> |  | YES | ignore |
| SCG Change Indication | O |  | <reference> |  | YES | ignore |
| Serving PLMN | O |  | PLMN Identity <reference> | The serving PLMN of the SCG in the S-NG-RAN node. | YES | ignore |
| Handover Restriction List | O |  | <reference> |  | YES | ignore |
| SCG Configuration Query | O |  | 9.2.13 |  | YES | ignore |
| UE Context Information |  | 0..1 |  |  | YES | reject |
| >UE Security Capabilities | O |  | <reference> |  | – | – |
| >SgNB Security Key | O |  | <reference> | Editor’s Note: terminology “S-KgNB” to be fixed with SA3 and RAN2 | – | – |
| >S-NG-RAN node UE Aggregate Maximum Bit Rate | O |  | UE Aggregate Maximum Bit Rate <reference> |  | – | – |
| > PDU sessions To Be Added List |  | 0..1 |  |  | – | – |
| >> PDU sessions To Be Added Item |  | 1 .. <maxnoofPDUsessions> |  |  | EACH | ignore |
| >>>CHOICE Bearer Option | M |  |  |  |  |  |
| >>>>SCG Bearer |  |  |  |  |  |  |
| >>>>> PDU sessions ID | M |  | <reference> |  | – | – |
| >>>>> PDU sessions Level QoS Parameters | M |  | <reference> | Includes necessary QoS parameters | – | – |
| >>>>>DL Forwarding | O |  | <reference> |  | – | – |
| >>>>>NG UL GTP Tunnel Endpoint | M |  | GTP Tunnel Endpoint <reference> | UPF endpoint of the NG transport bearer. For delivery of UL PDUs. | – | – |
| >>>>>Correlation ID | O |  | Correlation ID <reference> |  | – | – |
| >>>>>SIPTO Correlation ID | O |  | Correlation ID <reference> |  | – | – |
| >>>>Split Bearer |  |  |  |  |  |  |
| >>>>> PDU sessions ID | M |  | <reference> |  | – | – |
| >>>>> PDU sessions Level QoS Parameters | M |  | <reference> | Includes necessary QoS parameters | – | – |
| >>>>>M-NG-RAN node GTP Tunnel Endpoint | M |  | GTP Tunnel Endpoint <reference> | M-NG-RAN node endpoint of the Xn transport bearer. For delivery of UL PDUs. | – | – |
| >>>>>RLC Mode | O |  | RLC Mode 9.2.x |  | – | – |
| > PDU sessions To Be Modified List |  | 0..1 |  |  | – | – |
| >> PDU sessions To Be Modified Item |  | 1 .. <maxnoofPDUsessions> |  |  | EACH | ignore |
| >>>CHOICE Bearer Option | M |  |  |  |  |  |
| >>>>SCG Bearer |  |  |  |  |  |  |
| >>>>> PDU sessions ID | M |  | <reference> |  | – | – |
| >>>>> PDU sessions Level QoS Parameters | O |  | <reference> | Includes QoS parameters to be modified | – | – |
| >>>>>NG UL GTP Tunnel Endpoint | O |  | GTP Tunnel Endpoint <reference> | UPF endpoint of the NG transport bearer. For delivery of UL PDUs. | – | – |
| >>>>Split Bearer |  |  |  |  |  |  |
| >>>>> PDU sessions ID | M |  | <reference> |  | – | – |
| >>>>> PDU sessions Level QoS Parameters | O |  | <reference> | Includes QoS parameters to be modified | – | – |
| >>>>>M-NG-RAN node GTP Tunnel Endpoint | O |  | GTP Tunnel Endpoint <reference> | M-NG-RAN node endpoint of the Xn transport bearer. For delivery of UL PDUs. | – | – |
| > PDU sessions To Be Released List |  | 0..1 |  |  | – | – |
| >> PDU sessions To Be Released Item |  | 1 .. <maxnoofPDUsessions> |  |  | EACH | ignore |
| >>>CHOICE Bearer Option | M |  |  |  |  |  |
| >>>>SCG Bearer |  |  |  |  |  |  |
| >>>>> PDU sessions ID | M |  | <reference> |  | – | – |
| >>>>>DL Forwarding GTP Tunnel Endpoint | O |  | GTP Tunnel Endpoint <reference> | Identifies the Xn transport bearer used for forwarding of DL PDUs | – | – |
| >>>>>UL Forwarding GTP Tunnel Endpoint | O |  | GTP Tunnel Endpoint <reference> | Identifies the Xn transport bearer. used for forwarding of UL PDUs | – | – |
| >>>>Split Bearer |  |  |  |  |  |  |
| >>>>> PDU sessions ID | M |  | <reference> |  | – | – |
| >>>>>DL Forwarding GTP Tunnel Endpoint | O |  | GTP Tunnel Endpoint <reference> | Identifies the Xn transport bearer used for forwarding of DL PDUs | – | – |
| M-NG-RAN node to S-NG-RAN node Container | O |  | OCTET STRING | Includes the SCG-ConfigInfo message as defined in TS 38.331 [x] | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofPDUsessions | Maximum no. of E-RABs. Value is 256 |

#### 9.1.2.6	S-NODE MODIFICATION REQUEST ACKNOWLEDGE

This message is sent by the S-NG-RAN node to confirm the M-NG-RAN node’s request to modify the S-NG-RAN node resources for a specific UE.

Direction: S-NG-RAN node  M-NG-RAN node.

Editor’s Note: The tabular below is FFS.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | <reference> |  | YES | reject |
| M-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID <reference> | Allocated at the M-NG-RAN node | YES | ignore |
| S-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID <reference> | Allocated at the S-NG-RAN node | YES | ignore |
| PDU sessions Admitted List |  | 0..1 |  |  | YES | ignore |
| >PDU sessions Admitted To Be Added List |  | 1 |  |  | – | – |
| >>PDU sessions Admitted To Be Added Item |  | 1 .. <maxnoofPDUsessions> |  |  | EACH | ignore |
| >>>CHOICE Bearer Option | M |  |  |  |  |  |
| >>>>SCG Bearer |  |  |  |  |  |  |
| >>>>>PDU session ID | M |  | <reference> |  | – | – |
| >>>>>NG DL GTP Tunnel Endpoint | M |  | GTP Tunnel Endpoint <reference> | S-NG-RAN node endpoint of the NG transport bearer. For delivery of DL PDUs. | – | – |
| >>>>>DL Forwarding GTP Tunnel Endpoint | O |  | GTP Tunnel Endpoint <reference> | Identifies the Xn transport bearer used for forwarding of DL PDUs | – | – |
| >>>>>UL Forwarding GTP Tunnel Endpoint | O |  | GTP Tunnel Endpoint <reference> | Identifies the Xn transport bearer used for forwarding of UL PDUs | – | – |
| >>>>Split Bearer |  |  |  |  |  |  |
| >>>>>PDU session ID | M |  | <reference> |  | – | – |
| >>>>>S-NG-RAN node GTP Tunnel Endpoint | M |  | GTP Tunnel Endpoint <reference> | Endpoint of the Xn transport bearer at the S-NG-RAN node. | – | – |
| >>>QoS Flows Admitted List |  | 0..1 |  |  | - |  |
| >>>>QoS Flows List | O |  | 9.2.10 |  |  |  |
| >>>QoS Flows not Admitted List |  | 0..1 |  |  |  |  |
| >>>>QoS Flows List | O |  | 9.2.10 |  |  |  |
| >PDU sessions Admitted To Be Modified List |  | 0..1 |  |  | – | – |
| >>PDU sessions Admitted To Be Modified Item |  | 1 .. <maxnoofPDUsessions> |  |  | EACH | ignore |
| >>>CHOICE Bearer Option | M |  |  |  |  |  |
| >>>>SCG Bearer |  |  |  |  |  |  |
| >>>>>PDU session ID | M |  | <reference> |  | – | – |
| >>>>>NG DL GTP Tunnel Endpoint | O |  | GTP Tunnel Endpoint <reference> | S-NG-RAN node endpoint of the NG transport bearer. For delivery of DL PDUs. | – | – |
| >>>>Split Bearer |  |  |  |  |  |  |
| >>>>>PDU session ID | M |  | <reference> |  | – | – |
| >>>>>S-NG-RAN node GTP Tunnel Endpoint | O |  | GTP Tunnel Endpoint <reference> | Endpoint of the Xn transport bearer at the S-NG-RAN node. | – | – |
| >>>>>RLC Mode | O |  | RLC Mode 9.2.x |  | – | – |
| >>>QoS Flows Admitted List |  | 0..1 |  |  | - |  |
| >>>>QoS Flows List | O |  | 9.2.10 |  | - |  |
| >>>QoS Flows not Admitted List |  | 0..1 |  |  |  |  |
| >>>>QoS Flows List | O |  | 9.2.10 |  | - |  |
| >PDU sessions Admitted To Be Released List |  | 0..1 |  |  | – | – |
| >>PDU sessions Admitted To Be Released Item |  | 1 .. <maxnoPDUsessions> |  |  | EACH | ignore |
| >>>CHOICE Bearer Option | M |  |  |  |  |  |
| >>>>SCG Bearer |  |  |  |  |  |  |
| >>>>>PDU session ID | M |  | <reference> |  | – | – |
| >>>>Split Bearer |  |  |  |  |  |  |
| >>>>>PDU session ID | M |  | <reference> |  | – | – |
| PDU sessions Not Admitted List | O |  | PDU session List <reference> | A value for PDU session ID shall only be present once in PDU sessions Admitted List IE and in PDU sessions Not Admitted List IE. | YES | ignore |
| S-NG-RAN node to M-NG-RAN node Container | O |  | OCTET STRING | Includes the SCG-Config message as defined in TS xx.xxx [x] Editor’s Note: to be checked with RAN2 | YES | ignore |
| Criticality Diagnostics | O |  | <reference> |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofPDUsessions | Maximum no. of PDU sessions. Value is 256 |

<<<<<<<<<<<<<<<<<<<< End Text Proposal  >>>>>>>>>>>>>>>>>>>>