3GPP TSG-RAN WG3#99	R3-180933

Athens, Greece, 26 February – 02 March 2018

Agenda item:	10.1.1

Source:	Nokia, Nokia Shanghai Bell

Title:	Signalling 5QI in QoS Profile of TS 38.423

Document for:	Discussion and Decision

# Description and proposal

The QoS profile has been cleaned up with a new structure at last RAN3 through tdoc [3] as follows:

#### 9.3.1.23	QoS Flow Level QoS Parameters

Editor’s Note:	Further details FFS.

This IE defines the QoS parameters to be applied to a QoS flow.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| 5QI | O (FFS) |  | INTEGER (0..255) | Details in TS 23.501 [9]. | YES | reject |
| CHOICE QoS Characteristics | M |  |  |  | YES | reject |
| >Non-dynamic 5QI |  |  |  |  | - |  |
| >>Non Dynamic 5QI Descriptor | M |  | 9.3.1.40 |  | - |  |
| >Dynamic 5QI |  |  |  |  | - |  |
| >>Dynamic 5QI Descriptor | M |  | 9.3.1.29 |  | - |  |
| Allocation and Retention Priority | M |  | 9.3.1.30 |  | - |  |
| GBR QoS Flow Information | O |  | 9.3.1.10 | This IE shall be present for GBR QoS Flows only. | - |  |
| Reflective QoS Attribute | O |  | ENUMERATED(subject to, …) | Details in TS 23.501 [9]. This IE applies to non-GBR flows only and shall be ignored otherwise. | - |  |

The presence of 5QI within the QoS Profile IE has been left FFS at the last RAN3 meeting.

Tdoc [4] has explained that mandating 5QI presence is not harmful even if redundant in some cases but would have the merit to simplify the test cases and the interoperability. In total Tdoc [4] has proposed two variants to solve the FFS.

We therefore propose to agree one of the two variants presented in the text proposal below to mirror into TS 38.423 the change of the 5QI presence of tdoc [4].

# References

- RP-172109, Revised Work Item on New Radio (NR) Access Technology, NTT DOCOMO

- 3GPP TS 23.501, Architecture for Next Generation System, SA2

- R3-180628, Correction of QoS Profile for TS 38.413, Nokia, Nokia Shanghai Bell

- R3-180932, Signalling 5QI in the QoS Profile of TS 38.413, Nokia, Nokia Shanghai Bell

# Text Proposal for TS 38.423

Variant 1

#### 9.2.2.1	QoS Flow Level QoS Parameters

This IE defines the QoS Parameters to be applied to a QoS flow.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| 5QI | M |  | INTEGER (0..255) | See details in TS 23.501 [7]. | – |  |
| CHOICE QoS Characteristics | M |  |  |  | – |  |
| >Non Dynamic 5QI |  |  |  |  | – |  |
| >>Non dynamic 5QI Desciptor | M |  | 9.2.2.8 |  |  |  |
| >Dynamic 5QI |  |  |  |  | – |  |
| >>Dynamic 5QI Desciptor | M |  | 9.2.2.7 |  | – |  |
| Allocation and Retention Priority | M |  | 9.2.2.6 | Note: presence needs to be checked with latest SA2 status. | – |  |
| GBR QoS Flow Information | O |  | 9.2.2.2 | This IE shall be present for GBR QoS Flows only. | – |  |
| Reflective QoS Attribute | O |  | ENUMERATED (subject to, ...) | For details see TS 23.501 [7]. This IE applies to non-GBR bearers only and shall be ignored otherwise. | – |  |

Variant 2

#### 9.2.2.1	QoS Flow Level QoS Parameters

This IE defines the QoS Parameters to be applied to a QoS flow.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| CHOICE QoS Characteristics | M |  |  |  | – |  |
| >Non Dynamic 5QI |  |  |  |  | – |  |
| >>Non dynamic 5QI Desciptor | M |  | 9.2.2.8 |  |  |  |
| >Dynamic 5QI |  |  |  |  | – |  |
| >>Dynamic 5QI Desciptor | M |  | 9.2.2.7 |  | – |  |
| Allocation and Retention Priority | M |  | 9.2.2.6 | Note: presence needs to be checked with latest SA2 status. | – |  |
| GBR QoS Flow Information | O |  | 9.2.2.2 | This IE shall be present for GBR QoS Flows only. | – |  |
| Reflective QoS Attribute | O |  | ENUMERATED (subject to, ...) | For details see TS 23.501 [7]. This IE applies to non-GBR bearers only and shall be ignored otherwise. | – |  |

#### 9.2.2.7	Dynamic 5QI Descriptor

This IE defines the QoS characteristics for a non-standardized or not pre-configured 5QI for downlink and uplink.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| 5QI | O |  | INTEGER (0..255) | For details refer to TS 23.501 [7]. |
| Priority level | M |  | <reference> | For details refer to TS 23.501 [7]. |
| Packet Delay Budget | M |  | <reference> | For details refer to TS 23.501 [7]. |
| Packet Error Rate | M |  | <reference> | For details refer to TS 23.501 [7]. |
| Delay Critical | C-ifGBRflow |  | ENUMERATED (Delay critical, Non-delay critical) | For details refer to TS 23.501 [7]. |
| Averaging window | C-ifGBRflow |  | <reference> | For details refer to TS 23.501 [7]. |
| Maximum Data Burst Volume | O |  | <reference> | For details refer to TS 23.501 [7]. |

| Condition | Explanation |
| --- | --- |
| ifGBRflow | This IE shall be present if the GBR QoS Flow Information IE is present in the QoS Flow Level QoS Parameters IE. |

#### 9.2.2.8	Non dynamic 5QI Descriptor

This IE defines QoS characteristics for a standardized or pre-configured 5QI for downlink and uplink.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| 5QI | M |  | INTEGER (0..255) | For details refer to TS 23.501 [7]. |
| Priority level | O |  | <reference> | For details refer to TS 23.501 [7]. When included overrides standardized or pre-configured value. |
| Averaging window | O |  | <reference> | This IE applies to GBR QoS Flows only. For details refer to TS 23.501 [7]. When included overrides standardized or pre-configured value. |
| Maximum Data Burst Volume | O |  | <reference> | For details refer to TS 23.501 [7]. When included overrides standardized or pre-configured value. |