3GPP TSG-RAN WG3 Meeting #99	R3-181022

Athens, Greece, Feb. 26th – Mar. 2nd, 2018

Agenda item:		10.8.2.7

Source:	LG Electronics Inc.

Title:	Issues on Session AMBR for Option 7 family

Document for:		Approval

1. Introduction

Session AMBR is the issues to solve for LTE/NR interworking option 7. This paper is to investigate the related issues and the corresponding proposals are also provided together with stage 2 text proposal.

2. Discussion

In legacy, the UE AMBR is decided by EPC and signaled to RAN. Now in 5G, the UE-AMBR is decided by RAN, for which the following parameters should be sent to RAN node:

- Subscribed UE-AMBR

- Session AMBR

Based on the parameters above, RAN node decides the UE-AMBR. For EN-DC, the following principles are applied according to the current specification:

-	The MN decides the split of DL UE AMBR limits and UL UE AMBR limits among the MN and the SN, respectively, and indicates DL UE AMBR limits and UL UE AMBR limits to be respected by the SN.

-	The node that hosts the PDCP entity enforces the respective DL UE AMBR limits.

-	The node enforces the respective UL UE AMBR limits. For split bearers and SCG split bearers, if the node is not configured to serve the uplink, it ignores the indicated UL UE-AMBR.

In last meeting, there was discussion on whether to change the principles or not. But in any case, similar principles are estimated for MR-DC.

In addition, issue still exists on how to handle session AMBR.

Firstly, whether MN needs to decide the split of session AMBR and indicate the portion of session AMBR to be respected by the SN.

On this issue, basically we believe that the split of session AMBR would help the SN node to do better and more optimized radio resource control on its side for several reasons. The split of UE-AMBR is more like a hard split considering all the PDU sessions, which are a brief limits applied to all the PDU session offloaded to SN. Per PDU session AMBR split is more concrete and specific information to SN from offloaded flow or session point of view.

Proposal 1): MN decides the split of session AMBR and indicates the portion of session AMBR to be respected by the SN.

Secondly, whether information on the split of session AMBR for MN and SN should be indicated to SMF/UPF or not. On this issue, from UPF point of view if the information is available, it is better to control the bit rate limit of MN flows and that of SN flows respectively. But it requires a lot of signaling between RAN Node and AMF/SMF/UPF especially in case that the MN decides to change the split of session AMBR. The benefit needs to be reconsidered since the flows can be moved back and forward often between MN and SN, for which the signaling to CN may happen very often.

Proposal 2): The split result of session AMBR for MN and SN is not indicated to AMF/SMF/UPF.

3. Conclusion

In this contribution, the session AMBR issues were investigated. The following proposals are suggested to RAN3:

Proposal 1): MN decides the split of session AMBR and indicates the portion of session AMBR to be respected by the SN.

Proposal 2): The split result of session AMBR for MN and SN is not indicated to AMF/SMF/UPF.

Proposal 3): To adopt the Stage 2 change for TS 37.340.

4. References

- RP-170855, “NR Radio Access Technology WI”, NTT DOCOMO

- TR 38.801

- R3-174928, “Stage 2: TP on UE-AMBR for EN-DC” LGE

Text proposal for TS 37.340

************************* First change, the unchange text is skipped********************************

8.1	QoS aspects

In EN-DC, the E-UTRAN QoS framework defined in TS 36.300 [2] applies:

-	An S1-U bearer is established between the EPC and the SN for SN terminated bearers;

-	An X2-U bearer is established between the MN and the SN for split bearers, MN terminated SCG bearers and SN terminated MCG bearers;

-	MN terminated and SN terminated bearers may have either MCG or SCG radio resources or both, MCG and SCG radio resources, established;

-	The MN decides the split of DL UE AMBR limits and UL UE AMBR limits among the MN and the SN respectively, and indicates DL UE AMBR limits and UL UE AMBR limits to be respected by the SN;

-	The node that hosts the PDCP entity enforces the respective DL UE AMBR limits;

-	Each node enforces the respective UL UE AMBR limits. For SN terminated bearers, if the node is not configured to serve the uplink, it ignores the indicated UL UE-AMBR.

In MR-DC with 5GC:

-	The NG-RAN QoS framework defined in TS 38.300 [3] applies;

-	QoS flows belonging to the same PDU session may be mapped to different bearer types (see subclause 4.2.2) and as a result there may be two different SDAP entities configured for the same PDU session: one at the MN and another one at the SN, in which case the MN decides which QoS flows are assigned to the SDAP entity in the SN;

-	The MN or SN node that hosts the SDAP entity for a given QoS flow decides how to map the QoS flow to DRBs;

-	If the SDAP entity for a given QoS flow is hosted by the MN and the MN decides that SCG resources are to be configured it provides QoS flow to DRB mapping information and the respective per QoS flow information to the SN;

-	If the SDAP entity for a given QoS flow is hosted by the SN, the MN provides sufficient QoS related information to enable the SN to configure appropriate SCG resources and to request the configuration of appropriate MCG resources. The MN may offer MCG resources to the SN and may indicate for GBR QoS flows the amount offered to the SN on a per QoS flow level;

-	MN decides the split of session AMBR and indicates the SN Session AMBR to be respected by the SN.

To support PDU sessions mapped to different bearer types, MR-DC with 5GC provides the possibility for the MN to request the 5GC:

-	For some PDU sessions of a UE: Direct the User Plane traffic of the whole PDU session either to the MN or to the SN. In that case, there is a single NG-U tunnel termination at the NG-RAN for such PDU session.

-	The MN may request to change this assignment during the life time of the PDU session.

-	For some other PDU sessions of a UE: Direct the User Plane traffic of a subset of the QoS flows of the PDU session to the SN (respectively MN) while the rest of the QoS flows of the PDU session is directed to the MN (respectively SN). In that case, there are two NG-U tunnel terminations at the NG-RAN for such PDU session.

-	The MN may request to change this assignment during the life time of the PDU session.