3GPP TSG-RAN WG3 #99   	R3-181159

# Athens, Greece, 26 February-2 March 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.425 | CR | 0011 | rev |  | Current version: | 15.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on how to acquire status of re-transmitted packets |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NTT DOCOMO,INC. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  |  | Date: |  | 2018-1-12 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Scheduling delay may happen on split bearer when the amount of data is small. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Define indication from the node hosting PDCP to corresponding node i.e. “data exists” and “No data exists” for the bearer. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Avoiding scheduling delay would be difficult. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.5.2.1, 5.5.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  | Some changes are overlapped with the already agreed CR(R3-180707). |  |  |  |  |  |  |  |  |

----Unchanged sections are omitted---

5.5.2	Frame format for the NR user plane protocol

5.5.2.1	DL USER DATA (PDU Type 0)

This frame format is defined to allow the corresponding node to detect lost NR-U packets and is associated with the transfer of a Downlink NR PDCP PDU.

The following shows the respective DL USER DATA frame.

| Bits |  |  |  |  |  |  |  | Number of Octets |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |  |
| PDU Type (=0) |  |  |  | spare | DL Discard Blocks | DL Flush | Report polling | 1 |
|  |  |  |  |  |  | Data exists Indication |  |  |
| NR-U Sequence Number |  |  |  |  |  |  |  | 3 |
| DL discard NR PDCP PDU SN |  |  |  |  |  |  |  | 3 |
| DL discard Number of blocks |  |  |  |  |  |  |  | 1 |
| DL discard NR PDCP PDU SN start (first block) |  |  |  |  |  |  |  | 3 |
| Discarded Block size (first block) |  |  |  |  |  |  |  | 1 |
| … |  |  |  |  |  |  |  |  |
| DL discard NR PDCP PDU SN start (last block) |  |  |  |  |  |  |  | 3 |
| Discarded Block size (last block) |  |  |  |  |  |  |  | 1 |
| Spare extension |  |  |  |  |  |  |  | 0-4 |

Figure 5.5.2.1-1: DL USER DATA (PDU Type 0) Format

----Unchanged sections are omitted---

5.5.3	Coding of information elements in frames

----Unchanged sections are omitted---

5.5.3.X	Data exists Indication

Description: This parameter indicates whether the node hosting PDCP has some packets for the concerned data bearer.

Value range: {0= the node hosting PDCP has no packet for the concerned data bearer, 1= the node hosting PDCP has some packets for the concerned data bearer}.

Field length: 1 bit.

----Unchanged sections are omitted---