3GPP TSG RAN WG3 Meeting #99NR 	Tdoc R3-18xxxx

Athens, Greece, 26th February – 2nd March 2018

Agenda Item:	10.10.3

Source:	Ericsson

Title:	(TP for SA BL CR) gNB-DU admission control

Document for:	pCR TS 38.473 (TP for SA BL CR)

# Introduction

At RAN3 #98, there was a discussion on how the gNB-CU and gNB-DU can exchange information in case the UE is not admitted during the initial access procedure. In this contribution, we propose some enhancements to such procedures and propose to introduce a new gNB-DU Admission Result IE in the Initial UE RRC Message Transfer for this purpose.

# Discussion

At RAN3 #98 a discussion took place concerning the case that the gNB-DU does not admit the UE and communicates this to the gNB-CU. RAN3 decided that if the gNB-DU does not admit the UE, the gNB-DU shall not include the DU to CU RRC Information IE. While this approach covers the case where the gNB-DU has taken the drastic decision not to serve the UE, the solution does not allow the gNB-CU to reject the UE in case the gNB-DU is in overload. In the latter cases, the gNB-DU may serve UEs that access the system with high priority establishment causes, but it may not be able to serve all other UEs (the majority of UEs accessing for non-prioritized services). It is therefore suggested to enhance the mechanism by which the gNB-DU communicates to gNB-CU the results of its preliminary UE admission by adding a new IE in the F1: INITIAL UL RRC MESSAGE TRANSFER. This indication offers the possibility of a better and more streamlined use of the resources. As a result, the UEs with higher priority could be served in cases of high load.

Proposal 1: 	Introduce a new gNB-DU Admission Result IE in the Initial UE RRC Message Transfer.

With the above proposal it is possible to carry the results of the gNB-DU admission in the F1: INITIAL UL RRC MESSAGE to the gNB-CU and it is possible, based on such gNB-DU admission information, to allow the gNB-CU to admit the UE (i.e., to send an RRCConnectionSetup) or to reject the UE.

Figure 1 explains the message sequence chart in more details. It is worth describing the information included in step 2 of Figure 1. This information tells the gNB-CU one of possible outcome of the UE admission at the gNB-DU such as:

- The gNB-DU did not allocate resources to the UE i.e., the UE is allowed to signal MSG3 via SRB1, but it is not admitted to further resources in the gNB-DU. This information implies an RRCConnectionReject from the gNB-CU

- The gNB-DU can admit the UE i.e., allocate resources for low layer signalling, but the gNB-DU is under a high load situation. The gNB-CU can therefore judge from the RRC establishment cause whether to admit the UE (e.g., if the UE is connecting for an emergency call) or whether to reject the UE

- The gNB-DU has admitted the UE without any particular event to be signalled to the gNB-CU. In this case the gNB-DU can simply state to the gNB-CU that the UE was admitted.

Figure 1: Handling of RRC rejections or RRC connections setup

Based on the above we propose that a procedural text description of how to interpret the results of the gNB-DU admission is introduced in TS 38.473.

Proposal 2: 	A description of how to interpret the results of the gNB-DU admission should be given in TS 38.473

# Conclusion

In this contribution, we discussed how the gNB-CU can inform the gNB-DU in case that a UE is not admitted during the initial access procedure and ways to as early as possible utilize the results of the gNB-DU admission and made the following proposals

Proposal 1:	Introduce a new gNB-DU Admission Result IE in the Initial UE RRC Message Transfer.

Proposal 2: 	A description of how to interpret the results of the gNB-DU admission should be given in TS 38.473

# References

TS 38.473 V1.0.0, F1 Application Protocol

# Annex 1: TP for the BL CR to TS 38.473

Start of Text Proposal 1 for TS 38.473

## 8.4	RRC Message Transfer procedures

The purpose of the RRC message transfer procedure is to transfer RRC message over F1.

### 8.4.1 	Initial UL RRC Message Transfer

#### 8.4.1.1	General

The purpose of the Initial UL RRC Message Transfer procedure is to transfer the initial RRC message as an UL PDCP-PDU to the gNB-CU.

#### 8.4.1.2	Successful operation

Figure 8.4.1.2-1: Initial UL RRC Message Transfer procedure.

If the gNB-DU Admission Result IE is included in the INITIAL UL RRC MESSAGE TRANSFER, and if the IE is set to “Admitted”, the gNB-CU shall assume that the gNB-DU did not detect any issues with serving such UE.

If the gNB-DU Admission Result IE is included in the INITIAL UL RRC MESSAGE TRANSFER, and if the IE is set to “Rejected”, the gNB-CU shall reject the UE under the assumption that the gNB-DU is not able to serve such UE

If the gNB-DU Admission Result IE is included in the INITIAL UL RRC MESSAGE TRANSFER, and if the IE is set to “Admitted High Load”, the gNB-CU may admit the UE if it has sufficient priority.

#### 8.4.1.3	Abnormal Conditions

Not applicable.

Start of Text Proposal 2 for TS 38473

### 9.2.3 	RRC Message Transfer messages

#### 9.2.3.1	INITIAL UL RRC MESSAGE TRANSFER

This message is sent by the gNB-DU to transfer the initial layer 3 message to the gNB-CU over the F1 interface.

Direction: gNB-DU gNB-CU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| NCGI | M |  | 9.3.1.12 | NG-RAN Cell Global Identifier (NCGI) | YES | reject |
| C-RNTI | M |  | FFS | C-RNTI allocated at the gNB-DU | YES | reject |
| RRC-Container | M |  | 9.3.1.6 |  | YES | reject |
| DU to CU RRC Container | O |  | OCTET STRING | CellGroupConfig IE as defined in subclause 6.3.2 in TS 38.331. Required at least to carry SRB1 configuration | YES | reject |
| gNB-DU Admission Result | M |  | 9.3.1.y |  | YES | reject |

Start of Text Proposal 3 for TS 38.473

#### 9.3.1.y 	gNB-DU Admission Result

The gNB-DU Admission Result provides results of initial UE admission at the gNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| gNB-DU Admission Result | M |  | ENUMERATED (Admitted, Rejected, Admitted High Load, …) | Description of UE admission results after signalling of RRCConnectionRequest |

End of Text Proposal for TS 38473