3GPP TSG RAN WG3 NR #99	Tdoc R3-181278

Athens, Greece, 26 February - 2 March 2018

Agenda Item:	10.10.7

Source:	Ericsson

Title:	(TP for SA BL CR) RRC Container in UE Context Setup Request

Document for:	pCR TS 38.473 (TP for SA BL CR)

# Introduction

In this contribution, we observe that there is currently an inconsistency between the stage 2 and stage 3 specifications for the F1 interface. To solve this inconsistency, we propose to introduce an RRC container to the F1AP UE Context Setup Request message.

# Discussion

There is currently an inconsistency between the stage 2 specification - BL CR for TS 38.401 [1] - and the stage 3 specification - BL CR for TS 38.473 [2] - for the F1 interface. In [1] the UE Initial Access Procedure is described (subclause 8.1). It can be observed that in step 9 the gNB-CU sends the F1AP UE Context Setup Request message to the gNB-DU together with the RRC Security Mode Command. In the description of the procedure it is stated the following:

- “The gNB-CU sends the UE CONTEXT SETUP REQUEST message to establish the UE context in the gNB-DU. In this message, it may also encapsulate the RRC SECURITY MODE COMMAND message.”

To enable this, an RRC container needs to be included in the F1AP UE Context Setup Request message. However, this container is currently not present in the tabular format for the F1AP UE Context Setup Request message in TS 38.473 (nor in the BL CR for SA to TS 38.473). In the following, we provide a text proposal for the BL CR to TS 38.473 to introduce this RRC container.

Proposal 1	RAN3 is kindly to agree with the text proposal in Annex I.

# Conclusion

In this contribution, we proposed the addition of an RRC container to the F1AP UE Context Setup Request message, which allows to encapsulate the RRC Security Mode Command.

Proposal 1	RAN3 is kindly to agree with the text proposal in Annex I.

# References

[1]	Baseline CR for SA to TS 38.401.

[2] 	Baseline CR for SA to TS 38.473.

# Annex I: TP for the SA BL CR to TS 38.473

Start of Text Proposal for TS 38.473

#### 9.2.2.1	UE CONTEXT SETUP REQUEST

This message is sent by the gNB-CU to request the setup of a UE context.

Direction: gNB-CU  gNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | O |  | 9.3.1.5 |  | YES | ignore |
| Target cell ID (FFS) | FFS |  | FFS |  | FFS | FFS |
| PSCell ID | O |  | NCGI 9.3.1.12 | PSCell Identifier in SgNB | YES | Ignore |
| Cell Type [FFS] | M |  | FFS | Includes the cell type (PSCell, PCell, normal Cell) [FFS] | YES | reject |
| CU to DU RRC Information | M |  | 9.3.1.25 |  | YES | reject |
| DRX Cycle | O |  | DRX Cycle 9.3.1.24 |  | YES | ignore |
| Resource Coordination Transfer Container | O |  | OCTET STRING | Includes the MeNB Resource Coordination Information IE as defined in subclause 9.1.X.1 of TS 36.423 [9]. | YES | reject |
| SCell To Be Setup List |  | 0..1 |  |  | YES | ignore |
| >SCell to Be Setup Item IEs |  | 0 .. <maxnoofSCells> |  |  | EACH | ignore |
| >>SCell ID | M |  | NCGI 9.3.1.12 | SCell Identifier in SgNB | - | - |
| SRB to Be Setup List |  | 0..1 |  |  |  |  |
| >SRB to Be Setup Item IEs |  | 1 .. <maxnoofSRBs>] |  |  |  |  |
| >SRB ID | M |  | 9.3.1.7 |  |  |  |
| DRB to Be Setup List |  | 1 |  |  | YES | reject |
| >DRB to Be Setup Item IEs |  | 1 .. <maxnoofDRBs> |  |  | EACH | reject |
| >>DRB ID | M |  | 9.3.1.8 |  | - |  |
| >>DRB QoS profile (FFS) |  |  |  |  | - |  |
| >>Flow-level QoS profile (FFS) |  |  |  |  |  |  |
| >>Flow-to-DRB mapping (FFS) |  |  |  |  |  |  |
| >>E-UTRAN QoS | O |  | 9.3.1.19 | Shall be used for EN-DC case to convey E-RAB Level QoS Parameters |  |  |
| >> Tunnels to be setup List |  | 1 |  |  |  |  |
| >>>Tunnels to Be Setup Item IEs |  | 1 .. <maxnoofULTunnels> |  |  |  |  |
| >>>>UL GTP Tunnel Endpoint | M |  | GTP Tunnel Endpoint 9.3.2.1 | gNB-CU endpoint of the F1 transport bearer. For delivery of UL PDUs. | - | - |
| RRC-Container | O |  | 9.3.1.6 |  | YES | reject |

| Range bound | Explanation |
| --- | --- |
| maxnoofSCells | Maximum no. of SCells allowed towards one UE, the maximum value is 64. |
| maxnoofSRBs | Maximum no. of SRB allowed towards one UE, the maximum value is 8. |
| maxnoofDRBs | Maximum no. of DRB allowed towards one UE, the maximum value is 64. |
| maxnoofULTunnels | Maximum no. of tunnels allowed towards one DRB, the maximum value is 2. |

End of Text Proposal for TS 38.473