3GPP TSG RAN WG3 NR #99	Tdoc R3-181319

Athens, Greece, 26 February – 2 March 2018

Agenda Item:	31.1.3

Source:	Ericsson

Title:	(TP for NSA BL CR) Correction on SRB and DRB identity

Document for:	pCR TS 38.473 (TP for NSA BL CR)

# Introduction

In this contribution we propose to clarify the meaning of the DRB/SRB identifiers over the F1 interface.

# Discussion

In the F1AP specifications the DRB ID IE and the SRB ID IE are utilized to uniquely identify a DRB/SRB for a given UE [1]. However, the relationship between the DRB ID IE and the SRB ID IE that are used over the F1 and the DRB-Identity and the SRB-Identity that are used over RRC is not clearly specified. In TS 38.331 [2], the DRB-Identity and the SRB-Identity are defined as follows.

#### – 	DRB-Identity

The IE DRB-Identity is used to identify a DRB used by a UE.

-- ASN1START

-- TAG-DRB-IDENTITY-START

DRB-Identity ::=					INTEGER (4..32)

-- TAG-DRB-IDENTITY-STOP

-- ASN1STOP

#### –	SRB-Identity

The IE SRB-Identity is used to identify a Signalling Radio Bearer (SRB) used by a UE.

-- ASN1START

-- TAG-SRB-IDENTITY-START

SRB-Identity ::=					INTEGER (1..3)

-- TAG-SRB-IDENTITY-STOP

-- ASN1START

The question is then raised on whether there should be a one-to-one mapping between the DRB/SRB identities used over the F1 and over RRC. We currently see no obvious reason for having a different mapping. In addition, the one-to-one mapping may simplify the “book keeping” at the gNB-DU side (i.e., no need to have a matching table to map the DRB/SRB identities over F1 and RRC).

Proposal 1: 	Clarify that the DRB ID IE and the SRB ID IE correspond respectively to the DRB-Identity and the SRB-Identity used in RRC.

If the proposal above is agreed, then we should also align the range of the DRB/SRB identities over the F1 to match the range of DRB/SRB identities over RRC.

Proposal 2: 	Adjust the range of the DRB/SRB identities over the F1 to match the range of DRB/SRB identities over RRC.

# Conclusion

In this contribution, we discussed the DRB/SRB identities over F1.

Proposal 1: 	Clarify that the DRB ID IE and the SRB ID IE correspond respectively to the DRB-Identity and the SRB-Identity used in RRC.

Proposal 2: 	Adjust the range of the DRB/SRB identities over the F1 to match the range of DRB/SRB identities over RRC.

# References

TS 38.473 V15.0.0, F1 Application Protocol

TS 38.331 V15.0.0, NR Radio Recourse Control Protocol (RRC)

# Annex I

Start of Text Proposal 1 for TS 38.473

#### 9.3.1.7	SRB ID

This IE uniquely identifies a SRB for a UE.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| SRB ID | M |  | INTEGER (1..3, ...) | Corresponds to the SRB-Identity defined in TS 38.331 [x]. |

#### 9.3.1.8	DRB ID

This IE uniquely identifies a DRB for a UE.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| DRB ID | M |  | INTEGER (4..32, ...) | Corresponds to the DRB-Identity defined in TS 38.331 [x]. |

End of Text Proposal for TS 38.473