3GPP TSG-RAN WG3 #99	Tdoc R3-181406

Athens, Greece, 26 February – 2 March 2018

Agenda Item:	31.3.1

Source:	Ericsson

Title:	(TP for NSA BL CR) CU-CP Name over F1-C

Document for:	pCR TS 38.473 (TP for NSA BL CR)

# Introduction

This contribution proposes to complement the already agreed signalling of DU name over F1-C with the corresponding CU name.

# Annex I: TP for 38.473 (on EN-DC BL CR)

Start of Text Proposal for TS 38.473

#### 9.2.1.5	F1 SETUP RESPONSE

This message is sent by the gNB-CU to transfer information for a TNL association.

Direction: gNB-CU  gNB-DU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| gNB-CU Name | O |  | PrintableString(SIZE(1..150,…)) | Human readable name of the gNB-CU. | YES | ignore |
| Cells to be Activated List |  | 0.. 1 |  |  |  |  |
| >Cells to be Activated List Item |  | 1.. <maxCellingNBDU> |  | List of cells to be activated | YES | reject |
| >>NCGI | M |  | 9.3.1.12 |  | - | - |
| >>PCI | O |  | INTEGER (0..1007) | Physical Cell ID | - | - |

| Range bound | Explanation |
| --- | --- |
| maxCellingNBDU | Maximum no. cells that can be served by a gNB-DU. Value is 512. |

Start of Text Proposal for TS 38.473

### 9.4.4	PDU Definitions

-- **************************************************************

--

-- PDU definitions for F1AP.

--

-- **************************************************************

F1AP-PDU-Contents {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) f1ap (3) version1 (1) f1ap-PDU-Contents (1) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- **************************************************************

--

-- IE parameter types from other modules.

--

-- **************************************************************

IMPORTS

Cause,

Cells-Failed-to-be-Activated-List-Item,

Cells-to-be-Activated-List-Item,

Cells-to-be-Deactivated-List-Item,

CriticalityDiagnostics,

C-RNTI,

CUtoDURRCInformation,

DRBID,

DRBs-FailedToBeModifiedConf-Item,

DRBs-FailedToBeModified-Item,

DRBs-FailedToBeSetup-Item,

DRBs-FailedToBeSetupMod-Item,

DRBs-ModifiedConf-Item,

DRBs-Modified-Item,

DRBs-Required-ToBeModified-Item,

DRBs-Required-ToBeReleased-Item,

DRBs-Setup-Item,

DRBs-SetupMod-Item,

DRBs-ToBeModified-Item,

DRBs-ToBeReleased-Item,

DRBs-ToBeSetup-Item,

DRBs-ToBeSetupMod-Item,

DRXCycle,

DUtoCURRCInformation,

EUTRANQoS,

GNB-CU-F1AP-ID,

GNB-DU-F1AP-ID,

GNB-DU-ID,

GNB-DU-Served-Cells-Item,

GNB-DU-System-Information,

GNB-DU-Name,

GNB-CU-Name,

GTPTunnelEndpoint,

NCGI,

PCI,

ResourceCoordinationTransferContainer,

RRCContainer,

SCellIndex,

SCell-ToBeRemoved-Item,

SCell-ToBeSetup-Item,

SCell-ToBeSetupMod-Item,

Served-Cell-Information,

Served-Cells-To-Add-Item,

Served-Cells-To-Delete-Item,

Served-Cells-To-Modify-Item,

SRBID,

SRBs-FailedToBeSetup-Item,

SRBs-FailedToBeSetupMod-Item,

SRBs-Required-ToBeReleased-Item,

SRBs-Setup-Item,

SRBs-ToBeReleased-Item,

SRBs-ToBeSetup-Item,

SRBs-ToBeSetupMod-Item,

TimeToWait,

TransactionID,

TransmissionStopIndicator,

UE-associatedLogicalF1-ConnectionItem,

ULTunnels-ToBeSetup-Item

FROM F1AP-IEs

PrivateIE-Container{},

ProtocolExtensionContainer{},

ProtocolIE-Container{},

ProtocolIE-ContainerPair{},

ProtocolIE-SingleContainer{},

F1AP-PRIVATE-IES,

F1AP-PROTOCOL-EXTENSION,

F1AP-PROTOCOL-IES,

F1AP-PROTOCOL-IES-PAIR

FROM F1AP-Containers

id-Cause,

id-Cells-Failed-to-be-Activated-List,

id-Cells-Failed-to-be-Activated-List-Item,

id-Cells-to-be-Activated-List,

id-Cells-to-be-Activated-List-Item,

id-Cells-to-be-Deactivated-List,

id-Cells-to-be-Deactivated-List-Item,

id-CriticalityDiagnostics,

id-C-RNTI,

id-CUtoDURRCInformation,

id-DRBs-FailedToBeModifiedConf-Item,

id-DRBs-FailedToBeModifiedConf-List,

id-DRBs-FailedToBeModified-Item,

id-DRBs-FailedToBeModified-List,

id-DRBs-FailedToBeSetup-Item,

id-DRBs-FailedToBeSetup-List,

id-DRBs-FailedToBeSetupMod-Item,

id-DRBs-FailedToBeSetupMod-List,

id-DRBs-ModifiedConf-Item,

id-DRBs-ModifiedConf-List,

id-DRBs-Modified-Item,

id-DRBs-Modified-List,

id-DRBs-Required-ToBeModified-Item,

id-DRBs-Required-ToBeModified-List,

id-DRBs-Required-ToBeReleased-Item,

id-DRBs-Required-ToBeReleased-List,

id-DRBs-Setup-Item,

id-DRBs-Setup-List,

id-DRBs-SetupMod-Item,

id-DRBs-SetupMod-List,

id-DRBs-ToBeModified-Item,

id-DRBs-ToBeModified-List,

id-DRBs-ToBeReleased-Item,

id-DRBs-ToBeReleased-List,

id-DRBs-ToBeSetup-Item,

id-DRBs-ToBeSetup-List,

id-DRBs-ToBeSetupMod-Item,

id-DRBs-ToBeSetupMod-List,

id-DRXCycle,

id-DUtoCURRCInformation,

id-gNB-CU-F1AP-ID,

id-gNB-DU-F1AP-ID,

id-gNB-DU-ID,

id-GNB-DU-Served-Cells-Item,

id-gNB-DU-Served-Cells-List,

id-gNB-DU-Name,

id-gNB-CU-Name,

id-oldgNB-DU-F1AP-ID,

id-ResetType,

id-ResourceCoordinationTransferContainer,

id-RRCContainer,

id-SCell-ToBeRemoved-Item,

id-SCell-ToBeRemoved-List,

id-SCell-ToBeSetup-Item,

id-SCell-ToBeSetup-List,

id-SCell-ToBeSetupMod-Item,

id-SCell-ToBeSetupMod-List,

id-Served-Cells-To-Add-Item,

id-Served-Cells-To-Add-List,

id-Served-Cells-To-Delete-Item,

id-Served-Cells-To-Delete-List,

id-Served-Cells-To-Modify-Item,

id-Served-Cells-To-Modify-List,

id-SpCell-ID,

id-SRBID,

id-SRBs-FailedToBeSetup-Item,

id-SRBs-FailedToBeSetup-List,

id-SRBs-FailedToBeSetupMod-Item,

id-SRBs-FailedToBeSetupMod-List,

id-SRBs-Required-ToBeReleased-Item,

id-SRBs-Required-ToBeReleased-List,

id-SRBs-Setup-Item,

id-SRBs-Setup-List,

id-SRBs-ToBeReleased-Item,

id-SRBs-ToBeReleased-List,

id-SRBs-ToBeSetup-Item,

id-SRBs-ToBeSetup-List,

id-SRBs-ToBeSetupMod-Item,

id-SRBs-ToBeSetupMod-List,

id-TimeToWait,

id-TransactionID,

id-TransmissionStopIndicator,

id-UE-associatedLogicalF1-ConnectionItem,

id-UE-associatedLogicalF1-ConnectionListResAck,

maxCellingNBDU,

maxnoofDRBs,

maxnoofErrors,

maxnoofIndividualF1ConnectionsToReset,

maxnoofSCells,

maxnoofSRBs

FROM F1AP-Constants;

Start of Text Proposal for TS 38.473

-- **************************************************************

--

#### -- F1 SETUP ELEMENTARY PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

-- F1 Setup Request

--

-- **************************************************************

F1SetupRequest ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { {F1SetupRequestIEs} },

...

}

F1SetupRequestIEs F1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID				CRITICALITY reject	TYPE TransactionID				PRESENCE mandatory	}|

{ ID id-gNB-DU-ID					CRITICALITY reject	TYPE GNB-DU-ID					PRESENCE mandatory	}|

{ ID id-gNB-DU-Name						CRITICALITY ignore	TYPE	 GNB-DU-Name					PRESENCE optional	}|

{ ID id-gNB-DU-Served-Cells-List		CRITICALITY reject	TYPE	 GNB-DU-Served-Cells-List	PRESENCE mandatory	},

...

}

GNB-DU-Served-Cells-List 	::= SEQUENCE (SIZE(1.. maxCellingNBDU)) OF ProtocolIE-SingleContainer { { GNB-DU-Served-Cells-ItemIEs } }

GNB-DU-Served-Cells-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-GNB-DU-Served-Cells-Item		CRITICALITY reject	TYPE		GNB-DU-Served-Cells-Item	PRESENCE mandatory	},

...

}

-- **************************************************************

--

-- F1 Setup Response

--

-- **************************************************************

F1SetupResponse ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { {F1SetupResponseIEs} },

...

}

F1SetupResponseIEs F1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID				CRITICALITY reject	TYPE TransactionID					PRESENCE mandatory	}|

{ ID id-gNB-CU-Name						CRITICALITY ignore	TYPE	 GNB-CU-Name					PRESENCE optional	}|

{ ID id-Cells-to-be-Activated-List	CRITICALITY reject	TYPE	 Cells-to-be-Activated-List		PRESENCE optional	},

...

}

Cells-to-be-Activated-List	::= SEQUENCE (SIZE(1.. maxCellingNBDU))	OF ProtocolIE-SingleContainer { { Cells-to-be-Activated-List-ItemIEs } }

Cells-to-be-Activated-List-ItemIEs	F1AP-PROTOCOL-IES::= {

{ ID id-Cells-to-be-Activated-List-Item				CRITICALITY reject	TYPE	 Cells-to-be-Activated-List-Item						PRESENCE mandatory}	,

...

}

Start of Text Proposal for TS 38.473

#### -- G

GBR-QosInformation ::= SEQUENCE {

e-RAB-MaximumBitrateDL			BitRate,

e-RAB-MaximumBitrateUL			BitRate,

e-RAB-GuaranteedBitrateDL		BitRate,

e-RAB-GuaranteedBitrateUL		BitRate,

iE-Extensions					ProtocolExtensionContainer { { GBR-QosInformation-ExtIEs} } OPTIONAL,

...

}

GBR-QosInformation-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

GNB-CU-F1AP-ID		::= INTEGER (0..4294967295)

GNB-DU-F1AP-ID		::= INTEGER (0..4294967295)

GNB-DU-ID			::= INTEGER (0..68719476735)

GNB-DU-Served-Cells-Item ::= SEQUENCE {

served-Cell-Information		Served-Cell-Information,

gNB-DU-System-Information	GNB-DU-System-Information,

iE-Extensions				ProtocolExtensionContainer { { GNB-DU-Served-Cells-ItemExtIEs} }	OPTIONAL,

...

}

GNB-DU-Served-Cells-ItemExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

GNB-DU-System-Information ::= SEQUENCE {

mIB-message		MIB-message,

sIB1-message		SIB1-message,

iE-Extensions					ProtocolExtensionContainer { { GNB-DU-System-Information-ExtIEs } } OPTIONAL,

...

}

GNB-DU-System-Information-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

GNB-DU-Name ::= PrintableString(SIZE(1..150,...))

GNB-CU-Name ::= PrintableString(SIZE(1..150,...))

GTP-TEID				::= OCTET STRING (SIZE (4))

GTPTunnelEndpoint ::= SEQUENCE {

transportLayerAddress				TransportLayerAddress,

gTP-TEID							GTP-TEID,

iE-Extensions						ProtocolExtensionContainer { { GTPTunnelEndpoint-ExtIEs} }	OPTIONAL,

...

}

GTPTunnelEndpoint-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

End of Text Proposal for TS 38.473

Start of Text Proposal for TS 38.473

-- **************************************************************

--

-- IEs

--

-- **************************************************************

id-Cause											ProtocolIE-ID ::= 0

id-CriticalityDiagnostics							ProtocolIE-ID ::= 1

id-gNB-DU-F1AP-ID									ProtocolIE-ID ::= 2

id-gNB-CU-F1AP-ID									ProtocolIE-ID ::= 3

id-ResetType											ProtocolIE-ID ::= 4

id-TimeToWait										ProtocolIE-ID ::= 5

id-UE-associatedLogicalF1-ConnectionItem 			ProtocolIE-ID ::= 6

id-UE-associatedLogicalF1-ConnectionListResAck		ProtocolIE-ID ::= 7

id-RRCContainer										ProtocolIE-ID ::= 8

id-SRBID												ProtocolIE-ID ::= 9

id-gNB-DU-ID											ProtocolIE-ID ::= 10

id-gNB-DU-Name											ProtocolIE-ID ::= 11

id-NRCellID											ProtocolIE-ID ::= 12

id-gNB-DU-Served-Cells-List							ProtocolIE-ID ::= 14

id-Cells-to-be-Activated-List						ProtocolIE-ID ::= 15

id-Served-Cells-To-Add-List							ProtocolIE-ID ::= 16

id-Served-Cells-To-Modify-List						ProtocolIE-ID ::= 17

id-Served-Cells-To-Delete-List						ProtocolIE-ID ::= 18

id-Cells-to-be-Deactivated-List						ProtocolIE-ID ::= 19

id-Cells-Failed-to-be-Activated-List					ProtocolIE-ID ::= 20

id-TransactionID										ProtocolIE-ID ::= 21

id-SpCell-ID											ProtocolIE-ID ::= 28

id-SRBs-ToBeSetup-List								ProtocolIE-ID ::= 30

id-DRBs-ToBeSetup-List								ProtocolIE-ID ::= 31

id-CUtoDURRCInformation								ProtocolIE-ID ::= 36

id-DUtoCURRCInformation								ProtocolIE-ID ::= 37

id-SCell-ToBeSetup-List								ProtocolIE-ID ::= 40

id-ResourceCoordinationTransferContainer				ProtocolIE-ID ::= 41

id-DRBs-ToBeModified-List							ProtocolIE-ID ::= 42

id-DRBs-ToBeReleased-List							ProtocolIE-ID ::= 43

id-DRBs-Modified-List								ProtocolIE-ID ::= 44

id-DRBs-FailedToBeModified-List						ProtocolIE-ID ::= 46

id-DRXCycle											ProtocolIE-ID ::= 48

id-DRBs-Setup-List									ProtocolIE-ID ::= 49

id-SRBs-Setup-List									ProtocolIE-ID ::= 50

id-DRBs-FailedToBeSetup-List							ProtocolIE-ID ::= 51

id-SRBs-FailedToBeSetup-List							ProtocolIE-ID ::= 52

id-TransmissionStopIndicator							ProtocolIE-ID ::= 55

id-DRBs-Required-ToBeModified-List					ProtocolIE-ID ::= 56

id-DRBs-Required-ToBeReleased-List					ProtocolIE-ID ::= 57

id-SRBs-Required-ToBeReleased-List					ProtocolIE-ID ::= 58

id-oldgNB-DU-F1AP-ID									ProtocolIE-ID ::= 59

id-SRBs-ToBeReleased-List							ProtocolIE-ID ::= 60

id-DRBs-ModifiedConf-List							ProtocolIE-ID ::= 61

id-SCell-ToBeSetupMod-List							ProtocolIE-ID ::= 63

id-SRBs-ToBeSetupMod-List							ProtocolIE-ID ::= 64

id-DRBs-ToBeSetupMod-List							ProtocolIE-ID ::= 65

id-DRBs-FailedToBeModifiedConf-List					ProtocolIE-ID ::= 69

id-GNB-DU-Served-Cells-Item							ProtocolIE-ID ::= 71

id-Cells-to-be-Activated-List-Item					ProtocolIE-ID ::= 72

id-Served-Cells-To-Modify-Item						ProtocolIE-ID ::= 73

id-Served-Cells-To-Delete-Item						ProtocolIE-ID ::= 74

id-Cells-to-be-Deactivated-List-Item					ProtocolIE-ID ::= 75

id-Cells-Failed-to-be-Activated-List-Item			ProtocolIE-ID ::= 76

id-SCell-ToBeSetup-Item								ProtocolIE-ID ::= 77

id-SRBs-ToBeSetup-Item								ProtocolIE-ID ::= 78

id-DRBs-Setup-Item									ProtocolIE-ID ::= 80

id-SRBs-FailedToBeSetup-Item							ProtocolIE-ID ::= 82

id-DRBs-FailedToBeSetup-Item							ProtocolIE-ID ::= 83

id-SCell-ToBeSetupMod-Item							ProtocolIE-ID ::= 84

id-SRBs-ToBeSetupMod-Item							ProtocolIE-ID ::= 85

id-DRBs-ToBeSetupMod-Item							ProtocolIE-ID ::= 86

id-DRBs-ToBeModified-Item							ProtocolIE-ID ::= 87

id-SRBs-ToBeReleased-Item							ProtocolIE-ID ::= 88

id-DRBs-ToBeReleased-Item							ProtocolIE-ID ::= 89

id-DRBs-SetupMod-Item								ProtocolIE-ID ::= 90

id-DRBs-Modified-Item								ProtocolIE-ID ::= 91

id-SRBs-FailedToBeSetupMod-Item						ProtocolIE-ID ::= 92

id-DRBs-FailedToBeSetupMod-Item						ProtocolIE-ID ::= 93

id-DRBs-FailedToBeModified-Item						ProtocolIE-ID ::= 94

id-DRBs-Required-ToBeModified-Item					ProtocolIE-ID ::= 95

id-DRBs-Required-ToBeReleased-Item					ProtocolIE-ID ::= 96

id-SRBs-Required-ToBeReleased-Item					ProtocolIE-ID ::= 97

id-DRBs-ModifiedConf-Item							ProtocolIE-ID ::= 98

id-DRBs-FailedToBeModifiedConf-Item					ProtocolIE-ID ::= 99

id-Served-Cells-To-Add-Item							ProtocolIE-ID ::= 100

id-DRBs-ToBeSetup-Item								ProtocolIE-ID ::= 101

id-SRBs-Setup-Item									ProtocolIE-ID ::= 102

id-DRBs-SetupMod-List								ProtocolIE-ID ::= 103

id-SRBs-FailedToBeSetupMod-List						ProtocolIE-ID ::= 104

id-DRBs-FailedToBeSetupMod-List						ProtocolIE-ID ::= 105

id-C-RNTI											ProtocolIE-ID ::= 106

id-SCell-ToBeRemoved-List							ProtocolIE-ID ::= 107

id-SCell-ToBeRemoved-Item								ProtocolIE-ID ::= 108

id-gNB-CU-Name										ProtocolIE-ID ::= xxx

END

End of Text Proposal for TS 38.473