3GPP TSG-RAN WG3 #99	Tdoc R3-181428

Athens, Greece, 26 February – 2 March 2018

Agenda Item:	23.3

Source:	Ericsson

Title:	Bearer Context procedure

Document for:	pCR TS 38.463

# Introduction

This paper provides the TP for TS 38.463 according to the following email discussion.

Email# 3

Deadline 2018-03-09 1200 CET

(E///)

- keep changes in Sec. 8.3.x

- align with agreements, WAs

- remove all details

- only keep applicable headers in Sec. 9.2.1.x

# Text Proposal for 38.463

Start of Text Proposal 1 for TS 38.463

## 8.1	List of E1AP Elementary Procedures

In the following tables, all EPs are divided into Class 1 and Class 2 EPs (see subclause 3.1 for explanation of the different classes):

Table 1: Class 1 procedures

| Elementary Procedure | Initiating Message | Successful Outcome | Unsuccessful Outcome |
| --- | --- | --- | --- |
|  |  | Response message | Response message |
| Bearer Context Setup | BEARER CONTEXT SETUP REQUEST | BEARER CONTEXT SETUP RESPONSE | BEARER CONTEXT SETUP FAILURE |
| Bearer Context Modification (CU-CP initiated) | BEARER MODIFICATION REQUEST | BEARER CONTEXT MODIFICATION RESPONSE | BEARER CONTEXT MODIFICATION FAILURE |
| Bearer Context Modification Required (CU-UP initiated) | BEARER CONTEXT MODIFICATION REQUIRED | BEARER CONTEXT MODIFICATION CONFIRM |  |
| Bearer Context Release (CU-CP initiated) | BEARER CONTEXT RELEASE COMMAND | BEARER CONTEXT RELEASE COMPLETE |  |

Table 2: Class 2 procedures

| Elementary Procedure | Message |
| --- | --- |
| Bearer Context Release Request (CU-UP initiated) | BEARER CONTEXT RELEASE REQUEST |

## 8.2	Interface Management procedures

Editor’s note: this sub-clause describes the Interface Management procedures.

## 8.3	Bearer Context Management procedures

### 8.3.1	Bearer Context Setup

#### 8.3.1.1	General

The purpose of the Bearer Context Setup procedure is to allow the CU-CP to establish a bearer context in the CU-UP. The procedure uses UE-associated signalling.

#### 8.3.1.2	Successful Operation

Figure 8.3.1.2-1: Bearer Context Setup Request procedure: Successful Operation.

The CU-CP initiates the procedure by sending the BEARER CONTEXT SETUP REQUEST message to the CU-UP. If the CU-UP succeeds to establish the requested resources, it replies to the CU-CP with the BEARER CONTEXT SETUP RESPONSE message.

#### 8.3.1.3	Unsuccessful Operation

Figure 8.3.1.3-1: Bearer Context Setup procedure: Unsuccessful Operation.

If the CU-UP cannot establish the requested bearer context, it should respond with a BEARER CONTEXT SETUP FAILURE message and appropriate cause value.

#### 8.3.1.4	Abnormal Conditions

Not applicable.

### 8.3.2	Bearer Context Modification (CU-CP initiated)

Editor’s note: The description of this procedure is FFS.

### 8.3.3	Bearer Context Modification Required (CU-UP initiated)

Editor’s note: The description of this procedure is FFS.

### 8.3.4	Bearer Context Release (CU-CP initiated)

Editor’s note: The description of this procedure is FFS.

### 8.3.5	Bearer Context Release Request (CU-UP initiated)

Editor’s note: The description of this procedure is FFS.

Start of Text Proposal 3 for TS 38.463

### 9.2.2	Bearer Context Management messages

#### 9.2.1.1	BEARER CONTEXT SETUP REQUEST

This message is sent by the CU-CP to request the CU-UP to setup a bearer context.

Direction: CU-CP  CU-UP

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.X |  | YES | reject |
| CU-CP UE E1AP ID | M |  | 9.3.1.Y |  | YES | reject |

Editor’s note: The details of this message are FFS.

#### 9.2.1.2	BEARER CONTEXT SETUP RESPONSE

This message is sent by the CU-UP to confirm the setup of the requested bearer context,

Direction: CU-UP  CU-CP

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.X |  | YES | reject |
| CU-CP UE E1AP ID | M |  | 9.3.1.Y |  | YES | reject |
| CU-UP UE E1AP ID | M |  | 9.3.1.Z |  | YES | reject |

Editor’s note: The details of this message are FFS.

#### 9.2.1.3	BEARER CONTEXT SETUP FAILURE

This message is sent by the CU-UP to indicate the setup of the bearer context was unsuccessful.

Direction: CU-UP  CU-CP

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.X |  | YES | reject |
| CU-CP UE E1AP ID | M |  | 9.3.1.Y |  | YES | reject |
| CU-UP UE E1AP ID | O |  | 9.3.1.Z |  | YES | ignore |
| Cause | O |  | 9.3.1.a |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.3.1.b |  | YES | ignore |

Editor’s note: The details of this message are FFS.

#### 9.2.1.4	BEARER CONTEXT MODIFICATION REQUEST

Editor’s note: The details of this message are FFS.

#### 9.2.1.5	BEARER CONTEXT MODIFICATION RESPONSE

Editor’s note: The details of this message are FFS.

#### 9.2.1.6	BEARER CONTEXT MODIFICATION FAILURE

Editor’s note: The details of this message are FFS.

#### 9.2.1.7	BEARER CONTEXT MODIFICATION REQUIRED

Editor’s note: The details of this message are FFS.

#### 9.2.1.8	BEARER CONTEXT MODIFICATION CONFIRM

Editor’s note: The details of this message are FFS.

#### 9.2.1.9	BEARER CONTEXT RELEASE COMMAND

Editor’s note: The details of this message are FFS.

#### 9.2.1.10	BEARER CONTEXT RELEASE COMPLETE

Editor’s note: The details of this message are FFS.

#### 9.2.1.11	BEARER CONTEXT RELEASE REQUEST

Editor’s note: The details of this message are FFS.

End of Text Proposal for TS 38.463