3GPP TSG-RAN WG3#99bis	R3-181672

Sanya, China, 16 – 20 April 2018

Agenda item:	10.1.1

Source:	Nokia, Nokia Shanghai Bell

Title:	(TP for TS 38.473 SA BL CR) Handling of Notification Control over F1 interface

Document for:	Discussion and Decision

# 1	Description

Notification control is an indication allowing to report when a GBR flow cannot fulfil any longer the GFBR (Guaranteed flow bit rate). pCR for handling of notification control over NG has been presented in [3], and over Xn in [4].

This paper proposes the encoding for F1.

# 2	Proposal

It is proposed to agree the Text Proposal for TS 38.473 BL SA presented here-below which implements the notification control over F1 aligned with the principles described in [3] and [4].

# 3	References

- RP-172109, Revised Work Item on New Radio (NR) Access Technology, NTT DOCOMO

- 3GPP TS 23.501, Architecture for Next Generation System, SA2

- R3-181670, Handling of Notification Control in TS 38.413, Nokia

- R3-181671, Handling of Notification Control in TS 38.423, Nokia

# 4	Text Proposal for TS 38.473 SA BL CR

Beginning of changes

Table 2: Class 2 procedures

| Elementary Procedure | Message |
| --- | --- |
| Error Indication | ERROR INDICATION |
| UE Context Release Request (gNB-DU initiated) | UE CONTEXT RELEASE REQUEST |
| Initial UL RRC Message Transfer | INITIAL UL RRC MESSAGE TRANSFER |
| DL RRC Message Transfer | DL RRC MESSAGE TRANSFER |
| UL RRC Message Transfer | UL RRC MESSAGE TRANSFER |
| System Information Delivery | SYSTEM INFORMATION DELIVERY COMMAND |
| Paging | PAGING |
| DRB Resource Notify | DRB RESOURCE NOTIFY |

< unchanged text omitted>

Next change

### 8.2.X 	DRB Resource Notify

#### 8.2.X.1	General

The purpose of the PDU Session Resource Notify procedure is to allow the gNB-DU to send notification to the gNB-CU for the already established DRB(s) for a given UE. The procedure uses UE-associated signalling.

#### 8.2.X.2	Successful operation

Figure 8.2.X.2-1: DRB Resource Notify procedure.

Upon receiving the DRB Resource Notify message, the gNB-CU shall, if supported, take into account the DRB Notification Cause IE associated with each DRB included in the Bearers Notify List IE as described in [4].

#### 8.4.1.3	Abnormal Conditions

Not applicable.

Next change

#### 9.1.2.X	DRB RESOURCE NOTIFY

This message is sent by the gNB-DU to report that the QoS of a DRB involved in notification control can no longer be fulfilled for a specific UE or can be fulfilled again.

Direction: gNB-DU  gNB-CU.

Editor’s Note: The tabular below is FFS.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Bearers Notify List |  | 0..1 |  |  | - |  |
| >Bearers Notify Item IEs |  | 1 .. <maxnoofDRBs> |  |  | EACH | reject |
| >>DRB ID | M |  | <reference> |  | - |  |
| >>DRB Notification cause | M |  | ENUMERATED(fullfilled, not fulfilled, …) |  | - |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofDRBs | Maximum no. of DRBs. Value is FFS |