3GPP TSG-RAN WG3 #99bis						R3-181719

Sanya, China 16 - 20 April 2018

Agenda Item:	10.2.1.1

Source:	InterDigital Communications, ZTE

Title:	Registration with AMF Reallocation based on Slice Suitability

# Introduction

SA2 has discussed and agreed the concept of AMF relocation to select the appropriate slice or AMF to serve a UE. RAN3 so far has not addressed AMF reallocation except to blindly copy the NAS Redirect message when grabbing relevant procedures from 36.413. The discussion in SA2 was around two methods that involve RAN3, the first method (option A) transfers the NAS registration message from the initial AMF to the target AMF directly, the second method (option B) involves returning the NAS registration message to RAN with enough information so that the RAN can send to the correct target AMF. The figure below is the message sequence chart in TS 23.502 section 4.2.2.2.3:

# Current Status in other Groups

SA2:

SA2 agreed to include 2 methods during the study item and the work item. Method B (Reroute NAS) was used in LTE DÉCOR and method A was added for NR Among the advantages discussed for Method A is that the AMF can be changed with some knowledge whether registration can succeed in the Target AMF.

The open issues in SA2 are:

- The name of the N2 Message for Method A

- Editor's note:	The case when the target AMF cannot be determined and provided to the initial AMF, and the default AMF needs to be selected in step 3 is FFS.

This editor’s note is not tied to the use of Method A or Method B

- Editor's note:	Whether both option (A) and option (B) are acceptable from a RAN perspective needs to be evaluated by RAN WGs.

This was obviously put in because option A was new, and it has to be acceptable from a RAN perspective.

CT4:

CT4 has agreed in 29.518 to implement message 7A (used in method A) the Namf_Communication_N1MessageNotify message.

One usage of the message is for Registration with AMF Re-allocation Procedure:

TS 29.518 v050 In the Registration with AMF re-allocation procedure, the N1MessageNotify service operation is invoked by a NF Service Consumer, e.g. an Initial AMF, towards a target AMF, which is selected to serve the UE, by the initial AMF.

The open issues in CT4 are:

- Editor's Note 1:	It is assuming the operation semantics of N1MessageNotify is the request/response.

Obviously an internal CT4 issue

- Editor's Note 2:	Which alternative to be used is FFS. Alternatives are an HTTP Post or the service operation is invoked by a NF Service Producer

Obviously again an internal CT4 issue

The TS 29.518 is the control plane protocol between NR AMFs. In TS 29.274 which served as control plane protocol between MME for LTE, there is no procedure designed for forwarding NAS message to new MME. This indicates that in the NR phase, the 5G-C node control protocol has been able to support the re-allocation process (as method A).

Thus the only thing to discuss is SA2 issue 3, is option B acceptable from a RAN perspective? Since these two methods are transparent to CT and RAN2 (and obviously not an issue for RAN1 or RAN4 it is only up to RAN3 to decide, since it clearly was decided to be beneficial in SA2 and CT4.

# RAN3 Impacts for AMF Reallocation

Method B:

This method is basically reusing the Reroute NAS message from S1AP used for DECOR in LTE. The only thing to verify is whether the current message has all of the parameters so that the RAN node can reroute to the correct AMF based on slice selection. This is not addressed further in this paper

Method A:

There are 3 ways for the RAN to process the changing of target AMF when it is changed via AMF-AMF messaging.

A-1 (Downlink NAS Transport/ Initial UE Context Setup directly from Target AMF)	A Downlink NAS Transport message from the target AMF to the RAN is sent when an NAS message needs to be sent during a normal registration, for example an Identity Request, or a registration response.

The below message flow from the gNB’s perspective on what the flow would look like, there is of course AMF-AMF signalling between the INITIAL UE MESSAGE to the Initial AMF and the Downlink Message from the target AMF

INITIAL UE MESSAGE

INITIAL UE MESSAGE

UE

UE

gNB

gNB

Initial AMF

Initial AMF

Target AMF

Target AMF

RRC message (NAS PDU)

RRC message (NAS PDU)

DOWNLINK NAS TRANSPORT or INITIAL UE CONTEXT SETUP

DOWNLINK NAS TRANSPORT or INITIAL UE CONTEXT SETUP

Since the Initial UE message was sent to the initial AMF and the downlink NAS transport was received from the target AMF it is likely that this will be problematic. In this case only some text in Downlink NAS Transport would probably need to clarified that the Initial UE message can come from another AMF, the current text may only hint at that, therefore we aren’t considering this a solution to the problem.

Options A-2 and A-3 and A-4 depend on a new class 1 procedure to implement the redirection; it is class 1 because it is important to have a confirmation that procedure was executed.

A-2 (New AMF initiated Redirection message which establishes mapping of NGAP IDs) -New class 1 procedure that establishes the UE context like is done in downlink NAS transport/Initial Context Setup, but allow for a modification of the RAN UE NGAP ID.

A-3 (New AMF initiated Redirection message which doesn’t establish mapping of NGAP IDs) 	New class 1 procedure that does not establish a UE context it just notifies the NG RAN Node that the AMF has been changed and allows for a modification of the RAN UE NGAP ID.

A-4 (New AMF initiated Redirection message with AMF NGAP ID optional, if included the mapping is established) New class 1 procedure that establishes the UE context if the AMF UE NGAP ID is included similar to what is done in downlink NAS transport/Initial Context Setup, but allow for a modification of the RAN UE NGAP ID

One open issue is whether there needs to be an explicit indication in the AMF REDIRECTION REQUEST to identify the new and old AMF. If so AMF Name IE can be added to the message or an AMF ID IE can be added to the NG Setup Procedure and included in these messages.

Another open issue is whether there needs to be more assistance data (like NSSAI) added to the message for redirection request or response.

# pCR for Method A-2

### 8.6.2	Downlink NAS Transport

#### 8.6.2.1	General

Editor’s Note:	Further details are FFS.

#### 8.6.2.2	Successful Operation

Figure 8.6.2.2-1: Downlink NAS transport

If the AMF only needs to send a NAS message transparently via the NG-RAN node to the UE and a UE-associated logical NG-connection exists for the UE or if the AMF has received the RAN UE NGAP ID IE in an INITIAL UE MESSAGE message or in an AMF REDIRECTION RESPONSE message, the AMF shall send a DOWNLINK NAS TRANSPORT message to the NG-RAN node including the NAS message as a NAS-PDU IE. If the UE-associated logical NG-connection is not established, the AMF shall allocate a unique AMF UE NGAP ID to be used for the UE and include that in the DOWNLINK NAS TRANSPORT message; by receiving the AMF UE NGAP ID IE in the DOWNLINK NAS TRANSPORT message, the NG-RAN node establishes the UE-associated logical NG-connection.

### 8.6.x	AMF Redirection

#### 8.6.x.1	General

The purpose of the AMF Redirection procedure is to notify the NG-RAN node that the INITIAL UE MESSAGE message was forwarded to a new AMF to handle the UE context.

Editor’s Note:	Further details are FFS.

#### 8.6.x.2	Successful Operation

Figure 8.6.x.2-1: AMF Redirection: successful operation

The AMF initiates the procedure by sending an AMF REDIRECTION REQUEST message including the appropriate data to the NG-RAN node. The NG-RAN node responds with an AMF REDIRECTION RESPONSE message including the appropriate data.

If the New RAN UE NGAP ID IE is included in the AMF REDIRECTION RESPONSE message, the AMF shall, use it as the new RAN reference to this UE.

Upon successful completion of the procedure, the NG-RAN node establishes the UE-associated logical NG-connection.

Editor’s Note:	Further details are FFS.

#### 8.6.x.3	Unsuccessful Operation

Figure 8.7.1.3-1: AMF REDIECTION: unsuccessful operation

If the NG-RAN node cannot accept the redirection, it should respond with an AMF REDIRECTION FAILURE message and appropriate cause value.

Editor’s Note:	Further details are FFS.

#### 8.6.x.4	Abnormal Conditions

Editor’s Note:	Further details are FFS.

#### 9.2.5.x	AMF REDIRECTION REQUEST

Editor’s Note:	Message structure and IEs need further checking and completion. Further details FFS.

This message is sent by the AMF and is used for notifying the NG-RAN node that an AMF Redirection has occurred.

Direction: AMF  NG-RAN node

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| AMF UE NGAP ID | M |  | 9.3.3.1 |  | YES | reject |
| RAN UE NGAP ID | M |  | 9.3.3.2 |  | YES | reject |

#### 9.2.5.y	AMF REDIRECTION RESPONSE

Editor’s Note:	Message structure and IEs need further checking and completion. Further details FFS.

This message is sent by the NG-RAN node and is used to acknowledge an AMF Redirection.

Direction: NG-RAN node  AMF

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| AMF UE NGAP ID | M |  | 9.3.3.1 |  | YES | reject |
| RAN UE NGAP ID | M |  | 9.3.3.2 |  | YES | reject |
| New RAN UE NGAP ID | O |  | RAN UE NGAP ID 9.3.3.2 |  | YES | reject |

#### 9.2.5.z	AMF REDIRECTION FAILURE

Editor’s Note:	Further details FFS.

# pCR for Method A-3

### 8.6.x	AMF Redirection

#### 8.6.x.1	General

The purpose of the AMF Redirection procedure is to notify the NG-RAN node that the INITIAL UE MESSAGE message was forwarded to a new AMF to handle the UE context

Editor’s Note:	Further details are FFS.

#### 8.6.x.2	Successful Operation

Figure 8.6.x.2-1: AMF Redirection: successful operation

The AMF initiates the procedure by sending an AMF REDIRECTION REQUEST message including the appropriate data to the NG-RAN node. The NG-RAN node responds with an AMF REDIECTION RESPONSE message including the appropriate data.

If the New RAN UE NGAP ID IE is included in the AMF REDIECTION RESPONSE message, the AMF shall, use it as the new RAN reference to this UE.

Editor’s Note:	Further details are FFS.

#### 8.6.x.3	Unsuccessful Operation

Figure 8.7.1.3-1: AMF REDIECTION: unsuccessful operation

If the NG-RAN node cannot accept the redirection, it should respond with an AMF REDIRECTION FAILURE message and appropriate cause value.

Editor’s Note:	Further details are FFS.

#### 8.6.x.4	Abnormal Conditions

Editor’s Note:	Further details are FFS.

#### 9.2.5.x	AMF REDIRECTION REQUEST

Editor’s Note:	Message structure and IEs need further checking and completion. Further details FFS.

This message is sent by the AMF and is used for notifying the NG-RAN node that an AMF Redirection has occurred.

Direction: AMF  NG-RAN node

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| RAN UE NGAP ID | M |  | 9.3.3.2 |  | YES | reject |

#### 9.2.5.y	AMF REDIRECTION RESPONSE

Editor’s Note:	Message structure and IEs need further checking and completion. Further details FFS.

This message is sent by the NG-RAN node and is used to acknowledge an AMF Redirection.

Direction: NG-RAN node  AMF

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| RAN UE NGAP ID | M |  | 9.3.3.2 |  | YES | reject |
| New RAN UE NGAP ID | O |  | RAN UE NGAP ID 9.3.3.2 |  | YES | reject |

#### 9.2.5.z	AMF REDIRECTION FAILURE

Editor’s Note:	Further details FFS.

# pCR for Method A-4

### 8.6.2	Downlink NAS Transport

#### 8.6.2.1	General

Editor’s Note:	Further details are FFS.

#### 8.6.2.2	Successful Operation

Figure 8.6.2.2-1: Downlink NAS transport

If the AMF only needs to send a NAS message transparently via the NG-RAN node to the UE and a UE-associated logical NG-connection exists for the UE or if the AMF has received the RAN UE NGAP ID IE in an INITIAL UE MESSAGE message or in an AMF REDIRECTION RESPONSE message, the AMF shall send a DOWNLINK NAS TRANSPORT message to the NG-RAN node including the NAS message as a NAS-PDU IE. If the UE-associated logical NG-connection is not established, the AMF shall allocate a unique AMF UE NGAP ID to be used for the UE and include that in the DOWNLINK NAS TRANSPORT message; by receiving the AMF UE NGAP ID IE in the DOWNLINK NAS TRANSPORT message, the NG-RAN node establishes the UE-associated logical NG-connection.

### 8.6.x	AMF Redirection

#### 8.6.x.1	General

The purpose of the AMF Redirection procedure is to notify the NG-RAN node that the INITIAL UE MESSAGE message was forwarded to a new AMF to handle the UE context

Editor’s Note:	Further details are FFS.

#### 8.6.x.2	Successful Operation

Figure 8.6.x.2-1: AMF Redirection: successful operation

The AMF initiates the procedure by sending an AMF REDIRECTION REQUEST message including the appropriate data to the NG-RAN node. The NG-RAN node responds with an AMF REDIRECTION RESPONSE message including the appropriate data.

If the AMF UE NGAP ID IE is included in the AMF REDIECTION RESQUEST message, the NG-RAN node shall use it to identify the UE and include the AMF UE NGAP ID IE is included in the AMF REDIRECTION RESPONSE message.

If the New RAN UE NGAP ID IE is included in the AMF REDIECTION RESPONSE message, the AMF shall, use it as the new RAN reference to this UE.

If the AMF UE NGAP ID IE is included in the AMF REDIECTION RESQUEST message, upon successful completion of the procedure the NG-RAN node establishes the UE-associated logical NG-connection.

Editor’s Note:	Further details are FFS.

#### 8.6.x.3	Unsuccessful Operation

Figure 8.7.1.3-1: AMF REDIECTION: unsuccessful operation

If the NG-RAN node cannot accept the redirection, it should respond with an AMF REDIRECTION FAILURE message and appropriate cause value.

Editor’s Note:	Further details are FFS.

#### 8.6.x.4	Abnormal Conditions

Editor’s Note:	Further details are FFS.

#### 9.2.5.x	AMF REDIRECTION REQUEST

Editor’s Note:	Message structure and IEs need further checking and completion. Further details FFS.

This message is sent by the AMF and is used for notifying the NG-RAN node that an AMF Redirection has occurred.

Direction: AMF  NG-RAN node

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| AMF UE NGAP ID | O |  | 9.3.3.1 |  | YES | reject |
| RAN UE NGAP ID | M |  | 9.3.3.2 |  | YES | reject |

#### 9.2.5.y	AMF REDIRECTION RESPONSE

Editor’s Note:	Message structure and IEs need further checking and completion. Further details FFS.

This message is sent by the NG-RAN node and is used to acknowledge an AMF Redirection.

Direction: NG-RAN node  AMF

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| AMF UE NGAP ID | O |  | 9.3.3.1 |  | YES | reject |
| RAN UE NGAP ID | M |  | 9.3.3.2 |  | YES | reject |
| New RAN UE NGAP ID | O |  | RAN UE NGAP ID 9.3.3.2 |  | YES | reject |

#### 9.2.5.z	AMF REDIRECTION FAILURE

Editor’s Note:	Further details FFS.

# Conclusion

There have been stage 2/3 agreements in SA2 and CT4 on the need for a AMF-AMF transfer method, and it has been shown that at least one method to allow for AMF Redirection through direct AMF-AMF communications is doable in RAN3.

Proposal: Agree to implement AMF Redirection using one of the above methods:

A-1 (Downlink NAS Transport/ Initial UE Context Setup directly from Target AMF)

A-2 (New AMF initiated Redirection message which establishes mapping of NGAP IDs)

A-3 (New AMF initiated Redirection message which doesn’t establish mapping of NGAP IDs)

A-4 (New AMF initiated Redirection message with AMF NGAP ID optional, if included the mapping is established)

Also to be decided is whether an AMF identity needs to be included and if other assistance data (for example NSSAI) needs to be included in the messages.

Agree one as a starting point the corresponding pCR.

While any of the options A-2, A-3 or A-4 would work we believe that option A-4 is the one with the most flexibility so it is our preference. Of course if a new NGAP procedure that is used for another purpose could be used to accomplish the same basics we also fine with that.