3GPP TSG-RAN WG3 #99bis	R3-182086

Sanya, China, 16-20 April 2018

Agenda item:	10.10.7.3.

Source:	Google

Title:	TP for 38.401 to support delta configuration

Document for:	Discussion & Decision

- Introduction

This paper provides the TP for 38.401 if the decision is made after discussing [1].

References

[1] R3-182085, Supporting EN-DC delta configuration in CU-DU architecture

[2] TS 38.401 v15.1.0

# 2 Text Proposal to TS 38.401

8.4	Multi-Connectivity operation

8.4.1	Secondary Node Addition

8.4.1.1	EN-DC

This clause gives the SgNB addition procedure in EN-DC given that en-gNB consists of gNB-CU and gNB-DU(s), as shown in Figure 8.4.1.1-1.

Figure 8.4.1.1-1: SgNB addition procedure in EN-DC

1~8: refer to TS 37.340 [12]

a1. After receiving the SgNB Addition Request message from MeNB, the gNB-CU sends the UE Context Setup Request message to the gNB-DU to create a UE context

a2. The gNB-DU responds to the gNB-CU with the UE Context Setup Response message

NOTE:	The Step a1 UE Context Setup Request message may contain source cell group configuration to support delta configuration at the gNB-DU. If the gNB-DU decides to perform full configuration after receiving the source cell group configuration, it shall indicate that it has applied full configuration in the UE Context Setup Response message in the Step a2.