3GPP TSG-RAN WG3 Meeting #99bis	R3-182167

# Sanya, P.R. China, 16th- 20th April  2018

Agenda Item:	10.5.6

Source:	Ericsson

Title:	Data forwarding handling 4G to 5G

Document for:	pCR

# Information

This pCR introduce in the data forwarding 4G to 5G, that the target NG-RAN node when it accepts the proposed E-RAB for data forwarding would provide tunnel end pointer per E-RAB. The UPF provides intermediate tunneling end points for the data forwarding paths on a per E-RAB basis. There is no need for UPF to do the E-RAB to QoS flow/PDU session mapping for the forwarding tunnels.

NGAP already support Data Forwarding Response per DRB ID in the HANDOVER REQUEST ACKNOLEDGE.

The impact is to add the option to do Data Forwarding Response per E-RAB ID.

To reuse the current structure and clean up the specification, further updates to this procedure including:

- Remove the DRB data forwarding related information from 9.3.4.2, PDU Session Resource Setup Response Transfer;

- Create a new IE for Handover Request Acknowledge Transfer;

- Introduce a CHOICE to the current DRB Based data forwarding, so it is possible to do data forwarding either on DRB ID, or on E-RAB ID.

- Corrections:

  - Change the Range of  “Data Forwarding Response DRB Item IEs”  from 1 to 0, to support the possibility when there is no need for data forwarding per Data bearer.

  - Add the procedure text on the Intra system data forwarding.

# Text Proposal for NGAP TS 38.413 v0.7.0

<<<<<<<<<<<<<<<<<<<< Begin Text Proposal >>>>>>>>>>>>>>>>>>>>

### 8.4.2	Handover Resource Allocation

#### 8.4.2.1	General

The purpose of the Handover Resource Allocation procedure is to reserve resources at the target NG-RAN node for the handover of a UE.

Editor’s Note:	Further details are FFS.

#### 8.4.2.2	Successful Operation

Figure 8.4.2.2-1: Handover resource allocation: successful operation

The AMF initiates the procedure by sending the HANDOVER REQUEST message to the target NG-RAN node.

If the Masked IMEISV IE is contained in the HANDOVER REQUEST message the target NG-RAN node shall, if supported, use it to determine the characteristics of the UE for subsequent handling.

Upon receipt of the HANDOVER REQUEST message the target NG-RAN node shall

-	attempt to execute the requested PDU session configuration;

-	store the UE Aggregate Maximum Bit Rate in the UE context, and use the received UE Aggregate Maximum Bit Rate for non-GBR Bearers for the concerned UE; [FFS]

-	store the received Handover Restriction List in the UE context;

-	store the received UE Security Capabilities in the UE context;

-	store the received Security Key in the UE context, take it into use and associate it with the initial value of NCC as defined in TS xx.xxx [xx]; [FFS pending SA3]

For the Initial Context Setup an initial value for the Next Hop Chaining Count is stored in the UE context. [FFS pending SA3]

Upon receiving the PDU Session Resource To Be Setup List IE contained in the HANDOVER REQUEST message, the target NG-RAN node shall behave the same as defined in the PDU Session Resource Setup procedure.

The target NG-RAN node shall then report in the HANDOVER REQUEST ACKNOWLEDGE message the successful establishment of the result for all the requested PDU sessions. In particular, for each PDU session resource successfully setup, it shall include the PDU Session Resource Setup Response Transfer IE containing the following information:

-	The list of QoS flows which have been successfully established in the QoS Flows Setup List IE.

-	The list of QoS flows which have failed to be established, if any, in the QoS Flows Failed to Setup List IE.

-	The transport layer address and tunnel endpoint to be used for the PDU session.

The list of PDU session resources which failed to be setup, if any, shall be reported in the HANDOVER REQUEST ACKNOWLEDGE message with an appropriate cause value within the PDU Session Resource Setup Failed List IE.

Upon reception of the HANDOVER REQUEST ACKNOWLEDGE message the AMF shall, for each PDU session indicated in the PDU Session ID IE, transfer transparently the PDU Session Resource Setup Response Transfer IE to each SMF associated with the concerned PDU session.

When the target NG-RAN node reports unsuccessful establishment of a QoS flow, the cause value should be precise enough to know the reason for an unsuccessful establishment [FFS].

In case of intra-system handover, if the target NG-RAN node supports data forwarding on DRB basis, it may include for each DRB in the PDU session the UL and DL transport layer information for data forwarding in the HANDOVER REQUEST ACKNOWLEDGE message as specified in TS 38.300 [8].

In case of intra-system handover, if the target NG-RAN node supports DL data forwarding on QoS flow basis for fresh DL user data packets, it may include DL transport layer information for data forwarding in the HANDOVER REQUEST ACKNOWLEDGE as specified in TS 38.300 [8].

In case of inter-system handover from E-UTRAN, if the target NG-RAN node supports data forwarding on E-RAB basis, it may include for each DRB in the PDU session the UL and DL transport layer information for data forwarding in the HANDOVER REQUEST ACKNOWLEDGE message as specified in TS 38.300 [8].

In case of inter-system handover from E-UTRAN, the target NG-RAN node includes the Data Forwarding Accepted IE for each QoS flow that the DL forwarding IE is set to “DL forwarding proposed” for the corresponding E-RAB in the Source NG-RAN Node to Target NG-RAN Node Transparent Container and that the target NG-RAN node has admitted the proposed forwarding of downlink data for the QoS flow. If the target NG-RAN node accepts the downlink data forwarding for at least one QoS flow of an admitted PDU session it shall include the DL Forwarding Transport Layer Information IE in the PDU Session Resource Setup Response Transfer IE for that PDU session within the PDU Sessions Admitted List IE of the HANDOVER REQUEST ACKNOWLEDGE message.

The target NG-RAN node shall use the information in the Handover Restriction List IE if present in the HANDOVER REQUEST message to

-	determine a target for subsequent mobility action for which the target NG-RAN node provides information about the target of the mobility action towards the UE;

-	select a proper SCG during dual connectivity operation.

If the Handover Restriction List IE is not contained in the HANDOVER REQUEST message, the target NG-RAN node shall consider that no roaming and no access restriction apply to the UE. The target NG-RAN node shall also consider that no roaming and no access restriction apply to the UE when:

-	one of the setup PDU sessions includes a particular ARP value (TS 23.501 [9]). [FFS, need to align with PDU Session Management]

If the Trace Activation IE is included in the HANDOVER REQUEST message the target NG-RAN node shall, if supported, initiate the requested trace function as described in TS 32.422 [11]. In particular, the target NG-RAN node shall, if supported: [FFS pending RAN2 and SA5]

-	if the Trace Activation IE does not include the MDT Configuration IE, initiate the requested trace session as described in TS 32.422 [11];

-	if the Trace Activation IE includes the MDT Activation IE, within the MDT Configuration IE, set to “Immediate MDT and Trace”, initiate the requested trace session and MDT session as described in TS 32.422 [11];

-	if the Trace Activation IE includes the MDT Activation IE, within the MDT Configuration IE, set to “Immediate MDT Only”, or “Logged MDT only”, initiate the requested MDT session as described in TS 32.422 [11] and the target NG-RAN node shall ignore the Interfaces To Trace IE and Trace Depth IE;

-	if the Trace Activation IE includes the MDT Location Information IE, within the MDT Configuration IE, store this information and take it into account in the requested MDT session;

-	if the Trace Activation IE includes the Signalling based MDT PLMN List IE, within the MDT Configuration IE, the target NG-RAN node may use it to propagate the MDT Configuration as described in TS 37.320 [12].

If the UE Security Capabilities IE is included in the HANDOVER REQUEST message, the target NG-RAN node shall handle it accordingly. [FFS pending SA3]

If the Management Based MDT Allowed IE is contained in the HANDOVER REQUEST message, the target NG-RAN node shall use it, if supported, together with information in the Management Based MDT PLMN List IE, if available in the UE context, to allow subsequent selection of the UE for management based MDT defined in TS 32.422 [11]. [FFS pending RAN2 and SA5].

If the RRC Inactive Assistance Information IE is included in the HANDOVER REQUEST message, the target NG-RAN node shall, if supported, store this information in the UE context and use it for the RRC-INACTIVE state decision and configuration for the UE and RAN paging if any for a UE in RRC-INACTIVE state, as specified in TS 38.300 [8].

If the New Security Context Indicator IE is included in the HANDOVER REQUEST message, the target NG-RAN node shall, if supported, use this NSCI information as specified in TS 33.501 [13].

After all necessary resources for the admitted PDU Session Resources have been allocated, the target NG-RAN node shall generate the HANDOVER REQUEST ACKNOWLEDGE message.

------------------------------------------

Skip to next change

------------------------------------------

#### 9.2.3.5	HANDOVER REQUEST ACKNOWLEDGE

Editor’s Note:	Message structure and IEs need further checking and completion. Further details FFS.

This message is sent by the target NG-RAN node to inform the AMF about the prepared resources at the target.

Direction: NG-RAN node  AMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| AMF UE NGAP ID | M |  | 9.3.3.1 |  | YES | ignore |
| RAN UE NGAP ID | M |  | 9.3.3.2 | Allocated at the target NG-RAN node. | YES | ignore |
| PDU Sessions Admitted List |  | 1 |  |  | YES | ignore |
| >PDU Sessions Admitted Item IEs |  | 1..<maxnoofPDUSessions> |  |  | EACH | ignore |
| >>PDU Session ID | M |  | <ref> |  | - |  |
| >>Handover Request Acknowledge Transfer | M |  | 9.3.4.x |  | YES | ignore |
| PDU Session Resource Failed to Setup List | O |  | PDU Session List 9.3.1.11 |  | YES | ignore |
| Target to Source Transparent Container | M |  | 9.3.1.21 |  | YES | reject |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | reject |

------------------------------------------

Skip to next change

------------------------------------------

#### 9.3.4.2	PDU Session Resource Setup Response Transfer

This IE is transparent to the AMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Transport Layer Information | M |  | 9.3.2.2 | NG-RAN Transport Layer Address for the PDU session NG-U GTP-U tunnel transporting QoS flows listed in the QoS Flows Setup List IE. | - |  |
| DL Forwarding Transport Layer Information | O |  | Transport Layer Information 9.3.2.2 | To deliver forwarded DL SDAP SDUs. | - |  |
| QoS Flows Setup List |  | 1 |  |  | - |  |
| >QoS Flows Setup Item IEs |  | 1..<maxnoofQoSFlows> |  |  | - |  |
| >>QoS Flow Indicator | M |  | <ref> |  | EACH | reject |
| >>Data Forwarding Accepted | O |  | <ref> |  |  |  |
| QoS Flows Failed to Setup List | O |  | QoS Flow List 9.3.1.13 |  | - |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofQoSFlows | Maximum no. of QoS flows allowed within one PDU session. Value is 64. |

------------------------------------------

Skip to next change

------------------------------------------

#### 9.3.4.x	Handover Request Acknowledge Transfer

This IE is transparent to the AMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| PDU Session Resource Information | M |  | PDU Session Resource Setup Response Transfer 9.3.4.2 |  | - |  |
| Data Forwarding Response List |  | FFS: to be confirmed by SA2 |  |  | - |  |
| >Data Forwarding Response Item IEs |  | 0..<maxnoofDRBsE-RABs> |  |  | - |  |
| >>CHOICE Data Forwarding Basis | M |  |  |  | - |  |
| >>>Intra System HO |  |  |  |  |  |  |
| >>>>DRB ID | M |  | <ref> |  | - |  |
| >>>Inter System HO |  |  |  |  |  |  |
| >>>>E-RAB ID | M |  | 9.3.2.3 |  | - |  |
| >>DL Forwarding GTP Tunnel Endpoint | O |  | GTP-TEID 9.3.2.5 |  | - |  |
| >>UL Forwarding GTP Tunnel Endpoint | O |  | GTP-TEID 9.3.2.5 |  | - |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofDRBsE-RABs | Editor’s Note: Value is 32. |

<<<<<<<<<<<<<<<<<<<< End Text Proposal  >>>>>>>>>>>>>>>>>>>>