3GPP TSG RAN WG3 Meeting #99bis	Tdoc R3-182203

Sanya, P. R. China, 16th – 20th April 2018

Agenda Item:	10.10.5

Source:	Ericsson

Title:	UE identities for RAN and CN paging

Document for:	TP for NR BL CR for TS 38.473

# Introduction

At the previous RAN3 meeting, the issue of UE paging identities for RAN-based and CN-based paging was discussed. In this contribution we further look into the matter.

# Discussion

At the previous RAN3 meeting, the issue of UE paging identities for RAN-based and CN-based paging was discussed. In the SA BL CR for 38.473, the IEs RAN UE paging identity and CN UE paging identity were included in the paging message with FFS for the actual content of the IE.

In the last RAN2 meeting the following were decided:

- For RAN paging, I-RNTI is used as the UE identity in the paging record.

- 5G S-TMSI will be used for CN paging

Based on the above we propose the following

Proposal 1	In the F1-AP paging message the I-RNTI is used as the UE paging identity for RAN-based paging.

Proposal 2	In the F1-AP paging message the 5G S-TMSI is used as the UE paging identity for CN-based paging.

# Conclusion

In this contribution, we discussed the issue of UE paging identities for RAN-based and CN-based paging and made the following proposals

Proposal 1	In the F1-AP paging message the I-RNTI is used as the UE paging identity for RAN-based paging.

Proposal 2	In the F1-AP paging message the 5G S-TMSI is used as the UE paging identity for CN-based paging.

# Annex I

Start of Text Proposal1 for TS 38.473

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 38.470: "NG-RAN; F1 general aspects and principles".

[3]	3GPP TS 38.413: "NG-RAN; NG Application Protocol (NGAP)".

[4]	3GPP TS 38.401: "NG-RAN; Architecture Description".

[5]	ITU-T Recommendation X.691 (2002-07): "Information technology - ASN.1 encoding rules - Specification of Packed Encoding Rules (PER)".

[6]	3GPP TS 38.300: "NR; Overall description; Stage-2".

[7]	3GPP TS 37.340: "NR; Multi-connectivity; Overall description; Stage-2".

[8]	3GPP TS 38.331: "NR; Radio Resource Control (RRC); Protocol specification".

[9]	3GPP TS 36.423: "Evolved Universal Terrestrial Radio Access Network (E-UTRAN); X2 Application Protocol (X2AP)".

[10]	3GPP TS 23.401: "General Packet Radio Service (GPRS) enhancements for Evolved Universal Terrestrial Radio Access Network (E-UTRAN) access".

[11]	3GPP TS 23.203: "Policy and charging control architecture".

[12]	ITU-T Recommendation X.680 (07/2002): "Information technology – Abstract Syntax Notation One (ASN.1): Specification of basic notation".

[13]	ITU-T Recommendation X.681 (07/2002): "Information technology – Abstract Syntax Notation One (ASN.1): Information object specification".

[14]	3GPP TR 25.921 (version.7.0.0): "Guidelines and principles for protocol description and error".

[15]	3GPP TS 36.413: " Evolved Universal Terrestrial Radio Access Network (E-UTRAN); S1 Application Protocol (S1AP)".

[16]	3GPP TS 38.321: "NR; Medium Access Control (MAC) protocol specification".

[17]	3GPP TS 38.104: “NR; Base Station (BS) radio transmission and reception”.

[x1]	3GPP TS 23.501: "System Architecture for the 5G System".

[x2]	3GPP TS 38.472: "NG-RAN; F1 signalling transport".

[x3]	3GPP TS 23.003: “Numbering, addressing and identification”.

Start of Text Proposal2 for TS 38.473

### 9.2.x3	Paging messages

#### 9.2.x3.1	PAGING

This message is sent by the gNB-CU and is used to request the gNB-DU to page UEs.

Direction: gNB-CU  gNB-DU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| UE Identity Index value | M |  | 9.3.1.x3 |  | YES | reject |
| RAN UE Paging identity | O |  |  | I-RNTI to be defined in TS 38.331 | YES | reject |
| CN UE paging identity | O |  |  | 5G-S-TMSI as defined in TS 23.003 [x3] | YES | reject |
| Paging DRX | O |  | 9.3.1.x4 |  | YES | ignore |
| Paging Priority | O |  | 9.3.1.x5 |  | YES | ignore |
| Paging Cell List |  | 1 |  |  |  |  |
| >Paging Cell Item IEs |  | 1 .. <maxnoofPagingCells> |  |  | EACH | ignore |
| >>NR CGI | M |  | 9.3.1.12 |  | - |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofPagingCells | Maximum no. of paging cells, the maximum value is FFS. |

Editor’s note: The detailed definition for the IEs above and the inclusion of additional IEs is FFS.

End of Text Proposal for TS 38.473