3GPP TSG-RAN WG3 Meeting #99bis	R3-182217

# Sanya, P.R China, 16th- 20th April  2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.300 | CR |  | rev |  | Current version: | 15.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of the vendor specific UE differentiation Information |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, Vodafone |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NB_IOTenh2-Core |  |  |  |  |  |  | Date: |  | 2018-04-06 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The vendor specific UE differentiation information has been agreed to be useful for differentiate NB-IoT users. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce the “UE Differentiation Information” in stage 2. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The UE Differentiation Information is not supported in Rel 15 for NB-IoT. |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  | CR 1586, CR 1131 |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

# xx	UE Differentiation Information

UE Differentiation Information is generated by the E-UTRAN and contains vendor specific information of one or serveral different vendors that may be used and maintained by different E-UTRAN nodes of the same vendor over serveral IDLE-CONNECTED cycles. Each vendor specific information is tagged by a vendor identification, allocation of vendor identification values is managed by O&M.

UE Differentiation Information is provided to the MME at release of the UE associated S1 signalling connection e.g. within the UE CONTEXT RELEASE COMPLETE message and stored in the MME. When the MME receives UE Differentiation Information for the same vendor in a subsequent S1 signalling it overwrites the previously received one.

UE Differentiation Information is stored transparently in the MME.

UE Differentiation Information is provided to the E-UTRAN at establishment of the UE associated S1 signalling connection e.g. within the INITIAL CONTEXT SETUP message.

An eNB only understands and maintains UE Differentiation Information provided by another eNB from the same vendor and ignores information provided by other vendors. The eNB receiving UE Differentiation Information containing vendor specific information provided by other vendors keeps such information for later usage in other eNBs.