3GPP TSG-RAN WG3 #99bis	Tdoc R3-182242

Sanya, P. R. China, 16th- 20th April  2018

Agenda Item:	23.3

Source:	Ericsson

Title:	E1 Configuration Update

Document for:	pCR TS 38.463

# Introduction

This is a resubmission of R3-181308 with the addition of two new proposals (2 and 4).

As illustrated in TR 38.806 [1], during the study item it the following E1 procedures were discussed. At the time, there were no conclusion on whether these functions are needed:

- E1 Load Management: this procedure allows CU-UP and CU-CP to exchange load information. This function may also include information related to flow control;

- E1 Configuration Update; this procedure supports updates in CU-UP configuration;

- Inactivity Detection: This procedure allows CU-UP to indicate that the inactivity timer associated with a UE expires or that the user data is received for a UE whose inactivity timer has expired;

- Over NG-U interface associated with a new QoS Flow ID (QFI) for which the QoS parameters are already available in the CU-UP, and there is no association to an existent DRB. The discussion on this function is also dependent on whether this use-case will be supported.

In this contribution, we discuss the Configuration Update procedure.

# Discussion

During the study item, the E1 Configuration Update procedure was discussed. However, there was no conclusion on whether it is needed and, if needed, whether it should be in both directions.

In R3-182235, we discuss the need for supporting multiple TNL associations over the E1. We propose to support a similar mechanism as over F1 and NG. Therefore, to enable the CU-CP to add/remove the additional TNL associations, a “CU-CP Configuration Update” procedure is needed. The “CU-CP Configuration Update” procedure can also be used to update other relevant application data parameters.

Proposal 1: 	Introduce a Class-1 CU-CP Configuration Update procedure. The procedure includes: CU-CP Configuration Update, CU-CP Configuration Update Acknowledge and CU-CP Configuration Update Failure messages.

To allow the CU-CP to establish additional TNL associations, we propose also to include in the CU-CP Configuration Update message the additional TNL addresses that can be used at the CU-CP side. The details of the mechanism for supporting multiple TNL associations over the E1 can be aligned with F1 and NG.

Proposal 2: 	Include the Additional TNL Address List IE in the CU-CP Configuration Update message, to enable establishment of multiple TNL associations.

In R3-182237, we propose some initial content for the E1 Setup messages. It is expected that in the E1 Setup Response message the CU-UP will send several application parameters, such as supported slices. These parameters will guide the CU-CP in the selection of the proper CU-UP for bearer setup. Since these parameters may be updated by the operator (e.g., via O&M), the E1 should allow the CU-UP to report the updated parameters to the CU-CP. Therefore, a CU-UP Configuration Update procedure is needed.

Proposal 3: 	Introduce a Class-1 CU-UP Configuration Update procedure. The procedure includes: CU-UP Configuration Update, CU-UP Configuration Update Acknowledge and CU-UP Configuration Update Failure messages.

Proposal 4: 	Include in the CU-UP Configuration Update message the PLMN and slicing (supported S-NSSAI per PLMN) information.

In Annex I, we propose the initial structure for the Configuration Update messages. For the content of the messages, we suggest waiting for the discussion in other contributions, such as R3-181299 and R3-181304.

Proposal 3: 	RAN3 is kindly asked to agree with the TP in Annex I.

# Conclusion

In this contribution, we discussed the Configuration Update procedures.

Proposal 1: 	Introduce a Class-1 CU-CP Configuration Update procedure. The procedure includes: CU-CP Configuration Update, CU-CP Configuration Update Acknowledge and CU-CP Configuration Update Failure messages.

Proposal 2: 	Include the Additional TNL Address List IE in the CU-CP Configuration Update message, to enable establishment of multiple TNL associations.

Proposal 3: 	Introduce a Class-1 CU-UP Configuration Update procedure. The procedure includes: CU-UP Configuration Update, CU-UP Configuration Update Acknowledge and CU-UP Configuration Update Failure messages.

Proposal 4: 	Include in the CU-UP Configuration Update message the PLMN and slicing (supported S-NSSAI per PLMN) information.

Proposal 3: 	RAN3 is kindly asked to agree with the TP in Annex I.

# References

TR 38.806, Study of separation of NR Control Plane (CP) and User Plane (UP) for split option 2.

# Annex I: TP for 38.463

Start of Text Proposal 1 for TS 38.463

Table 1: Class 1 procedures

| Elementary Procedure | Initiating Message | Successful Outcome | Unsuccessful Outcome |
| --- | --- | --- | --- |
|  |  | Response message | Response message |
| CU-CP Configuration Update | CU-CP CONFIGURATION UPDATE | CU-CP CONFIGURATION UPDATE ACKNOWLEDGE | CU-CP CONFIGURATION UPDATE FAILURE |
| CU-UP Configuration Update | CU-UP CONFIGURATION UPDATE | CU-UP CONFIGURATION UPDATE ACKNOWLEDGE | CU-UP CONFIGURATION UPDATE FAILURE |

Start of Text Proposal 2 for TS 38.463

### 8.2.X	CU-CP Configuration Update

#### 8.2.X.1	General

The purpose of the CU-CP Configuration Update procedure is to update application level configuration data needed for the CU-CP and the CU-UP to interoperate correctly on the E1 interface. This procedure does not affect existing UE-related contexts, if any.

##### 8.2.X.2	Successful Operation

Figure 8.2.X.2-1: CU-CP Configuration Update procedure. Successful operation.

The CU-CP initiates the procedure by sending a CU-CP CONFIGURATION UPDATE message to the CU-UP including an appropriate set of updated configuration data that it has just taken into operational use. The CU-UP responds with the CU-CP CONFIGURATION UPDATE ACKNOWLEDGE message to acknowledge that it successfully updated the configuration data.

The updated configuration data shall be stored in both nodes and used as long as there is an available TNL association or until any further update is performed.

##### 8.2.X.3	Unsuccessful Operation

Figure 8.2.X.3-1: CU-CP Configuration Update procedure. Unsuccessful operation.

If the CU-UP cannot accept the update, it shall respond with a CU-CP CONFIGURATION UPDATE FAILURE message and appropriate cause value.

If the CU-UP CONFIGURATION UPDATE FAILURE message includes the Time to Wait IE, the CU-CP shall wait at least for the indicated time before reinitiating the CU-CP CONFIGURATION UPDATE message towards the same CU-UP.

#### 8.2.X.3	Abnormal Conditions

Not applicable.

### 8.2.Y	CU-UP Configuration Update

#### 8.2.Y.1	General

The purpose of the CU-UP Configuration Update procedure is to update application level configuration data needed for the CU-CP and the CU-UP to interoperate correctly on the E1 interface. This procedure does not affect existing UE-related contexts, if any.

##### 8.2.Y.2	Successful Operation

Figure 8.2.Y.2-1: CU-UP Configuration Update procedure. Successful operation.

The CU-UP initiates the procedure by sending a CU-UP CONFIGURATION UPDATE message to the CU-CP including an appropriate set of updated configuration data that it has just taken into operational use. The CU-CP responds with the CU-UP CONFIGURATION UPDATE ACKNOWLEDGE message to acknowledge that it successfully updated the configuration data.

The updated configuration data shall be stored in both nodes and used as long as there is an available TNL association or until any further update is performed.

##### 8.2.Y.3	Unsuccessful Operation

Figure 8.2.Y.3-1: CU-UP Configuration Update procedure. Unsuccessful operation.

If the CU-CP cannot accept the update, it shall respond with a CU-UP CONFIGURATION UPDATE FAILURE message and appropriate cause value.

If the CU-CP CONFIGURATION UPDATE FAILURE message includes the Time to Wait IE, the CU-UP shall wait at least for the indicated time before reinitiating the CU-UP CONFIGURATION UPDATE message towards the same CU-UP.

#### 8.2.Y.3	Abnormal Conditions

Not applicable.

Start of Text Proposal 3 for TS 38.463

#### 9.2.1.A	CU-CP CONFIGURATION UPDATE

This message is sent by the CU-CP to transfer updated information for a TNL association.

Direction: CU-CP  CU-UP

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.a |  | YES | reject |
| CU-CP Additional TNL Address List | O |  | 9.3.1.e | Additional CU-CP TNL addresses that can be used for multiple TNL associations. | YES | ignore |

Editor’s note: The presence and definition of all the IEs in this message need further checking.

#### 9.2.1.B	CU-CP CONFIGURATION UPDATE ACKNOWLEDGE

This message is sent by the CU-UP to acknowledge update of information for a TNL association.

Direction: CU-UP  CU-CP

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.a |  | YES | reject |
| Criticality Diagnostics | O |  | 9.3.1.v |  | YES | ignore |

Editor’s note: The presence and definition of all the IEs in this message need further checking.

#### 9.2.1.C	CU-CP CONFIGURATION UPDATE FAILURE

This message is sent by the CU-UP to report a CU-CP Configuration Update failure.

Direction: CU-UP  CU-CP

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.a |  | YES | reject |
| Cause | M |  | 9.3.1.c |  | YES | ignore |
| Time to wait | O |  | 9.3.1.d |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.3.1.b |  | YES | ignore |

#### 9.2.1.D	CU-UP CONFIGURATION UPDATE

This message is sent by the CU-UP to transfer updated information for a TNL association.

Direction: CU-UP  CU-CP

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.a |  | YES | reject |
| Supported PLMN List |  | 0.. 1 |  | PLMNs supported by the CU-UP. | YES | reject |
| >Supported PLMN Item IEs |  | 1..<maxnoofSPLMNs> |  |  | EACH | reject |
| >>PLMN Identity | M |  | 9.3.1.Y |  | - | - |
| >>Slice Support List | M |  | 9.3.1.Z |  | - | - |

Editor’s note: The presence and definition of all the IEs in this message need further checking.

#### 9.2.1.E	CU-UP CONFIGURATION UPDATE ACKNOWLEDGE

This message is sent by the CU-CP to acknowledge update of information for a TNL association.

Direction: CU-CP  CU-UP

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.a |  | YES | reject |
| Criticality Diagnostics | O |  | 9.3.1.v |  | YES | ignore |

#### 9.2.1.F	CU-UP CONFIGURATION UPDATE FAILURE

This message is sent by the CU-CP to report a CU-UP Configuration Update failure.

Direction: CU-CP  CU-UP

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.a |  | YES | reject |
| Transaction ID | M |  | 9.3.1.c |  | YES | reject |
| Cause | M |  | 9.3.1.d |  | YES | ignore |
| Time to wait | O |  | 9.3.1.b |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.3.1.a |  | YES | ignore |

Start of Text Proposal 2 for TS 38.463

#### 9.3.1.e	TNL List

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| TNL List |  | 0.. 1 |  |  | YES | ignore |
| >TNL Item |  | 1..<maxnoofTNLAddresses> |  |  | EACH | ignore |
| >>Transport Layer Address | M |  | 9.3.1.e1 |  | - | - |

| Range bound | Explanation |
| --- | --- |
| maxnoofTNLAddresses | Maximum no. of TNL End Points. Value is FFS. |

End of Text Proposal for TS 38.463