3GPP TSG RAN WG3 #99bis	Tdoc R3-182261

# Sanya, P. R. China, 16th- 20th April  2018

Agenda Item:	31.3.1

Source:	Ericsson

Title:	Correction on SRB ID range

Document for:	Discussion, Decision

# Introduction

At the last RAN3 meeting, the range of the SRB ID in F1AP was changed to (1.. 3, …) [1]. However, this creates a problem for stand-alone operation, as it is not possible to indicate that an RRC messages should be transmitted over SRB0. In this contribution, we analyse possible solutions to this problem.

# Discussion

In the F1AP specifications [1], the SRB ID is defined as follows:

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| SRB ID | M |  | INTEGER (1..3, ...) | Corresponds to the SRB-Identity defined in TS 38.331 [8]. |

Therefore, it seems that it is not possible to indicate that an RRC message is to be sent on SRB0. In EN-DC this is not a problem because SRB0 is not utilized (only SRB3 is needed). However, in SA the DL RRC Message Transfer is used to carry RRC messages for SRB0, such as the RRC-Connection-Setup message.

To solve this problem, one option could have been to omit the SRB ID as an indication that the RRC message is to be sent on SRB0. However, in the DL RRC Message Transfer the SRB ID is mandatory. Therefore, it seems that this is not possible.

Some other options to solve the problem are:

- Non-backward compatible change of the SRB ID range to (0..3, …);

- Backward compatible change, such as adding a new optional IE that indicates that the gNB-DU should ignore the SRB ID and send the RRC message on SRB0.

To keep the F1AP specifications clean, we believe that the first option is better.

Proposal 1:	Change the SRB ID range to (0..3, …).

# References

TS 38.473 V15.1.0, F1 Application Protocol.