3GPP TSG-RAN WG3 Meeting #99bis	R3-182266

# Sanya, P.R China, 16th- 20th April  2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.423 | CR | 1086 | rev | 1 | Current version: | 15.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of QMC for MTSI in EUTRAN |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15, EQoE_MTSI |  |  |  |  |  |  | Date: |  | 2018-04-06 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | A new QMC functionality for the MTSI services is introduced, and it would reuse the structure made in the introduction of QMC for streaming service. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce a service type; Correction to 9.2.1.121. MDT is incorrect. It is replaced by QMC. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The QMC for MSTI is not supported in EUTRAN in Rel 15. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.3, 9.2.1.121, 9.3.5, 9.3.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  | CR 1572, CR 1098, CR 1554, CR 3243 |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

ABS	Almost Blank Subframe

BBF	Broadband Forum

BL	Bandwidth reduced Low complexity

CCO	Cell Change Order

CE	Coverage Enhancement

CoMP	Coordinated Multi Point

DC	Dual Connectivity

DL	Downlink

EARFCN	E-UTRA Absolute Radio Frequency Channel Number

E-CID	Enhanced Cell-ID (positioning method)

eNB	E-UTRAN NodeB

EN-DC	E-UTRA-NR Dual Connectivity

EP	Elementary Procedure

EPC	Evolved Packet Core

E-RAB	E-UTRAN Radio Access Bearer

E-UTRAN	Evolved UTRAN

GNSS	Global Navigation Satellite System

GUMMEI	Globally Unique MME Identifier

HFN	Hyper Frame Number

IE	Information Element

L-GW	Local GateWay

LWA	LTE-WLAN Aggregation

MCG	Master Cell Group

MDT	Minimization of Drive Tests

MeNB	Master eNB

MME	Mobility Management Entity

MTSI	Multimedia Telephony Service for IMS

NAICS	Network-Assisted Interference Cancellation and Suppression

NR	New Radio

PDCP	Packet Data Convergence Protocol

PLMN	Public Land Mobile Network

ProSe	Proximity Service

QMC	QoE Measurement Collection

QoE	Quality of Experience

SCG	Secondary Cell Group

S-GW	Serving Gateway

SeNB	Secondary eNB

SgNB	Secondary gNB

SIPTO	Selected IP Traffic Offload

SIPTO@LN	Selected IP Traffic Offload at the Local Network

SN	Sequence Number

TAC	Tracking Area Code

UE	User Equipment

UL	Uplink

V2X	Vehicle-to-Everything

WLAN	Wireless Local Area Network

WT	WLAN Termination

------------------------------------------

Skip to next change

------------------------------------------

### 9.2.121	UE Application layer measurement configuration

The IE defines configuration information for the QoE Measurement Collection (QMC) function.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Container for application layer measurement configuration | M |  | Octet string (1..1000) | Indicates application layer measurement configuration, see Annex L in [36]. | - | - |
| CHOICE Area Scope of QMC | M |  |  |  | - | - |
| >Cell based |  |  |  |  |  | - |
| >>Cell ID List for QMC |  | 1 .. <maxnoofCellIDforQMC> |  |  |  | - |
| >>>E-CGI | M |  | 9.2.1.38 |  | - | - |
| >TA based |  |  |  |  |  | - |
| >>TA List for QMC |  | 1 .. <maxnoofTAforQMC> |  |  |  | - |
| >>>TAC | M |  | 9.2.3.7 | The TAI is derived using the current serving PLMN. | - | - |
| >TAI based |  |  |  |  | - | - |
| >>TAI List for QMC |  | 1 .. <maxnoofTAforQMC> |  |  | - | - |
| >>>TAI | M |  | 9.2.3.16 |  | - | - |
| >PLMN area based |  |  |  |  |  | - |
| >>PLMN List for QMC |  | 1 .. <maxnoofPLMNforQMC> |  |  |  | - |
| >>>PLMN Identity | M |  | 9.2.3.8 |  | - | - |
| Service Type | M |  | ENUMERATED (QMC for streaming service, QMC for MSTI service, ...) | This IE indicates the service type of UE application layer measurements. | - | - |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellIDforQMC | Maximum no. of Cell ID subject for QMC scope. Value is 32. |
| maxnoofTAforQMC | Maximum no. of TA subject for QMC scope. Value is 8. |
| maxnoofPLMNforQMC | Maximum no. of PLMNs in the PLMN list for QMC scope. Value is 16. |

### 9.3.5	Information Element definitions

-- **************************************************************

--

#### -- Information Element Definitions

--

-- **************************************************************

X2AP-IEs {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

eps-Access (21) modules (3) x2ap (2) version1 (1) x2ap-IEs (2) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

id-E-RAB-Item,

id-Number-of-Antennaports,

id-MBSFN-Subframe-Info,

id-PRACH-Configuration,

id-CSG-Id,

id-MDTConfiguration,

id-SignallingBasedMDTPLMNList,

id-MultibandInfoList,

id-FreqBandIndicatorPriority,

id-NeighbourTAC,

id-Time-UE-StayedInCell-EnhancedGranularity,

id-MBMS-Service-Area-List,

id-HO-cause,

id-eARFCNExtension,

id-DL-EARFCNExtension,

id-UL-EARFCNExtension,

id-M3Configuration,

id-M4Configuration,

id-M5Configuration,

id-MDT-Location-Info,

id-NRrestriction,

id-AdditionalSpecialSubframe-Info,

id-UEID,

id-enhancedRNTP,

id-ProSeUEtoNetworkRelaying,

id-M6Configuration,

id-M7Configuration,

id-OffsetOfNbiotChannelNumberToDL-EARFCN,

id-OffsetOfNbiotChannelNumberToUL-EARFCN,

id-AdditionalSpecialSubframeExtension-Info,

id-BandwidthReducedSI,

id-extended-e-RAB-MaximumBitrateDL,

id-extended-e-RAB-MaximumBitrateUL,

id-extended-e-RAB-GuaranteedBitrateDL,

id-extended-e-RAB-GuaranteedBitrateUL,

id-extended-uEaggregateMaximumBitRateDownlink,

id-extended-uEaggregateMaximumBitRateUplink,

id-E-RABUsageReport-Item,

id-SecondaryRATUsageReport-Item,

id-UEAppLayerMeasConfig,

id-DL-scheduling-PDCCH-CCE-usage,

id-UL-scheduling-PDCCH-CCE-usage,

id-serviceType,

maxnoofBearers,

maxCellineNB,

maxEARFCN,

maxEARFCNPlusOne,

newmaxEARFCN,

maxInterfaces,

------------------------------------------

Skip to next change

------------------------------------------

#### -- S

S1TNLLoadIndicator ::= SEQUENCE {

dLS1TNLLoadIndicator			LoadIndicator,

uLS1TNLLoadIndicator			LoadIndicator,

iE-Extensions					ProtocolExtensionContainer { { S1TNLLoadIndicator-ExtIEs} } OPTIONAL,

...

}

S1TNLLoadIndicator-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

SCGChangeIndication ::= ENUMERATED {pDCPCountWrapAround, pSCellChange, other, ...}

SecondaryRATUsageReportList ::= SEQUENCE (SIZE(1..maxnoofBearers)) OF ProtocolIE-Single-Container {{SecondaryRATUsageReport-ItemIEs}}

SecondaryRATUsageReport-ItemIEs	X2AP-PROTOCOL-IES ::= {

{ ID id-SecondaryRATUsageReport-Item		CRITICALITY reject	TYPE SecondaryRATUsageReport-Item		PRESENCE mandatory},

...

}

SecondaryRATUsageReport-Item ::= SEQUENCE {

e-RAB-ID						E-RAB-ID,

secondaryRATType				ENUMERATED {nr, ...},

e-RABUsageReportList			E-RABUsageReportList,

iE-Extensions					ProtocolExtensionContainer { { SecondaryRATUsageReport-Item-ExtIEs} } OPTIONAL,

...

}

SecondaryRATUsageReport-Item-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

SeNBSecurityKey ::= BIT STRING (SIZE(256))

SeNBtoMeNBContainer ::= OCTET STRING

ServedCells ::= SEQUENCE (SIZE (1.. maxCellineNB)) OF SEQUENCE {

servedCellInfo					ServedCell-Information,

neighbour-Info					Neighbour-Information			OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { {ServedCell-ExtIEs} } OPTIONAL,

...

}

ServedCell-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

ServedCell-Information ::= SEQUENCE {

pCI					PCI,

cellId				ECGI,

tAC					TAC,

broadcastPLMNs		BroadcastPLMNs-Item,

eUTRA-Mode-Info		EUTRA-Mode-Info,

iE-Extensions		ProtocolExtensionContainer { {ServedCell-Information-ExtIEs} } OPTIONAL,

...

}

ServedCell-Information-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

{ ID id-Number-of-Antennaports		CRITICALITY ignore	EXTENSION Number-of-Antennaports				PRESENCE optional}|

{ ID id-PRACH-Configuration			CRITICALITY ignore	EXTENSION PRACH-Configuration					PRESENCE optional}|

{ ID id-MBSFN-Subframe-Info			CRITICALITY ignore	EXTENSION MBSFN-Subframe-Infolist				PRESENCE optional}|

{ ID id-CSG-Id						CRITICALITY ignore	EXTENSION CSG-Id								PRESENCE optional}|

{ ID id-MBMS-Service-Area-List		CRITICALITY ignore	EXTENSION MBMS-Service-Area-Identity-List	PRESENCE optional}|

{ ID id-MultibandInfoList			CRITICALITY ignore	EXTENSION MultibandInfoList						PRESENCE optional}|

{ ID id-FreqBandIndicatorPriority	CRITICALITY ignore	EXTENSION FreqBandIndicatorPriority				PRESENCE optional}|

{ ID id-BandwidthReducedSI			CRITICALITY ignore	EXTENSION BandwidthReducedSI					PRESENCE optional},

...

}

ServiceType ::= ENUMERATED{

qMC-for-streaming-service,

qMC-for-MSTI-service,

...

}

------------------------------------------

Skip to next change

------------------------------------------

#### -- U

UEAggregateMaximumBitRate ::= SEQUENCE {

uEaggregateMaximumBitRateDownlink	BitRate,

uEaggregateMaximumBitRateUplink		BitRate,

iE-Extensions						ProtocolExtensionContainer { {UEAggregate-MaximumBitrate-ExtIEs} } OPTIONAL,

...

}

UEAggregate-MaximumBitrate-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

{ ID id-extended-uEaggregateMaximumBitRateDownlink	CRITICALITY ignore	EXTENSION ExtendedBitRate	PRESENCE optional}|

{ ID id-extended-uEaggregateMaximumBitRateUplink	CRITICALITY ignore	EXTENSION ExtendedBitRate	PRESENCE optional},

...

}

UEAppLayerMeasConfig ::= SEQUENCE {

containerForAppLayerMeasConfig			OCTET STRING (SIZE(1..1000)),

areaScopeOfQMC		AreaScopeOfQMC,

iE-Extensions		ProtocolExtensionContainer { {UEAppLayerMeasConfig-ExtIEs} } OPTIONAL,

...

}

UEAppLayerMeasConfig-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

{ID id-serviceType	CRITICALITY ignore	EXTENSION ServiceType	PRESENCE optional},

...

}

UE-ContextKeptIndicator ::= ENUMERATED {

true,

...

}

------------------------------------------

Skip to next change

------------------------------------------

### 9.3.7	Constant definitions

-- **************************************************************

--

#### -- Constant definitions

--

-- **************************************************************

X2AP-Constants {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

eps-Access (21) modules (3) x2ap (2) version1 (1) x2ap-Constants (4) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

ProcedureCode,

ProtocolIE-ID

FROM X2AP-CommonDataTypes;

-- **************************************************************

--

#### -- Elementary Procedures

--

-- **************************************************************

------------------------------------------

Skip to next change

------------------------------------------

id-ServedNRcellsToModifyListENDCConfUpd										ProtocolIE-ID ::= 261

id-ServedNRcellsToDeleteListENDCConfUpd										ProtocolIE-ID ::= 262

id-E-RABUsageReport-Item													ProtocolIE-ID ::= 263

id-Old-SgNB-UE-X2AP-ID														ProtocolIE-ID ::= 264

id-SecondaryRATUsageReportList												ProtocolIE-ID ::= 265

id-SecondaryRATUsageReport-Item												ProtocolIE-ID ::= 266

id-ServedNRCellsToActivate													ProtocolIE-ID ::= 267

id-ActivatedNRCellList														ProtocolIE-ID ::= 268

id-SelectedPLMN																ProtocolIE-ID ::= 269

id-serviceType										´						ProtocolIE-ID ::= xxx

END