3GPP TSG-RAN WG3 Meeting #99bis	R3-182426

# Sanya, P.R China, 16th- 20th April  2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.425 | CR | 0037 | rev | 1 | Current version: | 15.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on padding |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  |  | Date: |  | 2018-04-06 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Clarification and correction are needed in padding, spare extension. |  |  |  |  |  |  |  |  |
| Summary of change: |  | To clarify what is the padding (octets), padding bits. Change the spare extension to Future Extension. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The meaning of padding may not be clear. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.5, 5.5.1, 5.5.3.24, A.1, A.1.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

## 5.5	Elements for the NR user plane protocol

### 5.5.1	General

In the present document the structure of frames is specified by using figures similar to figure 5.5.1-1.

| Bits |  |  |  |  |  |  |  |  | Number of Octets |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 7 | 6 | 5 | 4 | 3 | 2 |  | 1 | 0 |  |
| Field 1 |  |  |  | Field 2 |  |  |  |  | 1 | Octet 1 |
| Field 3 |  |  |  |  |  | Field 4 |  |  | 2 | Octet 2 |
| Field 4 continue |  |  |  | Spare |  |  |  |  |  | Octet 3 |
| Field 6 |  |  |  |  |  |  |  |  | 2 | Octet 4 Octet 5 |
| Field 6 continue |  |  |  | Padding Bits |  |  |  |  |  |  |
| Future Extension |  |  |  |  |  |  |  |  | 0-m |  |
| Padding |  |  |  |  |  |  |  |  | 0-3 |  |

Figure 5.5.1-1: Example frame format

Unless otherwise indicated, fields which consist of multiple bits within an octet have the most significant bit located at the higher bit position (indicated above frame in figure 5.5.1-1). In addition, if a field spans several octets, most significant bits are located in lower numbered octets (right of frame in figure 5.5.1-1).

The frame is transmitted starting from the lowest numbered octet. Within each octet, the bits are sent according decreasing bit position (bit position 7 first).

Bits labelled "Spare" should be set to "0" by the sender and should not be checked by the receiver.

The header part of the frame is always an integer number of octets. The payload part is octet aligned (by adding 'Padding Bits' when needed). The total size of the frame shall not exceed 1018 octets (see TS 29.281 [2]).

The receiver should be able to remove an additional Future Extension field that may be present. 
See description of Future Extension field in A.1.

Padding octets may be added at the end of the frame, see Padding in 5.5.3.24.

------------------------------------------

Skip to next change

------------------------------------------

#### 5.5.3.24	Padding

Description: The padding is included at the end of the frame to ensure that the NR user plane protocol pdu length (including padding and the Future Extension) is (n*4– 2) octets, where n is a positive integer . If there is any Future Extension, the padding should be added after the Future Extensions.

Field Length: 0–3 octets.

------------------------------------------

Skip to next change

------------------------------------------

# A.1	Example of using Future Extension field

| New IE flag 7(E) | New IE flag 6 | New IE flag 5 | New IE flag 4 | New IE flag 3 | New IE flag 2 | New IE flag 1 | New IE flag 0 | 1 Octet New IE Flags |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| New IE 1 |  |  |  |  |  |  |  | 3 Octets |
| New IE 2 |  |  |  |  |  |  |  | 2 Octets |

Figure X.Y: Example of Future Extension field

In the Example of the Future Extension field, New IE flag 0 indicates if the New IE 1 is present or not. New IE flag 1 indicaes if the new IE 2 is present or not.

## A.1.1	New IE Flags

Description: The New IE Flags IE is only present if at least one new IE is present. The New IE Flags IE contains flags indicating which new IEs that are present following the New IE Flags IE. The last bit position of the New IE Flags IE is used as the Extension Flag to allow the extension of the New IE Flags IE in the future. Extension octets of the New IE Flags IE shall follow directly after the first octet of the New IE Flags IE. When an extension octet of the New IE Flags IE is present, then all previous extension octets of the New IE Flags IE and the New IE Flags IE shall also be present, even if they have all their flag bits indicating no presence of their respective new IEs.