3GPP TSG-RAN WG3 #99bis	Tdoc R3-182442

Sanya, P. R. China, 16th- 20th April  2018

Agenda Item:	23.3

Source:	Ericsson

Title:	Bearer Context Setup procedure

Document for:	pCR TS 38.463

# Introduction

This contribution is for CB 50.

CB: # 50_BearerMgmt

-  merge from 22412056,1733,1849,2132 if agreeable

- check details

- clarify use of bearer IDs; avoid the possibility of duplication

# Annex I: TP for 38.463

Start of Text Proposal 1 for TS 38.463

#### 9.2.1.1	BEARER CONTEXT SETUP REQUEST

This message is sent by the gNB-CU-CP to request the gNB-CU-UP to setup a bearer context.

Direction: gNB-CU-CP  gNB-CU-UP

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.X |  | YES | reject |
| gNB-CU-CP UE E1AP ID | M |  | 9.3.1.Y |  | YES | ignore |
| Security Information | M |  | 9.3.1.A |  | YES | reject |
| CHOICE system | M |  |  |  | YES | reject |
| >E-UTRAN |  |  |  |  |  |  |
| >>DRB To Setup List |  | 1 |  |  | YES | reject |
| >>>DRB To Setup Item |  | 1..<maxnoofDRBs> |  |  | EACH | reject |
| >>>>DRB ID | M |  |  |  | - | - |
| >>>>PDCP Configuration | M |  | OCTET STRING | PDCP-config as defined in TS 38.331 [y]. | - | - |
| >>>>E-UTRAN QoS | M |  |  |  | - | - |
| >>>>S1 UL UP Transport Layer Information | M |  |  | 9.3.1.G | - | - |
| >>>>Data forwarding information | O |  |  |  | - | - |
| >>>>Cell Group Information | M |  | 9.3.1.B |  | - | - |
| >NG-RAN |  |  |  |  |  |  |
| >>PDU Session Resource To Setup List |  | 1 |  |  | YES | reject |
| >>>PDU Session Resource To Setup Item |  | 1..<maxnoofPDUSessionResource> |  |  | EACH | reject |
| >>>>PDU Session ID | M |  |  |  | - | - |
| >>>>S-NSSAI | O |  |  |  | - | - |
| >>>>PDU Session Resource Setup Request Transfer | M |  |  | Defined in TS 38.413 [x]. | - | - |
| >>>>DRB To Setup List |  | 1 |  |  | YES | reject |
| >>>>>DRB To Setup Item |  | 1..<maxnoofDRBs> |  |  | EACH | reject |
| >>>>>>DRB ID | M |  |  |  | - | - |
| >>>>SDAP Configuration | M |  | OCTET STRING | SDAP-config as defined in TS 38.331 [y]. | - | - |
| >>>>>>PDCP Configuration | M |  | OCTET STRING | PDCP-config as defined in TS 38.331 [y] | - | - |
| >>>>>>Data forwarding information | O |  |  |  | - | - |
| >>>>>>Cell Group Information | M |  | 9.3.1.B |  | - | - |
| >>>>>>Flow Mapping Information | O |  | 9.3.1.C |  | - | - |

| Range bound | Explanation |
| --- | --- |
| maxnoofDRBs | Maximum no. of DRBs for a UE. Value is 32. |
| maxnoofPDUSessionResource | Maximum no. of PDU Sessions for a UE. Value is FFS. |

Editor’s note: The presence of all the IEs in this message needs further checking. The definition of the IEs is FFS.

Editor’s note:   Whether to introduce E-RAB ID for E-UTRAN is FFS.

Editor’s note:   Whether it is needed to distinguish between different interfaces (F1, X2, Xn) is FFS.

Editor’s note:   How to encode information about NAS reflective QoS (RQA) and AS reflective QoS is FFS (i.e., whether to have separate IEs or include it in the QoS flow level information).

#### 9.2.1.2	BEARER CONTEXT SETUP RESPONSE

This message is sent by the CU-UP to confirm the setup of the requested bearer context.

Direction: CU-UP  CU-CP

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.X |  | YES | reject |
| gNB-CU-CP UE E1AP ID | M |  | 9.3.1.Y |  | YES | reject |
| gNB-CU-UP UE E1AP ID | M |  | 9.3.1.Z |  | YES | reject |
| CHOICE system | M |  |  |  | YES | reject |
| >E-UTRAN |  |  |  |  |  |  |
| >>DRB Setup List |  | 1 |  |  | YES | ignore |
| >>>DRB Setup Item |  | 1..<maxnoofDRBs> |  |  | EACH | ignore |
| >>>>DRB ID | M |  |  |  | - | - |
| >>>>S1 DL UP Transport Layer Information | M |  | 9.3.1.G |  | - | - |
| >>>>Data forwarding information | O |  |  |  | - | - |
| >>>>UL UP Transport Parameters | M |  | 9.3.1.D |  | - | - |
| >>DRB Failed List |  | 0.. 1 |  |  | YES | ignore |
| >>>DRB Failed Item |  | 1..<maxnoofDRBs> |  |  | EACH | ignore |
| >>>>DRB ID | M |  |  |  | - | - |
| >>>>Cause | M |  |  |  | - | - |
| >NG-RAN |  |  |  |  |  |  |
| >>PDU Session Resource Setup List |  | 1 |  |  | YES | ignore |
| >>>PDU Session Resource Setup Item |  | 1..<maxnoofPDUSessionResource> |  |  | EACH | ignore |
| >>>>PDU Session ID | M |  |  |  | - | - |
| >>>>PDU Session Resource Setup Response Transfer | M |  |  | Defined in TS 38.413 [x]. | - | - |
| >>>>DRB Setup List |  | 1 |  |  | - | - |
| >>>>>DRB Setup Item |  | 1..<maxnoofDRBs> |  |  | - | - |
| >>>>>>DRB ID | M |  |  |  | - | - |
| >>>>>>Data forwarding information | O |  |  |  | - | - |
| >>>>>>UL UP Parameters | M |  | 9.3.1.D |  | - | - |
| >>>>>>Flow Setup List | M |  | 9.3.1.C |  |  |  |
| >>>>>>Flow Failed List | M |  | 9.3.1.C |  |  |  |
| >>>>DRB Failed List |  | 0.. 1 |  |  | YES | ignore |
| >>>>>DRB Failed Item |  | 1..<maxnoofDRBs> |  |  | EACH | ignore |
| >>>>>>DRB ID | M |  |  |  | - | - |
| >>>>>>Cause | M |  |  |  | - | - |
| >>PDU Session Resource Failed List |  | 0.. 1 |  |  | YES | ignore |
| >>>PDU Session Resource Failed Item |  | 1..<maxnoofPDUSessionResource> |  |  | EACH | ignore |
| >>>>PDU Session ID | M |  |  |  | - | - |
| >>>>Cause | M |  |  |  | - | - |

| Range bound | Explanation |
| --- | --- |
| maxnoofDRBs | Maximum no. of DRBs for a UE. Value is 32. |
| maxnoofPDUSessionResource | Maximum no. of PDU Sessions for a UE. Value is FFS. |

Editor’s note: The presence of all the IEs in this message needs further checking. The definition of the IEs is FFS.

Editor’s note:   Whether to introduce E-RAB ID for E-UTRAN is FFS.

Editor’s note:   Whether it is needed to distinguish between different interfaces (F1, X2, Xn) is FFS.

Start of Text Proposal 2 for TS 38.463

#### 9.2.2.4	BEARER CONTEXT MODIFICATION REQUEST

This message is sent by the gNB-CU-CP to request the gNB-CU-UP to modify a bearer context.

Direction: gNB-CU-CP  gNB-CU-UP

Editor’s note: The presence of all the IEs in this message needs further checking. The definition of the IEs is FFS.

Editor’s note:   Whether to introduce E-RAB ID for E-UTRAN is FFS.

Editor’s note:   Whether it is needed to distinguish between different interfaces (F1, X2, Xn) is FFS.

Editor’s note:   How to encode information about NAS reflective QoS (RQA) and AS reflective QoS is FFS (i.e., whether to have separate IEs or include it in the QoS flow level information).

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.X |  | YES | reject |
| gNB-CU-CP UE E1AP ID | M |  | 9.3.1.Y |  | YES | reject |
| gNB-CU-UP UE E1AP ID | M |  | 9.3.1.Z |  | YES | reject |
| Bearer Context Status Change | O |  | ENUMERATED (Suspend, Resume, …) | Indicates to Suspend or Resume the Bearer Context | YES | reject |
| CHOICE system | M |  |  |  | YES | reject |
| >E-UTRAN |  |  |  |  |  |  |
| >>DRB To Setup List |  | 0.. 1 |  |  | YES | reject |
| >>>DRB To Setup Item |  | 1..<maxnoofDRBs> |  |  | EACH | reject |
| >>>>DRB ID | M |  |  |  | - | - |
| >>>>PDCP Configuration | M |  | OCTET STRING | PDCP-config as defined in TS 38.331 [y]. | - | - |
| >>>>E-UTRAN QoS | M |  |  |  | - | - |
| >>>>S1 UL UP Trasnport Layer Information | M |  |  |  | - | - |
| >>>>Data forwarding information | O |  |  |  | - | - |
| >>>>Cell Group Information | M |  | 9.3.1.B |  | - | - |
| >>DRB To Modify List |  | 0.. 1 |  |  | YES | reject |
| >>>DRB To Modify Item |  | 1..<maxnoofDRBs> |  |  | EACH | reject |
| >>>>DRB ID | M |  |  |  | - | - |
| >>>>PDCP Configuration | O |  | OCTET STRING | PDCP-config as defined in TS 38.331 [y]. | - | - |
| >>>>E-UTRAN QoS | O |  |  |  | - | - |
| >>>>S1 UL UP Parameters | O |  |  |  | - | - |
| >>>>Data forwarding information | O |  |  |  | - | - |
| >>>>PDCP UL COUNT | O |  |  | PDCP count for first un-acknowledged UL packet. | - | - |
| >>>>PDCP DL COUNT | O |  |  | PDCP count for next DL packet to be assigned. | - | - |
| >>>>DL UP Parameters | O |  | 9.3.1.D |  | - | - |
| >>>>Cell Group To Add | O |  | 9.3.1.B |  | - | - |
| >>>>Cell Group To Modify | O |  | 9.3.1.B |  | - | - |
| >>>>Cell Group To Remove | O |  | 9.3.1.B |  | - | - |
| >>DRB To Remove List |  | 0.. 1 |  |  | YES | reject |
| >>>DRB To Remove Item |  | 1..<maxnoofDRBs> |  |  | EACH | reject |
| >>>>DRB ID | M |  |  |  | - | - |
| >NG-RAN |  |  |  |  |  |  |
| >>PDU Session Resource To Setup List |  | 0.. 1 |  |  | YES | reject |
| >>>PDU Session Resource To Setup Item |  | 1..<maxnoofPDUSessionResource> |  |  | EACH | reject |
| >>>>PDU Session ID | M |  |  |  | - | - |
| >>>>S-NSSAI | O |  |  |  | - | - |
| >>>>PDU Session Resource Setup Request Transfer | M |  |  | Defined in TS 38.413 [x]. | - | - |
| >>>>DRB To Setup List |  | 1 |  |  |  |  |
| >>>>>DRB To Setup Item |  | 1..<maxnoofDRBs> |  |  |  |  |
| >>>>>>DRB ID | M |  |  |  | - | - |
| >>>>SDAP Configuration | M |  | OCTET STRING | SDAP-config as defined in TS 38.331 [y]. | - | - |
| >>>>>>PDCP Configuration | M |  | OCTET STRING | PDCP-config as defined in TS 38.331 [y] | - | - |
| >>>>>>Data forwarding information | O |  |  |  | - | - |
| >>>>>>Cell Group Information | M |  | 9.3.1.B |  | - | - |
| >>>>>>Flow Mapping Information | O |  | 9.3.1.C |  | - | - |
| >>PDU Session Resource To Modify List |  | 0.. 1 |  |  | YES | reject |
| >>>PDU Session Resource To Modify Item |  | 1..<maxnoofPDUSessionResource> |  |  | EACH | reject |
| >>>>DRB To Setup List |  | 1 |  |  |  |  |
| >>>>>DRB To Setup Item |  | 1..<maxnoofDRBs> |  |  |  |  |
| >>>>>>DRB ID | M |  |  |  | - | - |
| >>>>>>SDAP Configuration | O |  | OCTET STRING | SDAP-config as defined in TS 38.331 [y]. | - | - |
| >>>>>>PDCP Configuration | O |  | OCTET STRING | PDCP-config as defined in TS 38.331 [y] | - | - |
| >>>>>>Data forwarding information | O |  |  |  | - | - |
| >>>>>>Cell Group Information | M |  | 9.3.1.B |  | - | - |
| >>>>>>Flow Mapping Information | O |  | 9.3.1.C |  | - | - |
| >>>>DRB To Modify List |  | 0.. 1 |  |  |  |  |
| >>>>>DRB To Modify Item |  | 1..<maxnoofDRBs> |  |  | EACH | reject |
| >>>>>>DRB ID | M |  |  |  | - | - |
| >>>>>>SDAP Configuration | O |  | OCTET STRING | SDAP-config as defined in TS 38.331 [y]. | - | - |
| >>>>>>PDCP Configuration | O |  | OCTET STRING | PDCP-config as defined in TS 38.331 [y]. | - | - |
| >>>>>>Data forwarding information | O |  |  |  | - | - |
| >>>>>>PDCP UL COUNT | O |  |  | PDCP count for first un-acknowledged UL packet. | - | - |
| >>>>>>PDCP DL COUNT | O |  |  | PDCP count for next DL packet to be assigned. | - | - |
| >>>>>>DL UP Parameters | O |  | 9.3.1.D |  | - | - |
| >>>>>>Cell Group To Add | O |  | 9.3.1.B |  | - | - |
| >>>>>>Cell Group To Modify | O |  | 9.3.1.B |  | - | - |
| >>>>>>Cell Group To Remove | O |  | 9.3.1.B |  | - | - |
| >>>>DRB To Remove List |  | 0.. 1 |  |  | YES | reject |
| >>>>>DRB To Remove Item |  | 1..<maxnoofDRBs> |  |  | EACH | reject |
| >>>>>>DRB ID | M |  |  |  | - | - |
| >>PDU Session Resource To Remove List |  | 0.. 1 |  |  | YES | reject |
| >>>PDU Session Resource To Remove Item |  | 1..<maxnoofPDUSessionResource> |  |  | EACH | reject |
| >>>>PDU Session ID | M |  |  |  | - | - |

### 9.3.1	Radio Network Layer Related IEs

#### 9.3.1.A	Security information

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Ciphering Algorithm | M |  | 9.3.1.B2 |  | YES | reject |
| User Plane Security Keys | M |  | 9.3.1.B3 |  | YES | reject |

Editor’s note: The definition of this IE needs further checking.

#### 9.3.1.B	Cell Group Information

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Cell Group List |  | 1 |  |  | YES | reject |
| >Cell Group Item |  | 1..<maxnoofCellGroups> |  |  | EACH | reject |
| >>Cell Group ID | M |  | INTEGER (0..3, …) | Cell group ID as defined in TS 38.331 (0=MCG, 1=SCG). | - | - |
| >>UL Configuration | O |  | 9.3.1.G2 | If CG is used for UL traffic. | - | - |
| >>DL Configuraiton | O |  | 9.3.1.G3 | If CG is used for DL traffic. | - | - |
| >>RAT Type | O |  | ENUMERATED (E-UTRA, NR, …) | Indicates the RAT. |  |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellGroups | Maximum no. of cell groups for a DRB. Value is FFS. |

Editor’s note: The definition of this IE is FFS.

#### 9.3.1.C	Flow List

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| QoS Flow List |  | 1 |  |  |  |  |
| >QoS Flow Item |  | 1..<maxnoofflows> |  |  |  |  |
| >>QoS Flow Indicator | M |  | 9.3.1.O |  | - | - |

Editor’s note: The definition of this IE is FFS.

#### 9.3.1.D	UP Parameters

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| UP Parameters List |  | 1 |  |  | - | - |
| >UP Parameters Item |  | 1..<maxnoofUPParameters> |  |  | - | - |
| >>UP Transport Layer Information | M |  | 9.3.1.G | As defined in TS 38.413 [x]. | - | - |
| >>Cell Group ID | O |  | INTEGER (0..3, …) | Cell group ID as defined in TS 38.331 (0=MCG, 1=SCG). | - | - |

| Range bound | Explanation |
| --- | --- |
| maxnoofUPParameters | Maximum no. of UP parameters (e.g., GTP tunnels) for a DRB. Value is FFS. |

Editor’s note: The definition of this IE is FFS.

End of Text Proposal for TS 38.463