3GPP TSG RAN WG3 #99bis	Tdoc R3-182455

# Sanya, P. R. China, 16th- 20th April  2018

Agenda Item:	10.10.1

Source:	Ericsson

Title:	Clarifications on multiple TNL associations

Document for:	TP for NR BL CR for TS 38.401

# Introduction

This contribution contains a text proposal based on the following CB:

CB: # 57_multSCTPsetup

-  simplify if agreeable

- keep figures if possible

- check details

# Annex 1: TP for BL CR to TS 38.401

Start of Text Proposal 1 for TS 38.401

## 8.X		Multiple TNLAs for F1-C

In the following, the procedure for managing multiple TNLAs for F1-C is described.

Figure 8.X.1: Managing multiple TNLAs for F1-C.

1. The gNB-DU establishes the first TNLA with the gNB-CU using a configured TNL address.

2. – 3. Once the TNLA has been established, the gNB-DU initiates the F1 setup procedure to exchange application level configuration data

4. – 5. When needed, the gNB-CU may add additional TNL Endpoint(s) to be used for F1-C signalling between the gNB-CU and the gNB-DU pair using the gNB-CU Configuration Update procedure. The gNB-CU Configuration Update procedure also allows the gNB-CU to request the gNB-DU to modify or release TNLA(s).

Editor’s note: Further clarification regarding the establishment of the TNLA are needed.

The F1AP UE TNLA binding is a binding between a F1AP UE association and a specific TNL association for a given UE. After the F1AP UE TNLA binding is created, the gNB-CU can update the UE TNLA binding by sending the F1AP message for the UE to the gNB-DU via a different TNLA. The gNB-DU shall update the F1AP UE TNLA binding with the new TNLA.

End of Text Proposal for TS 38.401