3GPP TSG-RAN WG3#NR AH 1807	R3-183613

Montreal, Canada, 02 - 06 July 2018

Agenda item:	10.5.6.1

Source:	Nokia, Nokia Shanghai Bell

Title:	(TP for NR BL CR for TS 38.413): Handling of Inter-system end markers

Document for:	Discussion and Decision

# 1	Description

Tdoc [3] and tdoc [4] have explained and proposed that:

- Either QoS flows are forwarded without QFI tag over DRB (intra-system)/E-RAB (inter-system) tunnels and there are end markers (without QFI tag) for each for those tunnels (1),

- Or QoS flows are forwarded with QFI-tag over a PDU session tunnel (intra-system) and end markers per PDU session tunnel are sent without QFI-tag – (2)

NOTE: at last RAN3 meetings it was agreed that for intra-system, for a given one QoS flow, either (1) or (2) applies.

Because (1) and (2) constitutes different treatment, they shall be distinguished at stage 3 level. Moreover:

- Given that a given QoS flow is either handled by (1) or by (2), a loop per QoS flows can be encoded in NGAP,

- For (1), similar handling is applied for DRB or E-RAB so no need to distinguish to exploit the commonality i.e. NG-RAN node can give one TEID per DRB/E-RAB and SMF can manage towards the peer side either DRB ID (intra-system) or E-RAB ID (for E-RAB case).

# 2	Conclusion and Proposal

Proposal: agree the TP below to encode TS 38.413.

# 3	Text Proposal for TS 38.413

#### 9.3.4.10	Handover Command Transfer

This IE is transparent to the AMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| DL Forwarding UP TNL Information | O |  | UP Transport Layer Information 9.3.2.2 | To deliver forwarded DL PDUs over the PDU Session tunnel. |
| QoS Flow to be Forwarded List |  | 1 |  |  |
| >QoS Flow to be Forwarded Item IEs |  | 1..<maxnoofQoSFlows> |  |  |
| >>QoS Flow Indicator | M |  | 9.3.1.51 |  |
| Data Forwarding Response Tunnel List | O |  | 9.3.1.77 | Contains information of the QoS flows mapped to the DRB or E-RAB tunnel. |

| Range bound | Explanation |
| --- | --- |
| maxnoofQoSFlows | Maximum no. of QoS flows allowed within one PDU session. Value is 64. |

#### 9.3.4.11	Handover Request Acknowledge Transfer

This IE is transparent to the AMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| DL NG-U UP TNL Information | M |  | UP Transport Layer Information 9.3.2.2 | NG-RAN node endpoint of the NG-U transport bearer, for delivery of DL PDUs. |
| DL Forwarding UP TNL Information | O |  | UP Transport Layer Information 9.3.2.2 | To deliver forwarded DL PDUs over the PDU Session tunnel. |
| Security Result | O |  | 9.3.1.59 |  |
| QoS Flow Setup Response List |  | 1 |  | Contains information of the QoS flows mapped to the PDU Session forwarding tunnel. |
| >QoS Flow Setup Response Item IEs |  | 1..<maxnoofQoSFlows> |  |  |
| >>QoS Flow Indicator | M |  | 9.3.1.51 |  |
| >>Data Forwarding Accepted | O |  | 9.3.1.62 |  |
| QoS Flow Failed to Setup List | O |  | QoS Flow List 9.3.1.13 |  |
| Data Forwarding Response Tunnel List | O |  | 9.3.1.77 | Contains information of the QoS flows mapped to the DRB or E-RAB tunnel. |

| Range bound | Explanation |
| --- | --- |
| maxnoofQoSFlows | Maximum no. of QoS flows allowed within one PDU session. Value is 64. |

#### 9.3.1.77	Data Forwarding Response Tunnel List

This IE indicates data forwarding related information.

| IE/Group Name |  | Presence |  | Range |  | IE type and reference |  | Semantics description |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Data Forwarding Response Tunnel Item IEs |  | 1..<maxnoofDRBs> |  |  |
| >QoS Flow Mapping List |  |  |  | 1 |  |  |  |  |  |
| >>QoS Flow Mapping Item IEs |  |  |  | 1..<maxnoofQoSFlows> |  |  |  |  |  |
| >>>QoS Flow Indicator | M |  | 9.3.1.51 |  |  |
| >DL Forwarding UP TNL Information |  | M |  |  |  | UP Transport Layer Information 9.3.2.2 |  |  |  |
| >UL Forwarding UP TNL Information |  | O |  |  |  | UP Transport Layer Information 9.3.2.2 |  |  |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofDRBs | Maximum no. of DRBs allowed towards one UE. Value is 32. |

Next Change

### 9.4.4	PDU Definitions

-- **************************************************************

--

-- PDU definitions for NGAP.

--

-- **************************************************************

NGAP-PDU-Contents {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-Access (22) modules (3) ngap (1) version1 (1) ngap-PDU-Contents (1) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- **************************************************************

--

#### -- IE parameter types from other modules.

--

-- **************************************************************

IMPORTS

AllowedNSSAI,

AMFName,

AMFSetID,

AMF-UE-NGAP-ID,

AssistanceDataForPaging,

BroadcastCancelledAreaList,

BroadcastCompletedAreaList,

CancelAllWarningMessages,

Cause,

CellIDListForRestart,

ConcurrentWarningMessageInd,

CPTransportLayerInformation,

CriticalityDiagnostics,

DataCodingScheme,

DirectForwardingPathAvailability,

EmergencyAreaIDListForRestart,

EmergencyFallbackIndicator,

EUTRA-CGI,

FiveG-S-TMSI,

GlobalRANNodeID,

GUAMI,

HandoverCommandTransfer,

HandoverRequestAcknowledgeTransfer,

HandoverType,

IMSVoiceSupportIndicator,

IndexToRFSP,

InfoOnRecommendedCellsAndRANNodesForPaging,

KamfChangeInd,

LocationReportingRequestType,

MaskedIMEISV,

MessageIdentifier,

MobilityRestrictionList,

NAS-PDU,

NGRAN-CGI,

NGRANTraceID,

NR-CGI,

NRPPa-PDU,

NumberOfBroadcastsRequested,

PagingDRX,

PagingOrigin,

PagingPriority,

PathSwitchRequestAcknowledgeTransfer,

PathSwitchRequestTransfer,

PDUSessionID,

PDUSessionList,

PDUSessionResourceModifyConfirmTransfer,

PDUSessionResourceModifyIndicationTransfer,

PDUSessionResourceModifyRequestTransfer,

PDUSessionResourceModifyResponseTransfer,

PDUSessionResourceNotifyTransfer,

PDUSessionResourceSetupRequestTransfer,

PDUSessionResourceSetupResponseTransfer,

PDUSessionResourceSubjectToHandover,

PLMNSupportList,

PWSFailedCellIDList,

RANNodeName,

RANPagingPriority,

RANStatusTransfer-TransparentContainer,

RAN-UE-NGAP-ID,

RelativeAMFCapacity,

RepetitionPeriod,

RoutingID,

RRCEstablishmentCause,

RRCInactiveAssistanceInformation,

SecurityContext,

SecurityKey,

SerialNumber,

ServedGUAMIList,

SliceSupportList,

S-NSSAI,

SONConfigurationTransfer,

SourceToTarget-TransparentContainer,

SupportedTAList,

TAI,

TAIListForRestart,

TargetID,

TargetToSource-TransparentContainer,

TimeStamp,

TimeToWait,

TNLAssociationList,

TNLAssociationUsage,

TNLAssociationWeightFactor,

TraceActivation,

TransportLayerAddress,

UEAggregateMaximumBitRate,

UE-associatedLogicalNG-ConnectionItem,

UEContextRequest,

UEIdentityIndexValue,

UE-NGAP-IDs,

UEPagingIdentity,

UEPresenceInAreaOfInterestList,

UERadioCapability,

UERadioCapabilityForPaging,

UESecurityCapabilities,

UnavailableGUAMIList,

UserLocationInformation,

WarningAreaList,

WarningMessageContents,

WarningSecurityInfo,

WarningType

FROM NGAP-IEs

#### -- D

DataCodingScheme ::= BIT STRING (SIZE(8))

DataForwardingAccepted ::= ENUMERATED {

data-forwarding-accepted,

...

}

DataForwardingNotPossible ::= ENUMERATED {

data-forwarding-not-possible,

...

}

DataForwardingResponseTunnelList ::= SEQUENCE (SIZE(1..maxnoofDRBs)) OF DataForwardingResponseTunnelItem

DataForwardingResponseTunnelItem ::= SEQUENCE {

qosFlowMappingList						QosFlowMappingList,

dLForwardingUP-TNLInformation		UPTransportLayerInformation								,

uLForwardingUP-TNLInformation		UPTransportLayerInformation								OPTIONAL,

iE-Extensions		ProtocolExtensionContainer {{DataForwardingResponseDRBItem-ExtIEs}}		OPTIONAL,

...

}

DataForwardingResponseDRBItem-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

DelayCritical ::= ENUMERATED {

delay-critical,

non-delay-critical,

...

}

#### -- H

HandoverCommandTransfer ::= SEQUENCE {

dLForwardingUP-TNLInformation		UPTransportLayerInformation						OPTIONAL,

qosFlowToBeForwardedList			QosFlowToBeForwardedList,

dataForwardingResponseTunnelList		DataForwardingResponseTunnelList 					OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {HandoverCommandTransfer-ExtIEs} } OPTIONAL,

...

}

HandoverCommandTransfer-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

HandoverRequestAcknowledgeTransfer ::= SEQUENCE {

dL-NGU-UP-TNLInformation			UPTransportLayerInformation,

dLForwardingUP-TNLInformation		UPTransportLayerInformation										OPTIONAL,

securityResult						SecurityResult													OPTIONAL,

qosFlowSetupResponseList			QosFlowSetupResponseListHOReqAck,

qosFlowFailedToSetupList			QosFlowList														OPTIONAL,

dataForwardingResponseTunnelList		DataForwardingResponseTunnelList									OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {HandoverRequestAcknowledgeTransfer-ExtIEs} } 	OPTIONAL,

...

}

HandoverRequestAcknowledgeTransfer-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

# References

- RP-172109, Revised Work Item on New Radio (NR) Access Technology, NTT DOCOMO

- 3GPP TS 23.501, Architecture for Next Generation System, SA2

- R3-183611, Handling of end marker packets in 5GS intra-system

- R3-183612, Handling of end marker inter-system