3GPP TSG-RAN WG3#NR AH 1807	R3-183633

Montreal, Canada, 02 – 06 July 2018

Agenda item:	31.3.1.13

Source:	Nokia, Nokia Shanghai Bell

Title:	(TP for NR BL CR for TS 38.423): Handling of Notification Control over Xn

Document for:	Discussion and Decision

# Description

Notification control is an indication allowing to report when a GBR flow cannot fulfil any longer the GFBR (Guaranteed flow bit rate).

At RAN3#99bis, tdoc [3] was agreed which provides encoding for the NGAP Notification message. The starting point of this discussion is that MN always report the notification over NG on a per flow basis.

This paper focusses now on the notification over Xn. For the notification over Xn two cases should be distinguished:

MN-terminated bearers

If the GBR flow has MCG bearer type, MN has all the information for the NG reporting. No Xn signalling is needed.

If the GBR flow has MCG split bearer type, the SN will need to report that guaranteed bit rate is not fulfilled at DRB level. We need a DRB-level reporting from SN to MN.

SN-terminated bearers

If the GBR flow has SCG bearer type, the SN is responsible for the flow-DRB mapping. It can freely decide to re-map whenever some congestion issue appears to a DRB. Therefore, a congestion issue on a DRB does not necessarily translate into the need to report to MN. These remapping operations are – and should – remain invisible to MN. In the end, if the situation becomes unsustainable, the SN will report to MN which of the offloaded flows cannot be guaranteed any longer. We need for this case a Flow-level reporting from SN to MN.

If the GBR flow has SCG split bearer type, the MN has received a request for MCG resources at DRB level. It could report an issue that guaranteed bit rate cannot be sustained at DRB level. So we need for this case a DRB-level reporting from MN to SN. Upon receiving this report, the SN could do some remapping to improve the situation, especially remapping some of the flows could relieve some of the flows from the problem. If in the end the situation becomes not sustainable, it will do the flow-level reporting from SN to MN.

Proposal 1: introduce a Notification control from SN to MN with DRB-level reporting for MN terminated and flow-level reporting for SN terminated.

Proposal 2: introduce a Notification control from MN to SN with DRB-level reporting for SN terminated.

# Proposal

It is proposed to agree the Text Proposal for TS 38.423 presented here-below which implements the two proposals above.

# References

- RP-172109, Revised Work Item on New Radio (NR) Access Technology, NTT DOCOMO

- 3GPP TS 23.501, Architecture for Next Generation System, SA2

- R3-182393, Handling of Notification Control in TS 38.413, Nokia

# 3	Text Proposal for TS 38.423

Beginning of changes

Table 8.1-2: Class 2 Elementary Procedures

| Elementary Procedure | Initiating Message |
| --- | --- |
| Handover Cancel | HANDOVER CANCEL |
| SN Status Transfer | SN STATUS TRANSFER |
| RAN Paging | RAN PAGING |
| Data Forwarding Address Indication | DATA FORWARDING ADDRESS INDICATION |
| S-NG-RAN node Reconfiguration Completion | S-NODE RECONFIGURATION COMPLETE |
| S-NG-RAN node Counter Check | S-NODE COUNTER CHECK REQUEST |
| UE Context Release | UE CONTEXT RELEASE |
| RRC Transfer | RRC TRANSFER |
| PDU Session Resource Notify | PDU SESSION RESOURCE NOTIFY |

Next change

### 8.3.X	PDU SESSION RESOURCE NOTIFY

#### 8.3.X.1	General

The purpose of the PDU Session Resource Notify procedure is to allow one NG-RAN node to send notification to another NG-RAN node for the already established QoS flow(s) or DRB(s) for a given UE. The procedure uses UE-associated signalling.

#### 8.3.X.2	Successful Operation

Figure 8.3.X.2-1: Notification Control

The NG-RAN node initiates the procedure by sending a PDU Session Resource Notify message.

The PDU Session Resource Notify message shall contain notification for the already established PDU sessions or QoS flows or DRBs which the QoS requirements are not fulfilled anymore or are fulfilled again.

For each PDU session, upon receiving the PDU Session Notify Info – MN terminated IE in the PDU Session Resource Notify message, the M-NG-RAN node shall, if supported, take into account the DRB Notification Cause IE associated with each DRB included in the Bearers Notify List IE and may inform the 5GC that the QoS of some QoS Flows can no longer be met or can be met again.

For each PDU session, upon receiving the PDU Session Notify Info – SN terminated IE in the PDU Session Resource Notify message, the S-NG-RAN node shall, if supported, take into account the DRB Notification Cause IE associated with each DRB included in the Bearers Notify List IE and may inform the M-NG-RAN node that the QoS of some QoS Flows can no longer be met or can be met again.

For each PDU session, upon receiving the PDU Session Notify Info – SN terminated IE in the PDU Session Resource Notify message, the M-NG-RAN node shall, if supported, take into account the QoS Flow Notification Cause IE associated with each QoS Flow included in the QoS Flows Notify List IE and may inform the 5GC that the QoS of some QoS Flows can no longer be met or can be met again.

#### 8.3.X.3	Abnormal Conditions

Editor’s Note:	Further details are FFS.

Next change

#### 9.1.2.X	PDU SESSION RESOURCE NOTIFY

This message is sent by the S-NG-RAN node or the M-NG-RAN node to report that the QoS of a QoS Flow/DRB involved in notification control can no longer be fulfilled for a specific UE.

Direction: S-NG-RAN node  M-NG-RAN node or NG-RAN node  M-NG-RAN node.

Editor’s Note: The tabular below is FFS.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | <reference> |  | YES | reject |
| M-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID <reference> | Allocated at the M-NG-RAN node | YES | reject |
| S-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID <reference> | Allocated at the S-NG-RAN node | YES | reject |
| PDU sessions To Be Released List |  | 0..1 |  |  | YES | ignore |
| >PDU sessions To Be Released Item |  | 1 .. <maxnoofPDUsessions> |  |  | EACH | ignore |
| >>PDU session ID | M |  | <reference> |  | – | – |
| >>Bearer Configuration to be reported |  | 1 .. <maxnoofBearerConfigs> | <reference> |  | – | – |
| >>>Choice Bearer Configuration | M |  |  |  | – | – |
| >>>>SN terminated bearer |  |  |  |  | – | – |
| >>>>>PDU Session Notify Info – SN terminated | M |  | 9.2.1.X |  | – | – |
| >>>>MN terminated bearer |  |  |  |  | – | – |
| >>>>>PDU Session Notify Info – MN terminated | M |  | 9.2.1.Y |  | – | – |

| Range bound | Explanation |
| --- | --- |
| maxnoofPDUsessions | Maximum no. of PDU sessions. Value is 256 |
| maxnoofBearerConfigs | Maximum no. of BearerConfigurations Value is FFS. Editor’s Note: So far, only MN- and SN-terminated bearer configurations are considered. |

Next change

#### 9.2.1.X	PDU Session Notify Info – SN terminated

Editor’s Note: 	Further details FFS.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Bearer Type | M |  | <reference> |  | - |  |
| QoS Flows Notify List |  | 0..1 |  |  | - |  |
| >QoS Flows Notify Item IEs |  | 1 .. <maxnoofQoSFlows> |  |  | EACH | reject |
| >>QoS Flow Indicator | M |  | <reference> |  | - |  |
| >>QoS Flow Notification cause | M |  | ENUMERATED(fullfilled, not fulfilled, …) |  | - |  |
| >>Assistance Information [FFS] | O |  |  |  | - |  |
| Bearers Notify List |  | 0..1 |  |  | - |  |
| >Bearers Notify Item IEs |  | 1 .. <maxnoofDRBs> |  |  | EACH | reject |
| >>DRB ID | M |  | <reference> |  | - |  |
| >>DRB Notification cause | M |  | ENUMERATED(fullfilled, not fulfilled, …) |  | - |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofQoSFlows | Maximum no. of QoS flows. Value is FFS |
| maxnoofDRBs | Maximum no. of DRBs. Value is FFS |

#### 9.2.1.Y	PDU Session Notify Info – MN terminated

Editor’s Note: 	Further details FFS.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Bearers Notify List |  | 0..1 |  |  | - |  |
| >Bearers Notify Item IEs |  | 1 .. <maxnoofDRBs> |  |  | EACH | reject |
| >>DRB ID | M |  | <reference> |  | - |  |
| >>DRB Notification cause | M |  | ENUMERATED(fullfilled, not fulfilled, …) |  | - |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofDRBs | Maximum no. of DRBs. Value is FFS |