3GPP TSG-RAN WG3 meeting #AH-1807	R3-183743

Montreal, Canada, 2 – 6 July 2018

Agenda item:	31.3.1.9

Source:	Nokia, Nokia Shanghai Bell

Title:	Editorial correction of NR PCI range

Document for:	Approval

# 1	Introduction

NR PCI has been agreed to have the range of 0 through 1007, but shall not be extendable. This was included in X2AP including ASN.1, but in one instance the tabular definition of a message contains still extendable NR PCI format.

# 5	Text proposal

The below changes are proposed to be included in the X2AP baseline CR.

*** First change, ommited text not changed ***

### 9.2.110	Served NR Cell Information

This IE contains cell configuration information of an NR cell that a neighbour eNB may need for the X2 AP interface.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| NR-PCI | M |  | INTEGER (0..1007) | NR Physical Cell ID | – | – |
| Cell ID | M |  | NR CGI 9.2.111 |  | – | – |
| 5GS-TAC | M |  | OCTET STRING(3) | Broadcast 5GS Tracking Area Code | – | – |
| Configured TAC | O |  | OCTET STRING (2) | This is the TAC configured in the en-gNB, different from the 5GS TAC broadcast in the NR cell and enables application of Roaming and Access Restrictions for EN-DC as specified in TS 37.340 [32]. | – |  |
| Served PLMNs |  | 1..<maxnoofBPLMNs> |  | Broadcast PLMNs | – | – |
| >PLMN Identity | M |  | 9.2.4 |  | – | – |
| CHOICE NR-Mode-Info | M |  |  |  | – | – |
| >FDD |  |  |  |  |  |  |
| >>FDD Info |  | 1 |  |  | – | – |
| >>>UL FreqInfo | M |  | NR Frequency Info 9.2.106 |  | – | – |
| >>>DL FreqInfo | M |  | NR Frequency Info 9.2.106 |  | – | – |
| >>>UL Transmission Bandwidth | M |  | NR Transmission Bandwidth 9.2.114 |  | – | – |
| >>>DL Transmission Bandwidth | M |  | NR Transmission Bandwidth 9.2.114 |  | – | – |
| >TDD |  |  |  |  |  |  |
| >>TDD Info |  | 1 |  |  | – | – |
| >>>NRFreqInfo | M |  | NR Frequency Info 9.2.106 |  | – | – |
| >>> Transmission Bandwidth | M |  | NR Transmission Bandwidth 9.2.114 |  | – | – |
| Measurement Timing Configuration | M |  | OCTET STRING | Contains the MeasurementTimingConfiguration inter-node message for the served cell, as defined in TS 38.331 [31]. | – | – |

| Range bound | Explanation |
| --- | --- |
| maxnoofBPLMNs | Maximum no. of broadcast PLMN Ids. Value is 6. |

*** Remaining text not changed ***