3GPP TSG RAN WG3 NR AH 18-07	R3-184098

Montreal, Canada, 2nd – 6th July 2018

Agenda Item:	31.3.1.8

Source:	Ericsson

Title:	Addition of SRB ID in the UE Context Release Command

Document for:	TP for BL CR for TS 38.473

# Introduction

The message UE Context Release Command message contains as an optional field the RRC Container IE. In this contribution we present ways to take advantage of this IE and the enhancements needed in the UE Context Release Command message to achieve this.

# Discussion

The UE Context Release Command message contains as an optional field the RRC Container IE. This IE could be used to convey an RRC-Reject or an RRC-Release message to the UE.

This is currently not possible though. The RRC-Reject is sent on SRB0, while the RRC-Release is sent on SRB1. The gNB-DU is not able to decide which SRB to use for sending the RRC message to the UE, as there is no SRB ID in the UE Context Release Command message.

One enhancement that is an easy way to solve the issue is to add the SRB ID in the UE Context Release Command message. The presence of the SRB ID will be conditional to the presence of the RRC container IE. In that way both cases of RRC-Reject and RRC-Release can be accommodated.

Therefore, based on the above we propose to add an SRB ID in the UE Context Release Command.

Proposal 1: It is proposed to add an SRB ID in the UE Context Release Command.

# Conclusion

In this contribution, we highlighted the fact that to be able to use the UE Context Release Command message to carry either RRC-Release or RRC-Reject, an indication about the SRB ID IE is needed.

Proposal 1: It is proposed to add an SRB ID in the UE Context Release Command.

# Annex 1: TP for the BL CR to TS 38.473

Start of Text Proposal 1 for TS 38.473

#### 9.2.2.5	UE CONTEXT RELEASE COMMAND

This message is sent by the gNB-CU to request the gNB-DU to release the UE-associated logical F1 connection.

Direction: gNB-CU  gNB-DU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| Cause | M |  | 9.3.1.2 |  | YES | ignore |
| RRC-Container | O |  | 9.3.1.6 |  | YES | ignore |
| SRB ID | O |  | 9.3.1.7 | It shall be included if the RRC-Container IE is present. The gNB-DU shall send the RRC message on the indicated SRB. | YES | ignore |

Start of Text Proposal 2 for TS 38.473

-- **************************************************************

--

-- UE Context Release (gNB-CU initiated) ELEMENTARY PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

-- UE CONTEXT RELEASE COMMAND

--

-- **************************************************************

UEContextReleaseCommand ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { UEContextReleaseCommandIEs} },

...

}

UEContextReleaseCommandIEs F1AP-PROTOCOL-IES ::= {

{ ID id-gNB-CU-UE-F1AP-ID				CRITICALITY reject	TYPE GNB-CU-UE-F1AP-ID				PRESENCE mandatory	}|

{ ID id-gNB-DU-UE-F1AP-ID				CRITICALITY reject	TYPE GNB-DU-UE-F1AP-ID				PRESENCE mandatory	}|

{ ID id-Cause							CRITICALITY ignore	TYPE Cause							PRESENCE mandatory	}|

{ ID id-RRCContainer					CRITICALITY ignore	TYPE RRCContainer					PRESENCE optional		}|

{ ID id-SRBID							CRITICALITY ignore	TYPE SRBID							PRESENCE optional		},

...

}