3GPP TSG-RAN WG3 NR AH 1807	Tdoc R3-184101

Montreal, Canada, 2nd – 6th July

Agenda Item:	31.3.1.8

Source:	Ericsson

Title:	Correction in F1 Setup Request

Document for:	TP for BL CR for TS 38.473

# Introduction

Currently, the F1 Setup Request message includes a mandatory list of served cells. This may cause some problems in installation scenarios where the operator wants to test network connectivity without configuring cell parameters. It may also cause some extra-complexity for network operation because it creates dependency between interface handling and cell handling. Therefore, we propose to make the list of served cells optional in the F1 Setup Request message.

# Discussion

The F1 Setup Request message in TS 38.473 includes a mandatory list of served cells. The reason for this design choice was related to the fact that the gNB-DU would not have reason to initiate a F1 Setup procedure, unless there are cells configured and ready to be activated.

While this is normally the case, after a more careful analysis, we identified that there are scenarios where the possibility of initiating the F1 Setup procedure without configuring cells in advance may be beneficial. For example, in installation scenarios the operators may want to test the equipment and the network connectivity before performing the configuration of the cells parameters at the nodes. In these cases, the operator may not be even yet allowed to start transmission over the air and therefore configuring the cell parameters would be an unnecessary operation.

Proposal 1: 	Change the optionality of the gNB-DU Served Cells List IE in the F1 Setup Request from Mandatory to Optional.

# Conclusion

In this contribution, we explained that there are scenarios where it is beneficial to perform F1 Setup procedure without configuring cells in advance.

Proposal 1: 	Change the optionality of the gNB-DU Served Cells List IE in the F1 Setup Request from Mandatory to Optional.

# TP for BL CR for TS 38.473

<<<<<< NEXT CHANGE >>>>>>

#### 9.2.1.4	F1 SETUP REQUEST

This message is sent by the gNB-DU to transfer information for a TNL association.

Direction: gNB-DU  gNB-CU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| gNB-DU ID | M |  | 9.3.1.9 |  | YES | reject |
| gNB-DU Name | O |  | PrintableString(SIZE(1..150,…)) |  | YES | ignore |
| gNB-DU Served Cells List |  | 0.. 1 |  | List of cells configured in the gNB-DU | YES | reject |
| >gNB-DU Served Cells Item |  | 1.. <maxCellingNBDU> |  |  | EACH | reject |
| >>Served Cell Information | M |  | 9.3.1.10 | Information about the cells configured in the gNB-DU | - | - |
| >>gNB-DU System Information | O |  | 9.3.1.18 | RRC container with system information owned by gNB-DU | - | - |

| Range bound | Explanation |
| --- | --- |
| maxCellingNBDU | Maximum no. cells that can be served by a gNB-DU. Value is 512. |

<<<<<< NEXT CHANGE >>>>>>

-- **************************************************************

--

-- F1 SETUP ELEMENTARY PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

-- F1 Setup Request

--

-- **************************************************************

F1SetupRequest ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { {F1SetupRequestIEs} },

...

}

F1SetupRequestIEs F1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID					CRITICALITY reject	TYPE TransactionID				PRESENCE mandatory	}|

{ ID id-gNB-DU-ID						CRITICALITY reject	TYPE GNB-DU-ID					PRESENCE mandatory	}|

{ ID id-gNB-DU-Name						CRITICALITY ignore	TYPE GNB-DU-Name				PRESENCE optional	}|

{ ID id-gNB-DU-Served-Cells-List		CRITICALITY reject	TYPE GNB-DU-Served-Cells-List	PRESENCE optional	},

...

}

GNB-DU-Served-Cells-List 	::= SEQUENCE (SIZE(1.. maxCellingNBDU)) OF ProtocolIE-SingleContainer { { GNB-DU-Served-Cells-ItemIEs } }

GNB-DU-Served-Cells-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-GNB-DU-Served-Cells-Item		CRITICALITY reject	TYPE		GNB-DU-Served-Cells-Item	PRESENCE mandatory	},

...

}