3GPP TSG-RAN WG3 Meeting #Ad-Hoc 1807	R3-184111

# Montreal, Canada, 2nd- 6th July  2018

Agenda Item:	31.3.1.11

Source:	Ericsson

Title: 	Criticality handling related to SMF IEs

Document for:	Discussion and Approval

# 1	Introduction

This paper discusses the criticality handling of the NGAP SMF IEs which are defined as Octets String and are transparent to AMF.

# 2	Discussion

Due to the separation between AMF and SMF in 5GC, some information elements (so called SMF IEs) are defined over NGAP as octets string and are transparent to AMF.

These IEs are designed to hold the information between SMF and NG-RAN nodes. AMF only passes the IE as octets string.

As we know, the criticality information instructs the receiver how to act when receiving an IE or an IE group that is not comprehended.

In the current specification, when the SMF IE is included in the message received by AMF, the criticality is defined at the top IE level as “reject”. For example, in the Path Switch Request, it is at the PDU Session Resource to be Switched in Downlink Item IEs level. In the case of “reject”, the Criticality Diagnostics would not be able to point out which IEs in the SMF IE structure caused failure.

#### PATH SWITCH REQUEST

This message is sent by the NG-RAN node to inform the AMF of the new serving NG-RAN node and to transfer some NG-U DL tunnel termination point(s) to the SMF via the AMF for one or multiple PDU session resources.

Direction: NG-RAN node  AMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| RAN UE NGAP ID | M |  | 9.3.3.2 |  | YES | reject |
| Source AMF UE NGAP ID | M |  | AMF UE NGAP ID 9.3.3.1 |  | YES | reject |
| User Location Information | M |  | 9.3.1.16 |  | YES | ignore |
| UE Security Capabilities | M |  | 9.3.1.86 |  | YES | ignore |
| PDU Session Resource to be Switched in Downlink List |  | 1 |  |  | YES | reject |
| >PDU Session Resource to be Switched in Downlink Item IEs |  | 1..<maxnoofPDUSessions> |  |  | EACH | reject |
| >>PDU Session ID | M |  | 9.3.1.50 |  | - |  |
| >>Path Switch Request Transfer | M |  | OCTET STRING | Containing the Path Switch Request Transfer IE specified in subclause 9.3.4.8. | - |  |
| PDU Session Resource Failed to Setup List | O |  | PDU Session List 9.3.1.11 |  | YES | ignore |

#### PATH SWITCH REQUEST FAILURE

This message is sent by the AMF to inform the NG-RAN node that a failure has occurred in the 5GC during the Path Switch Request procedure.

Direction: AMF  NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| AMF UE NGAP ID | M |  | 9.3.3.1 |  | YES | ignore |
| RAN UE NGAP ID | M |  | 9.3.3.2 |  | YES | ignore |
| Cause | M |  | 9.3.1.2 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |

If we would like to get the detailed information in the Criticality Diagonstics for the SMF IE, we would need to define inside the SMF IE structure the own criticality for each sub IEs.

Proposal 1	RAN3 to discuss if to introduce criticality for the SMF IEs and communicate with CT4 when needed.

# 3	Conclusion

Proposal 1	RAN3 to discuss if to introduce criticality for the SMF IEs and communicate with CT4 when needed.

# Appendix

#### 9.3.4.8	Path Switch Request Transfer

This IE is transparent to the AMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Path Switch Request Transfer |  | 1 |  |  | YES | reject |
| >DL NG-U UP TNL Information | M |  | UP Transport Layer Information 9.3.2.2 | NG-RAN node endpoint of the NG-U transport bearer, for delivery of DL PDUs. | - |  |
| >User Plane Security Information | O |  | 9.3.1.60 |  | - |  |
| >QoS Flow Accepted List |  | 1 |  |  |  |  |
| >>QoS Flow Accepted Item IEs |  | 1..<maxnoofQoSFlows> |  |  | - |  |
| >>>QoS Flow Indicator | M |  | 9.3.1.51 |  | - |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofQoSFlows | Maximum no. of QoS flows allowed within one PDU session. Value is 64. |