3GPP TSG-RAN WG3 Meeting #Ad-Hoc 1807	R3-184114

# Montreal, Canada, 2nd- 6th July  2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.413 | CR |  | rev |  | Current version: | 1.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to PDU Session Resource Release Command |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  |  | Date: |  | 2018-06-25 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The SMF IE “PDU Session Resource Release Command Transfer ” should be introduced. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce SMF IE PDU Session Resource Release Command Transfer and the corresponding ASN.1 change. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unclear specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.2.1.3, 9.3.x.y, 9.4.4, 9.4.5, 9.4.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

### 8.2.2	PDU Session Resource Release

#### 8.2.2.1	General

The purpose of the PDU Session Resource Release procedure is to enable the release of already established PDU session resources for a given UE. The procedure uses UE-associated signalling.

#### 8.2.2.2	Successful Operation

Figure 8.2.2.2-1: PDU session resource release: successful operation

The AMF initiates the procedure by sending a PDU SESSION RESOURCE RELEASE COMMAND message.

The PDU SESSION RESOURCE RELEASE COMMAND message shall contain the information required by the NG-RAN node to release at least one PDU session in the PDU Session Resource Release Item IEs IE within the PDU Session Resource to Release List IE. If a NAS-PDU IE is contained in the message, the NG-RAN node shall pass it to the UE.

Upon reception of the PDU SESSION RESOURCE RELEASE COMMAND message the NG-RAN node shall execute the release of the requested PDU sessions. For each PDU session to be released the NG-RAN node shall release the corresponding resources over Uu and over NG.

If the RAN Paging Priority IE is included in the PDU SESSION RESOURCE RELEASE COMMAND message, the NG-RAN node may use it to determine a priority for paging the UE in RRC_INACTIVE state.

The NG-RAN node shall, if supported, report in the PDU SESSION RESOURCE RELEASE RESPONSE message location information of the UE in the User Location Information IE.

After sending a PDU SESSION RESOURCE RELEASE RESPONSE message, the NG-RAN node shall be prepared to receive a PDU SESSION RESOURCE SETUP REQUEST message requesting establishment of a PDU session with a PDU Session ID corresponding to one of the PDU Session IDs that were present in the PDU Session Resource to Release List IE of the PDU SESSION RESOURCE RELEASE COMMAND message.

If the User Location Information IE is included in the PDU SESSION RESOURCE RELEASE RESPONSE message, the AMF shall handle this information as specified in TS 23.501 [9].

#### 8.2.2.3	Unsuccessful Operation

The unsuccessful operation is specified in the successful operation section.

#### 8.2.2.4	Abnormal Conditions

Void.

--------------------------------

Skip unchanged text

---------------------------------

#### 9.2.1.3	PDU SESSION RESOURCE RELEASE COMMAND

This message is sent by the AMF and is used to request the NG-RAN node to release already established PDU session resources for a given UE.

Direction: AMF  NG-RAN node

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| AMF UE NGAP ID | M |  | 9.3.3.1 |  | YES | reject |
| RAN UE NGAP ID | M |  | 9.3.3.2 |  | YES | reject |
| RAN Paging Priority | O |  | 9.3.3.15 |  | YES | ignore |
| NAS-PDU | O |  | 9.3.3.4 |  | YES | ignore |
| PDU Session Resource to Release List |  | 1 |  |  | YES | reject |
| >PDU Session Resource Release Item IEs |  | 1..<maxnoofPDUSessionResources> |  |  | EACH | reject |
| >>PDU Session ID | M |  | 9.3.1.50 |  | - |  |
| >> PDU Session Resource Release Command Transfer | M | OCTET STRING | Containing the PDU Session Resource Release Command Transfer IE specified in subclause 9.3.x.y |  | - |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofPDUSessions | Maximum no. of PDU sessions allowed towards one UE. Value is 256. |

--------------------------------

Skip unchanged text

---------------------------------

#### 9.3.x.y	PDU Session Resource Release Command Transfer

This IE is transparent to the AMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Cause | M |  | 9.3.1.2 |  |

### 9.4.4	PDU Definitions

-- **************************************************************

--

-- PDU definitions for NGAP.

--

-- **************************************************************

NGAP-PDU-Contents {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-Access (22) modules (3) ngap (1) version1 (1) ngap-PDU-Contents (1) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- **************************************************************

--

#### -- IE parameter types from other modules.

--

-- **************************************************************

--------------------------------

Skip unchanged text

---------------------------------

PDUSessionList,

PDUSessionResourceModifyConfirmTransfer,

PDUSessionResourceModifyIndicationTransfer,

PDUSessionResourceModifyRequestTransfer,

PDUSessionResourceModifyResponseTransfer,

PDUSessionResourceNotifyTransfer,

PDUSessionResourceReleaseCommandTransfer,

PDUSessionResourceSetupRequestTransfer,

PDUSessionResourceSetupResponseTransfer,

PDUSessionResourceSubjectToHandover,

--------------------------------

Skip unchanged text

---------------------------------

id-PDUSessionResourceSetupListHOReq,

id-PDUSessionResourceSetupListSUReq,

id-PDUSessionResourceSetupListSURes,

id-PDUSessionResourceSubjectToForwardingItem,

id-PDUSessionResourceSubjectToForwardingList,

id-PDUSessionResourceToBeSwitchedDLItem,

id-PDUSessionResourceToBeSwitchedDLList,

id-PDUSessionResourceToBeSwitchedULItem,

id-PDUSessionResourceToBeSwitchedULList,

id-PDUSessionResourceToReleaseList,

id-PDUSessionResourceToReleaseItem,

id-PLMNSupportList,

id-PWSFailedCellIDList,

id-RANNodeName,

id-RANPagingPriority,

id-RANStatusTransfer-TransparentContainer,

id-RAN-UE-NGAP-ID,

id-RelativeAMFCapacity,

id-RepetitionPeriod,

--------------------------------

Skip unchanged text

---------------------------------

-- **************************************************************

--

##### -- PDU SESSION RESOURCE RELEASE COMMAND

--

-- **************************************************************

PDUSessionResourceReleaseCommand ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ {PDUSessionResourceReleaseCommandIEs} },

...

}

PDUSessionResourceReleaseCommandIEs NGAP-PROTOCOL-IES ::= {

{ ID id-AMF-UE-NGAP-ID						CRITICALITY reject	TYPE AMF-UE-NGAP-ID				PRESENCE mandatory	}|

{ ID id-RAN-UE-NGAP-ID						CRITICALITY reject	TYPE RAN-UE-NGAP-ID				PRESENCE mandatory	}|

{ ID id-RANPagingPriority					CRITICALITY ignore	TYPE RANPagingPriority			PRESENCE optional		}|

{ ID id-NAS-PDU								CRITICALITY ignore	TYPE NAS-PDU					PRESENCE optional		}|

{ ID id-PDUSessionResourceToReleaseList		CRITICALITY ignore	TYPE PDUSessionResourceToReleaseList				PRESENCE mandatory	},

...

}

PDUSessionResourceToReleaseList ::= SEQUENCE (SIZE(1..maxnoofPDUSessions)) OF ProtocolIE-SingleContainer { {PDUSessionResourceToReleaseItemIEs} }

PDUSessionResourceToReleaseItemIEs NGAP-PROTOCOL-IES ::= {

{ ID id-PDUSessionResourceToReleaseItem	 	CRITICALITY reject 	TYPE PDUSessionResourceToReleaseItem 	PRESENCE mandatory	},

...

}

PDUSessionResourceToReleaseItem ::= SEQUENCE {

pDUSessionID								PDUSessionID,

pDUSessionResourceReleaseCommandTransfer	OCTET STRING (CONTAINING PDUSessionResourceReleaseCommandTransfer),

iE-Extensions								ProtocolExtensionContainer { {PDUSessionResourceToReleaseItem-ExtIEs} }	OPTIONAL,

...

}

PDUSessionResourceToReleaseItem-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

--------------------------------

Skip unchanged text

---------------------------------

### 9.4.5	Information Element Definitions

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

NGAP-IEs {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-Access (22) modules (3) ngap (1) version1 (1) ngap-IEs (2) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

maxnoofAllowedAreas,

maxnoofAllowedS-NSSAIs,

maxnoofBPLMNs,

maxnoofCellIDforWarning,

maxnoofCellinEAI,

maxnoofCellsingNB,

maxnoofCellsinngeNB,

maxnoofCellinTAI,

maxnoofDRBs,

maxnoofEmergencyAreaID,

maxnoofEAIforRestart,

--------------------------------

Skip unchanged text

---------------------------------

#### -- P

PacketDelayBudget ::= INTEGER (0..63)	-- This IE may need to be refined

PacketErrorRate ::= INTEGER (0..63)		-- This IE may need to be refined

PacketLossRate ::= INTEGER (0..1000)

PagingAttemptInformation ::= SEQUENCE {

pagingAttemptCount					PagingAttemptCount,

intendedNumberOfPagingAttempts		IntendedNumberOfPagingAttempts,

nextPagingAreaScope					NextPagingAreaScope									OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {PagingAttemptInformation-ExtIEs} }	OPTIONAL,

...

}

PagingAttemptInformation-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

PagingAttemptCount ::= INTEGER (1..16, ...)

PagingDRX ::= INTEGER (0..63)		-- This IE may need to be refined

PagingOrigin ::= ENUMERATED {

non-3gpp,

...

}

PagingPriority ::= ENUMERATED {

priolevel1,

priolevel2,

priolevel3,

priolevel4,

priolevel5,

priolevel6,

priolevel7,

priolevel8,

...

}

PathSwitchRequestAcknowledgeTransfer ::= SEQUENCE {

uL-NGU-UP-TNLInformation		UPTransportLayerInformation,

securityIndication				SecurityIndication													OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {PathSwitchRequestAcknowledgeTransfer-ExtIEs} }	OPTIONAL,

...

}

PathSwitchRequestAcknowledgeTransfer-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

PathSwitchRequestTransfer ::= SEQUENCE {

dL-NGU-UP-TNLInformation			UPTransportLayerInformation,

userPlaneSecurityInformation		UserPlaneSecurityInformation						OPTIONAL,

qosFlowAcceptedList					QosFlowAcceptedList,

iE-Extensions		ProtocolExtensionContainer { {PathSwitchRequestTransfer-ExtIEs} }	OPTIONAL,

...

}

PathSwitchRequestTransfer-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

PDUSessionID ::= INTEGER (0..255)

PDUSessionList ::= SEQUENCE (SIZE(1..maxnoofPDUSessions)) OF PDUSessionItem

PDUSessionItem ::= SEQUENCE {

pDUSessionID		PDUSessionID,

cause				Cause,

iE-Extensions		ProtocolExtensionContainer { {PDUSessionItem-ExtIEs} } OPTIONAL,

...

}

PDUSessionItem-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

PDUSessionResourceInformationList ::= SEQUENCE (SIZE(1..maxnoofPDUSessions)) OF PDUSessionResourceInformationItem

PDUSessionResourceInformationItem ::= SEQUENCE {

pDUSessionID					PDUSessionID,

qosFlowInformationList			QosFlowInformationList,

dRBsToQosFlowsMappingList		DRBsToQosFlowsMappingList										OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {PDUSessionResourceInformationItem-ExtIEs} }	OPTIONAL,

...

}

PDUSessionResourceInformationItem-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

PDUSessionResourceModifyConfirmTransfer ::= SEQUENCE {

qosFlowModifyConfirmList		QosFlowModifyConfirmList,

qosFlowFailedToModifyList		QosFlowList																OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {PDUSessionResourceModifyConfirmTransfer-ExtIEs} }	OPTIONAL,

...

}

PDUSessionResourceModifyConfirmTransfer-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

PDUSessionResourceModifyRequestTransfer ::= SEQUENCE {

pDUSessionAggregateMaximumBitRate		BitRate															OPTIONAL,

uL-NGU-UP-TNLInformation				UPTransportLayerInformation										OPTIONAL,

qosFlowAddOrModifyRequestList			QosFlowAddOrModifyRequestList									OPTIONAL,

qosFlowToReleaseList					QosFlowList														OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {PDUSessionResourceModifyRequestTransfer-ExtIEs} }	OPTIONAL,

...

}

PDUSessionResourceModifyRequestTransfer-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

PDUSessionResourceModifyResponseTransfer ::= SEQUENCE {

dL-NGU-UP-TNLInformation			UPTransportLayerInformation												OPTIONAL,

qosFlowAddOrModifyResponseList		QosFlowAddOrModifyResponseList											OPTIONAL,

qosFlowFailedToAddOrModifyList		QosFlowList																OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {PDUSessionResourceModifyResponseTransfer-ExtIEs} }	OPTIONAL,

...

}

PDUSessionResourceModifyResponseTransfer-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

PDUSessionResourceModifyIndicationTransfer ::= SEQUENCE {

dL-UP-TNLInformation		UP-TNLInformation																OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {PDUSessionResourceModifyIndicationTransfer-ExtIEs} }	OPTIONAL,

...

}

PDUSessionResourceModifyIndicationTransfer-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

PDUSessionResourceNotifyTransfer ::= SEQUENCE {

qosFlowNotifyList		QosFlowNotifyList														OPTIONAL,

qosFlowReleasedList		QosFlowList																OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {PDUSessionResourceNotifyTransfer-ExtIEs} }	OPTIONAL,

...

}

PDUSessionResourceNotifyTransfer-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

PDUSessionResourceReleaseCommandTransfer ::= SEQUENCE {

Cause					Cause,

iE-Extensions		ProtocolExtensionContainer { {PDUSessionResourceReleaseCommandTransfer-ExtIEs} }	OPTIONAL,

...

}

PDUSessionResourceReleaseCommandTransfer-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

PDUSessionResourceSetupRequestTransfer ::= SEQUENCE {

pDUSessionAggregateMaximumBitRate		BitRate,

uL-NGU-UP-TNLInformation				UPTransportLayerInformation,

additionalUL-NGU-UP-TNLInformation		UPTransportLayerInformation										OPTIONAL,

dataForwardingNotPossible				DataForwardingNotPossible										OPTIONAL,

pDUSessionType							PDUSessionType,

securityIndication						SecurityIndication												OPTIONAL,

qosFlowSetupRequestList					QosFlowSetupRequestList,

iE-Extensions		ProtocolExtensionContainer { {PDUSessionResourceSetupRequestTransfer-ExtIEs} }	OPTIONAL,

...

}

PDUSessionResourceSetupRequestTransfer-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

PDUSessionResourceSetupResponseTransfer ::= SEQUENCE {

dL-NGU-UP-TNLInformation		UPTransportLayerInformation,

securityResult					SecurityResult															OPTIONAL,

qosFlowSetupResponseList		QosFlowSetupResponseListSURes,

qosFlowFailedToSetupList		QosFlowList																OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {PDUSessionResourceSetupResponseTransfer-ExtIEs} }	OPTIONAL,

...

}

PDUSessionResourceSetupResponseTransfer-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

--------------------------------

Skip unchanged text

---------------------------------

### 9.4.7	Constant Definitions

-- **************************************************************

--

-- Constant definitions

--

-- **************************************************************

NGAP-Constants {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-Access (22) modules (3) ngap (1) version1 (1) ngap-Constants (4) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- **************************************************************

--

#### -- IE parameter types from other modules.

--

-- **************************************************************

IMPORTS

ProcedureCode,

ProtocolIE-ID

FROM NGAP-CommonDataTypes;

--------------------------------

Skip unchanged text

---------------------------------

id-UEPresenceInAreaOfInterestList						ProtocolIE-ID ::= 125

id-UERadioCapability									ProtocolIE-ID ::= 126

id-UERadioCapabilityForPaging							ProtocolIE-ID ::= 127

id-UESecurityCapabilities								ProtocolIE-ID ::= 128

id-UnavailableGUAMIList									ProtocolIE-ID ::= 129

id-UserLocationInformation								ProtocolIE-ID ::= 130

id-WarningAreaList										ProtocolIE-ID ::= 131

id-WarningMessageContents								ProtocolIE-ID ::= 132

id-WarningSecurityInfo									ProtocolIE-ID ::= 133

id-WarningType											ProtocolIE-ID ::= 134

id-PDUSessionResourceToReleaseItem						ProtocolIE-ID ::= xxx

END