3GPP TSG RAN WG3 NR AH 18-07	Tdoc R3-184130

Montreal, Canada, 2nd – 6th July

Agenda Item:	31.3.4

Source:	Ericsson, NTT DOCOMO, INC.

Title:	Clarification on SCTP association establishment – for 38.300

Document for:	TP for BL CR for TS 38.300

# Introduction

This CR follows the discussion in R3-181774 and aims to clarify that in case of failure during the establishment of an SCTP association toward an IP address, the eNB can try using a different (source and/or destination) IP addresses among the ones available.

# TP for TS 38.300

<<<<<<<<<<<<<<<<<<<< First Change >>>>>>>>>>>>>>>>>>>>

### 15.3.1	Dynamic configuration of the NG-C interface

#### 15.3.1.1	Prerequisites

The following prerequisites are assumed:

-	An initial remote IP end point to be used for SCTP initialisation is provided to the NG-RAN node for each AMF the NG-RAN node is supposed to connect to.

NOTE: The NG-RAN node may use different source and/or destination IP end point(s) if the TNL establishment towards one IP end point fails.

NOTE: How the NG-RAN node gets the remote IP end point(s) and its own IP address are outside the scope of this specification.

<<<<<<<<<<<<<<<<<<<< Second Change >>>>>>>>>>>>>>>>>>>>

### 15.3.2	Dynamic Configuration of the Xn interface

#### 15.3.2.1	Prerequisites

The following prerequisites are necessary:

-	An initial remote IP end point to be used for SCTP initialisation is provided to the NG-RAN node.

NOTE: The NG-RAN node may use different source and/or destination IP end point(s) if the TNL establishment towards one IP end point fails.

<<<<<<<<<<<<<<<<<<<< End of Change >>>>>>>>>>>>>>>>>>>>