3GPP TSG-RAN WG3 NR AH 1807	Tdoc R3-184302

Montreal, Canada, 2nd – 6th July

Agenda Item:	31.3.1.11

Source:	Ericsson

Title:	Parallel procedures on E1

Document for:	TP for BL CR to TS 38.463

# Introduction

This contribution is part of the following CB:

- CB: # 49_ParallelProceduresE1

  - only keep change for UE-associated procedures

  - transaction IDs for non-UE-associated procedures?

  - transaction IDs for UE-associated procedures?

# Annex I: TP for BL CR to TS 38.463

<<<<<< NEXT CHANGE >>>>>>

# 5	E1AP services

E1AP provides the signalling service between the gNB-CU-CP and the gNB-CU-UP that is required to fulfil the E1AP functions described in clause 7. E1AP services are divided into two groups:

Non UE-associated services:	They are related to the whole E1 interface instance between the gNB-CU-CP and gNB-CU-UP utilising a non UE-associated signalling connection.

UE-associated services:	They are related to one UE. E1AP functions that provide these services are associated with a UE-associated signalling connection that is maintained for the UE in question.

Unless explicitly indicated in the procedure specification, at any instance in time one protocol endpoint shall have a maximum of one ongoing E1AP procedure related to a certain UE.

<<<<<< NEXT CHANGE >>>>>>