RA3GPP TSG-RAN WG3 NR AH 1807	Tdoc R3-184303

Montreal, Canada, 2nd – 6th July

Agenda Item:	31.3.1.11

Source:	Ericsson

Title:	E1 parallel procedures: summary of offline discussions

Document for:	Discussion

# Introduction

This contribution summarizes the offline discussions about how to handle non-UE-associated E1 parallel procedures, as part of the following CB:

- CB: # 49_ParallelProceduresE1

  - only keep change for UE-associated procedures

  - transaction IDs for non-UE-associated procedures?

  - transaction IDs for UE-associated procedures?

# Discussion

The offline discussion focused on how to handle non-UE-associated procedures over E1. For the non-UE-associated procedures, it was found that there may be need to support parallel procedures for being able to perform parallel E1 Reset.

The following options have been discussed:

- Add a restriction to forbid performing parallel non-UE-associated procedures: This approach was proposed in R3-184105. It was identified that this solution may introduce unnecessary limitation for the E1 Reset procedure.

- Do not add any restriction for parallel non-UE-associated procedures (“do nothing”): This approach may cause some problems, as it would allow performing e.g., multiple configuration updates that may cause misconfigurations in the network.

- Do not add any restriction for parallel non-UE-associated procedures and introduce Transaction ID:

  - Option 1: Add Transaction ID only for non-UE-associated procedures. This would follow the approach over F1AP.

  - Option 2: Add Transaction ID in the message header (affecting all procedures, both UE-associated and non-UE associated). This would follow the approach of UTRA interfaces.

- Do not add any restriction for parallel non-UE-associated procedures and introduce specific restrictions for procedures (e.g., configuration update) if/when needed.

  - This would follow the approach over some E-UTRA interfaces: e.g., X2AP.

Conclusion: continue discussion at the next meeting based on the options above.