3GPP TSG RAN WG3 NR AH 18-07	Tdoc R3-184322

Montreal, Canada, 2nd – 6th July

Agenda Item:	31.3.4

Source:	Ericsson, NTT DOCOMO, INC.

Title:	Clarification on SCTP association establishment – for 38.300

Document for:	TP for BL CR for TS 38.300

# Introduction

This CR follows the discussion in R3-181774 and aims to clarify that in case of failure during the establishment of an SCTP association toward an IP address, the eNB can try using a different (source and/or destination) IP addresses among the ones available.

# TP for TS 38.300

<<<<<<<<<<<<<<<<<<<< First Change >>>>>>>>>>>>>>>>>>>>

#### 15.3.1.2	SCTP initialization

NG-RAN establishes the first SCTP (IETF RFC 4960 [23]) using a configured IP address.

NOTE: The NG-RAN node may use different source and/or destination IP end point(s) if the SCTP establishment towards one IP end point fails. How the NG-RAN node gets the remote IP end point(s) and its own IP address are outside the scope of this specification.

<<<<<<<<<<<<<<<<<<<< Second Change >>>>>>>>>>>>>>>>>>>>

#### 15.3.2.2	SCTP initialization

NG-RAN establishes the first SCTP (IETF RFC 4960 [23]) using a configured IP address.

NOTE: The NG-RAN node may use different source and/or destination IP end point(s) if the SCTP establishment towards one IP end point fails.

<<<<<<<<<<<<<<<<<<<< End of Change >>>>>>>>>>>>>>>>>>>>