3GPP TSG-RAN WG2 #Ad Hoc 1807	R3-184347

Montreal, Canada, 2nd – 6th July, 2018

Agenda Item:	21.1.2.1

Source: 	Huawei

Title: 		Control plane protocols for architecture group 1

Document for:	TP

- Introduction

This paper provides text proposal on control plan protocols for architecture 1a according to

| R3-183787 | Control plane protocols for architecture group 1 (Huawei) | discussion CB: # 68_ArchGrp1 - merge CP aspects from: 4060,3863, as agreeable - check details - pCR (HW) Rev in R3-184347 |
| --- | --- | --- |

- TP for TR 38.874

### 8.3.4	CP alternatives for architecture 1a

In architecture 1a, the UE’s and the MT’s UP and RRC traffic can be protected via PDCP over the wireless backhaul. A mechanism has to be defined to also protect F1-AP traffic over the wireless backhaul.

The following four alternatives can be considered. Other alternatives are not precluded.

Figure 8.3.4- 1: Example for alternative 1 of architecture 1a. 1a: UE’s RRC, 1b: MT’s RRC, 1c: DU’s F1-AP

Alternative 1:

Figure 8.3.4-1 shows protocol stacks for UE’s RRC, MT’s RRC and DU’s F1-AP for alternative 1. In these examples, the adaptation layer is placed on top of RLC. On the IAB-node’s access link, the adaptation layer may or may not be included. The example does not preclude other options. This alternative has the following main features:

- The UE’s and the MT’s RRC are carried over SRB.

- On the UE’s or MT’s access link, the SRB uses an RLC-channel.

- On the wireless backhaul links, the SRB’s PDCP layer is carried over RLC-channels with adaptation layer. The adaptation layer placement in the RLC channel is the same for C-plane as for U-plane. The information carried on the adaptation layer may be different for SRB than for DRB.

- The DU’s F1-AP is encapsulated in RRC of the collocated MT. F1-AP is therefore protected by the PDCP of the underlying SRB.

- Within the IAB-donor, the baseline is to use native F1-C stack (see section 9).

Figure 8.3.4 - 2: Example for alternative 2 of architecture 1a. 2a: UE’s RRC, 2b: MT’s RRC, 2c: DU’s F1-AP

Alternative 2:

Figure 8.3.4 - 2 shows protocol stacks for UE’s RRC, MT’s RRC and DU’s F1-AP for alternative 2. In these examples, the adaptation layer resides on top of RLC. On the IAB-node’s access link, the adaptation layer may or may not be included. The example does not preclude other options. This alternative has the following main features:

- The UE’s and the MT’s RRC are carried over SRB.

- On the UE’s or MT’s access link, the SRB uses an RLC-channel.

- On the wireless backhaul link, the PDCP of the RRC’s SRB is encapsulated into F1-AP.

- The DU’s F1-AP is carried over an SRB of the collocated MT. F1-AP is protected by this SRB’s PDCP.

- On the wireless backhaul links, the PDCP of the F1-AP’s SRB is carried over RLC-channels with adaptation layer. The adaptation layer placement in the RLC channel is the same for C-plane as for U-plane. The information carried on the adaptation layer may be different for SRB than for DRB.

- Within the IAB-donor, the baseline is to use native F1-C stack (see section 9)

Figure 8.3.4 - 3: Example for alternative 3 of architecture 1a. 3a: UE’s RRC, 3b: MT’s RRC, 3c: DU’s F1-AP

Alternative 3:

Figure 8.3.4 - 3 shows protocol stacks for UE’s RRC, MT’s RRC and DU’s F1-AP for alternative 3. In these examples, the adaptation layer resides on top of RLC. On the IAB-node’s access link, the adaptation layer may or may not be included. The example does not preclude other options. This alternative has the following main features:

- The UE’s and the MT’s RRC are carried over SRB.

- On the UE’s or MT’s access link, the RRC’s SRB uses an RLC-channel. On the wireless backhaul links, the SRB’s PDCP layer is carried over RLC-channels with adaptation layer. The adaptation layer placement in the RLC channel is the same for C-plane as for U-plane. The information carried on the adaptation layer may be different for SRB than for DRB.

- The DU’s F1-AP is also carried over an SRB of the collocated MT. F1-AP is protected by this SRB’s PDCP.

- On the wireless backhaul links, the PDCP of the SRB is also carried over RLC-channels with adaptation layer.

- Within the IAB-donor, the baseline is to use native F1-C stack (see section 9).

Figure 8.3.4 - 4: Example for alternative 4 of architecture 1a. 1a: UE’s RRC, 1b: MT’s RRC, 1c: DU’s F1-AP

Alternative 4:

Figure 8.3.4 - 4 shows protocol stacks for UE’s RRC, MT’s RRC and DU’s F1-AP for alternative 4. In these examples, the adaptation layer resides on top of RLC and carries an IP-layer as discussed in section 8.2.2. This alternative has the following main features:

- The IP-layer carried by adapt is connected to the fronthaul’s IP-plane through a routing function at the IAB-donor DU. On this IP-layer, all IAB-nodes hold IP-addresses, which are routable from the IAB-donor CU-CP.

- The extended IP-plane allows native F1-C to be used between IAB-node DU and IAB-donor CU-DP. Signalling traffic can be prioritized on this IP routing plane using DSCP markings in compliance with TS 38.474.

- F1-C is protected via NDS, e.g. via D-TLS, as established by S3-181838.

- The UE’s and the MT’s RRC use SRB, which is carried over F1-C in compliance with TS 38.470.

Figure 8.3.4- X: Example for alternative Y of architecture 1a. Ya: UE’s RRC, Yb: MT’s RRC, Yc: DU’s F1-AP

Alternative 5:

Figure 8.3.4-X shows protocol stacks for UE’s RRC, MT’s RRC and DU’s F1-AP for alternative Y. In these examples, the adaptation layer is placed on top of RLC. On the IAB-node’s access link, the adaptation layer may or may not be included. The example does not preclude other options. This alternative has the following main features:

- The UE’s and the MT’s RRC are carried over SRB.

- On the UE’s or MT’s access link, the SRB uses an RLC-channel.

- On the wireless backhaul links, the SRB’s PDCP layer is carried over RLC-channels with adaptation layer. The adaptation layer placement in the RLC channel is the same for C-plane as for U-plane. The information carried on the adaptation layer may be different for SRB than for DRB.

- The DU’s F1-AP is carried over a DRB. F1-AP is therefore protected by this DRB’s PDCP.

- Within the IAB-donor, the baseline is to use native F1-U stack. The DU’s F1-AP is carried over E1 interface.

Summary:

For Encapsulation (for relaying RRC messages):

- Without F1-AP Encapsulation: The IAB node doesn’t use F1-AP to carry UE’s RRC/MT’s RRC. The IAB node maps UE’s RRC/MT’s RRC directly on RLC-channels

- Using F1-AP Encapsulation: The IAB node uses F1-AP to carry UE’s RRC/MT’s RRC. The IAB node encapsulates UE’s RRC/MT’s RRC with F1-AP RRC message containers

- Using F1-AP Encapsulation with SCTP/IP: The IAB node uses F1-AP to carry UE’s RRC/MT’s RRC. In addition, the IAB node uses SCTP/IP for adaptation layer.

For Using DRB or SRB for transmission of CP signaling (F1-AP mapping on PDCP entity):

- Encapsulated in RRC of the collocated MT: The IAB node encapsulates DU’s F1-AP. F1-AP is protected by the PDCP of the underlying SRB.

- Carried via SRB: The IAB node uses another SRB to carry DU’s F1-AP without encapsulation in RRC

- Carried over native F1-C: The IAB node uses native F1-C format to carry DU’s F1-AP

- Carried over DRB: The IAB node uses a DRB to carry DU’s F1-AP.

For Security of F1-AP:

- Via PDCP: F1-AP is protected by the PDCP

- Via DTLS: F1-AP is protected by the DTLS

The comparison analysis of the five CP alternatives are provided in the Table 8.3.4-x. More comparison aspects are not excluded.

Table 8.3.4-x. Comparison of the five CP alternatives of architecture 1a.

| Comparison aspects |  | Alt 1 | Alt 2 | Alt 3 | Alt 4 | Alt Y | Comparison analysis |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Transport for CP signaling on wireless plane | UE/IAB-MT’s RRC | SRB in access link, SRB over RLC channel in backhaul links | Same as alt 1 | Same as alt 1 | Same as alt 1 | Same as alt 1 | SRB is recommended to carry UE/IAB-MT’s RRC signaling in all the alternatives. [TBD for IAB DU’s F1AP] |
|  | IAB-DU’s F1AP | SRB of collocated MT | Same as alt 1 | Same as alt 1 | DRB | [TBD] |  |
| Encapsulation | UE/IAB-MT’s RRC | Within PDCP but without encapsulation in F1-AP of serving IAB node | Within PDCP and F1-AP of serving IAB node | Same as alt 1 | Same as alt 2 | Same with alt 1 | [TBD] |
|  | IAB-DU’s F1AP | Within RRC of collocated MT | Within PDCP of collocated MT | Same as Alt 2 | Within DTLS/SCTP/IP above RLC channel | Within PDCP of collocated MT | [TBD] |
| Security of F1AP |  | Protected by PDCP | Same as alt 1 | Same as alt 1 | Protected by DTLS | Protected by PDCP | [TBD] |
| Routing of control plane PDUs |  | Adaptation layer is responsible for routing | Same as alt 1 | Same as alt 1 | Same as alt 1 | Same as alt 1 | In all alternatives, the adaptation layer is used for routing. |
| Impact to IAB donor |  | Native F1-C as baseline | Same as alt 1 | Same as alt 1 | [TBD] | Native F1-C over E1 | [TBD] The detailed impact on native F1-AP needs further study. |

- Reference

- R3-183787, Control plane protocols for architecture group 1. Huawei

- R3-184060. Protocol stack for F1 message transfer in architecture 1a. LG Electronics Inc.

- R3-183863. Discussions on IAB CP protocol stacks of Alt1~Alt3 for architecture 1a. Samsung.

- R3-183693. Summary table for architecture group 1 CP alternatives (KDDI Corporation).