| 3GPP TSG-RAN Working Group 3 IP UTRAN Ad Hoc Meeting Swindon, U. K. September 27-29, 2000 | ### TSGR3-IPAdHoc(00)2431 |
| --- | --- |

Agenda:            3.iii) Transport network bandwidth utilisation

Source:	Motorola

Title:	Simulation Results for Iub IP Based User Plane IP Protocol Stacks

Document for:	Discussion/Approval

_________________________________________________________________________________________

# Introduction

This study area is related to the efficiency of bandwidth utilisation by e.g.  multiplexing/header compression, resource management, and the use of segmentation. Lower speed links, such as E1, or shared higher speed links may require different techniques (e.g. header compression and multiplexing) than dedicated higher speed links.

The paper presents the simulation result and comparison of the capacity (defined by the number of voice users and that of data users a system can support simultaneously) of three protocol stacks on the Iub. AAL2/ATM is as a reference. Two other low-overhead protocol stacks are studied: PPPmux/HDL-C, and LIPE.

# Protocol stacks

The simulation models are described in [1]. The size of FP PDU can also be found in [1]. We list the average overhead of a FP PDU for each protocol stack.

- ATM/AAL2: Each ATM cell assumes a payload of 45 bytes, and padding bits are not assumed. The size of a layer-2 packet is

[FP PDU + 3 (AAL2 header)] * 53.0 /45.0 (bytes).

- PPPmux/HDL-C (see [2]):   For voice frmaes, we limit the size of PPPmux to be less than 420 bytes (the maximal number of multiplex is 10). The number is arbitrary chosen such that some packets that have higher priority than voice packets, for example, real-time, out-band, call control packets, are delayed by no more than 1.75 ms behind a voice PPPmux frame. For data frames, we do not use PPPmux to avoid to delay higher priority traffic, e.g., voice traffic,  because most data frames are larger than 340 bytes (need 1.4 ms to transmit). Assuming multiplexing ten voice packets together and multiplexing no data packets, the size of a layer-2 packet is computed in the following.

PPPmux for voice: FP PDU + 4 (cUDP + Len) + [1(HDLC)+ 1 (PPPmux) + 1 (PPP ID) + 2(CRC)]/10 (bytes)

PPP for data: FP PDU + 3 (cUDP) + 1 (PPP ID)  + 1 + (HDLC) + 2 (CRC) (bytes)

- LIPE (see [3]): The criteria for voice traffic and data traffic of  LIPE is the same as in (2) PPPmux/HDL-C. Assuming multiplexing ten voice packets together and multiplexing no data packets, the size of a layer-2 packet is computed in the following.

LIPE for voice: FP PDU + 3 (MH) + [1(HDLC)+ 3 (cUDP) + 1 (PPPmux) + 2(CRC)]/10 (bytes)

PPP for data: FP PDU + 3 (cUDP) + 1 (PPP ID)  + 1 + (HDLC) + 2 (CRC) (bytes)

# Summary of Simulation Results

As a baseline case for comparison, simulation cases are run with voice and data traffic transported on AAL2 protocol stacks over an E1 line (30 DSO). Four scenarios are studied: 100% Voice Traffic, 100% Data Traffic, “80% Voice: 20% Data” Traffic, and “20% Voice: 80% Data” Traffic.  Comparison of three radio-link data rates is studied: 64, 144, 384 Kbps. Each statistic is the average of ten simulation runs. Each simulation run is 20,000 sec. The 95% confidence intervals of 99.9% voice frame delay and of 99.9% data frame delay are measured.

The capacity is decided only according to the “knee” of the curve of 99.9% voice frame delay, and of the curve of 99.9% data frame delay. However, other criteria should also be taken into account, for example, the delay requirement from RNC to Node B. The “80% Voice: 20% Data” scenario, and “20% Voice: 80% Data” scenario are roughly according to the ratio of throughput of voice traffic and that of data traffic. The summary of simulation results is included in this section. Its details are in section 4.

| Voice only (AAL2/ATM; PPPmux/HDL-C; LIPE) |  |  |  |
| --- | --- | --- | --- |
|  | Capacity (# of voice users) | 99.9% delay for voice frames (ms) | Utilisation of E1 (%) |
| AAL2/ATM | 120 | <= 1.39 | 89.03 |
| PPPmux/HDL-C | 136 | <= 3.04 | 87.46 |
| LIPE | 140 | <= 4.14 | 90.06 |

| Data only for 64, 144, 384 Kbps (AAL2/ATM; PPP/HDL-C) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Capacity (# of data users) | 99.9% delay for data frames (ms) |  | Utilisation of E1 (%) |  |
| 64 Kbps |  |  |  |  |  |  |
| AAL2/ATM | 120 |  |  | <= 95.84 |  | 56.66 |
| PPP/HDL-C | 140 |  |  | <= 56.3 |  | 56.56 |
| 144 Kbps |  |  |  |  |  |  |
| AAL2/ATM | 90 |  |  | <= 81.18 |  | 44.26 |
| PPP/HDL-C | 110 |  |  | <= 67.4 |  | 46.64 |
| 384 Kbps |  |  |  |  |  |  |
| AAL2/ATM | 30 |  |  | <= 62.54 |  | 16.38 |
| PPP/HDL-C | 50 |  |  | <= 67.89 |  | 23.13 |

| “80% voice : 20% data” for 64, 144, 384 Kbps (AAL2; PPPmux/HDL-C; LIPE) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Capacity |  | 99.9% delay (ms) |  | Utilisation (%) |  |
|  | # voice users | # data users | Voice | Data | Voice | Data |
| 64 Kbps |  |  |  |  |  |  |
| AAL2/ATM | 73 | 33 | <= 1.9 | <= 26.35 | 54.03 | 15.54 |
| PPPmux/HDL-C | 88 | 37 | <= 3.5 | <= 81.63 | 58.06 | 14.95 |
| LIPE | 88 | 37 | <= 3.48 | <= 17.88 | 56.59 | 15.06 |
| 144 Kbps |  |  |  |  |  |  |
| AAL2/ATM | 62 | 29 | <= 4.0 | <= 37.38 | 45.9 | 14.39 |
| PPPmux/HDL-C | 73 | 33 | <= 5.2 | <= 22.85 | 48.17 | 13.68 |
| LIPE | 80 | 35 | <= 5.08 | <= 64.33 | 51.45 | 14.72 |
| 384 Kbps |  |  |  |  |  |  |
| AAL2/ATM | 24 | 15 | <= 8.5 | <= 25.57 | 17.78 | 8.19 |
| PPPmux/HDL-C | 40 | 21 | <= 9 | <= 86.95 | 26.4 | 9.53 |
| LIPE | 40 | 21 | <= 8.98 | <= 48.76 | 25.72 | 9.62 |

| “20% voice : 80% data” for 64, 144, 384 Kbps (AAL2; PPPmux/HDL-C; LIPE) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Capacity |  | 99.9% delay (ms) |  | Utilisation (%) |  |
|  | # voice users | # data users | Voice | Data | Voice | Data |
| 64 Kbps |  |  |  |  |  |  |
| AAL2/ATM | 14 | 100 | <= 1.9 | <= 31.13 | 10.36 | 46.83 |
| PPPmux/HDL-C | 18 | 120 | <= 3.5 | <= 41.87 | 11.87 | 48.32 |
| LIPE | 17 | 225 | <= 3.48 | <= 11.2 | 10.93 | 46.27 |
| 144 Kbps |  |  |  |  |  |  |
| AAL2/ATM | 10 | 80 | <= 4 | <= 73.87 | 7.41 | 39.62 |
| PPPmux/HDL-C | 14 | 100 | <= 5.2 | <= 51.85 | 9.24 | 41.62 |
| LIPE | 15 | 105 | <= 5.18 | <= 82.84 | 9.64 | 44.04 |
| 384 Kbps |  |  |  |  |  |  |
| AAL2/ATM | * | * | * | * | * | * |
| PPPmux/HDL-C | * | * | * | * | * | * |
| LIPE | * | * | * | * | * | * |

PS. The curve of 99.9% data frame delay for “384 Kbps” does not show knees.

# Simulation results

## 4.1 100% Voice Traffic (AAL2; PPPmux; LIPE)

Figure 1 shows that the knee is at 120 voice users for AAL2/ATM, at 136 voice users for PPPmux/HDL-C, and at 140 voice users for LIPE. Table 1, Table 2, and Table 3 present the statistics of intersts.

| # of voice users | 118 | 119 | 120 | 121 | 122 |
| --- | --- | --- | --- | --- | --- |
| AAL2 utilization (%) | 87.36 | 88.1 | 89.03 | 89.58 | 90.31 |
| 99.9%voice delay(ms) | 0.9 | 0.98 | 1.39 | 4.65 | 10.49 |

Table 1. The statistics of voice only case for AAL2/ATM

| # of voice users | 134 | 135 | 136 | 137 | 138 | 139 |
| --- | --- | --- | --- | --- | --- | --- |
| PPPmux utilization(%) | 86.16 | 86.83 | 87.46 | 88.11 | 88.76 | 89.38 |
| 99.9% voice delay(ms) | 2.6 | 2.66 | 3.04 | 4.7 | 10.28 | 18.65 |

Table 2. The statistics of voice only case for PPPmux/HDL-C

| # of voice users | 137 | 138 | 139 | 140 | 141 | 142 |
| --- | --- | --- | --- | --- | --- | --- |
| LIPE utilization (%) | 88.11 | 88.76 | 89.38 | 90.06 | 90.08 | 91.33 |
| 99.9% voice delay(ms) | 2.57 | 2.61 | 2.8 | 4.14 | 8.64 | 14.05 |

Table 3. The statistics of voice only case for LIPE

## 4.2 100% Data Traffic (AAL2; PPP)

Figure 2 shows the knee is at 120 data users for AAL2/ATM, and at 140 data users for PPP/HDL-C.

| # of data users | 80 | 90 | 100 | 110 | 120 | 130 | 140 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| AAL2 utilization (%) | 37.74 | 42.5 | 46.67 | 52.46 | 56.66 | 61.29 | 65.79 |
| 99.9% delay (ms) | 6.16 | 7.04 | 7.9 | 41.63 | 95.84 | 248.49 | 474.64 |

Table 4. The statistics of data only case at 64 Kbps for AAL2/ATM

| # of Data users | 90 | 100 | 110 | 120 | 130 | 140 | 150 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| PPP utilization (%) | 36.54 | 40.13 | 45.1 | 48.72 | 52.69 | 56.56 | 61.02 |
| 99.9% delay (ms) | 5.19 | 5.7 | 6.54 | 7.19 | 11.62 | 21.08 | 204.93 |

Table 5. The statistics of data only case at 64 Kbps for PPP/HCL-C

Figure 3 shows the knee for AAL2/ATM is at 90 data users for AAL2, and at 110 data users for PPP/HDL-C.

| # of data users | 60 | 70 | 80 | 90 | 100 | 110 | 120 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| AAL2 utilization (%) | 29.4 | 34.38 | 39.46 | 44.26 | 49.35 | 54.49 | 59.59 |
| 99.9% delay (ms) | 12.74 | 14.52 | 18.5 | 81.18 | 314.12 | 275.16 | 697.55 |

Table 6. The statistics of data only case at 144 Kbps for AAL2/ATM

| # of data users | 80 | 90 | 100 | 110 | 120 | 130 | 140 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| PPP utilization (%) | 33.76 | 37.87 | 42.27 | 46.62 | 50.98 | 54.73 | 59.13 |
| 99.9% delay (ms) | 11.95 | 13.63 | 37.71 | 67.4 | 144.31 | 259.12 | 553.35 |

Table 7. The statistics of data only case at 144 Kbps for PPP/HDL-C

Figure 4 shows the knee is at 30 data users for AAL2/ATM, and at 50 data users for PPP/HDL-C.

| # of data users | 20 | 30 | 40 | 50 |
| --- | --- | --- | --- | --- |
| AAL2 utilization (%) | 10.87 | 16.38 | 21.76 | 27.17 |
| 99.9% delay (ms) | 25.52 | 62.54 | 166.78 | 302.85 |

Table 8. The statistics of data only case at 384 Kbps for AAL2/ATM

| # of data users | 40 | 50 | 60 | 70 |
| --- | --- | --- | --- | --- |
| PPP utilization (%) | 18.53 | 23.13 | 27.68 | 32.38 |
| 99.9% delay (ms) | 29.14 | 67.89 | 185.31 | 413.46 |

Table 9. The statistics of data only case at 384 Kbps for PPP/HDL-C

## 4.3 “80% Voice : 20% data” traffic

According to the utilisation of data and voice in section 2.1 and section 2.2, we roughly come out Table 10 and Table 11. In these tables, each pair of the number of voice users and that of data users yields approximately 80% voice utilisation and 20% utilisation.

| Pair | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| # of voice users | 24 | 29 | 35 | 40 | 46 | 51 | 57 |
| # of data users | 15 | 17 | 19 | 21 | 23 | 25 | 27 |

Table 10. The combination of 80% voice and 20% data (1)

| Pair (Cnt.) | 8 | 9 | 10 | 11 | 12 | 13 | 14 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| # of voice users | 62 | 68 | 73 | 80 | 88 | 93 | 98 |
| # of data users | 29 | 31 | 33 | 35 | 37 | 39 | 41 |

Table 11. The combination of 80% voice and 20% data (2)

The 99.9% delay of voice frames is consistent for all pairs within eack protocol stack: 1.9 ms for AAL2/ATM, from 3.4 ms to 3.5 ms for PPPmux/HDL-C, and from 3.38 ms to 3.48 ms for LIPE. Figure 5 shows the knee of 99.9% data frame delay is at 73 voice users/33 data users for AAL2/ATM, at 88 voice users/37 data users for both PPPmux/HDL-C and LIPE. Table 12 summarizes the utilisation while the system reaches capacity.

| Protocol stacks | AAL2/ATM | PPPmux/HDL-C | LIPE |
| --- | --- | --- | --- |
| # of voice users | 73 | 88 | 88 |
| # of data users | 33 | 37 | 37 |
| Voice utilization (%) | 54.03 | 58.06 | 56.59 |
| Data utilization (%) | 15.54 | 14.95 | 15.06 |
| Total utilization (%) | 69.57 | 73.01 | 71.65 |

Table 12. The statistics of 80% voice v. s. 20 % data at 64 Kbps

The 99.9% delay of voice frames is consistent for all pairs within eack protocol stack: from 3.9 ms to 4.0 ms for AAL2/ATM, from 5.1 ms to 5.2 ms for PPPmux/HDL-C, and from 4.98 ms to 5.08 ms for LIPE. Figure 6 shows the knee of 99.9% data frame delay is at 62 voice users/29data users for AAL2/ATM, at 73 voice users/33 data users for PPPmux/HDL-C,  and at 80 voice users/35 data users for LIPE. Table 13 summarizes the utilisation while the system reaches capacity.

| Protocol stacks | AAL2/ATM | PPPmux/HDL-C | LIPE |
| --- | --- | --- | --- |
| # of voice users | 62 | 73 | 80 |
| # of data users | 29 | 33 | 35 |
| Voice utilization (%) | 45.9 | 48.17 | 51.45 |
| Data utilization (%) | 14.39 | 13.68 | 14.72 |
| Total utilization (%) | 60.29 | 61.85 | 66.17 |

Table 13. The statistics of 80% voice v. s. 20 % data at 144 Kbps

The 99.9% delay of voice frames is consistent for all pairs within eack protocol stack: from 8.4 ms to 8.5 ms for AAL2/ATM, from 8.9 ms to 9.0 ms for PPPmux/HDL-C, and from 8.88 ms to 8.98 ms for LIPE. Figure 7 shows the knee of 99.9% data frame delay is at 24 voice users/15data users for AAL2/ATM, at 40 voice users/21 data users for PPPmux/HDL-C,  and at 40 voice users/21 data users for LIPE. Table 14 summarizes the utilisation while the system reaches capacity.

| Protocol stacks | AAL2/ATM | PPPmux/HDL-C | LIPE |
| --- | --- | --- | --- |
| # of voice users | 24 | 40 | 40 |
| # of data users | 15 | 21 | 21 |
| Voice utilization (%) | 17.78 | 26.4 | 25.72 |
| Data utilization (%) | 8.19 | 9.53 | 9.62 |
| Total utilization (%) | 25.97 | 35.93 | 35.34 |

Table 14. The statistics of 80% voice v. s. 20 % data at 384 Kbps

## 4.4  “20% Voice: 80% data” traffic

According to the utilisation of data and voice in section 2.1 and section 2.2, we roughly come out Table 15. In these tables, each pair of the number of voice users and that of data users yields approximately 20% voice utilisation and 80% data utilisation.

| Pair | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| # of voice users | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 |
| # of data users | 80 | 85 | 90 | 95 | 100 | 105 | 110 | 115 | 120 | 125 |

Table 15. The combination of 20% voice and 80% data (1)

The 99.9% delay of voice frames is consistent for all pairs within eack protocol stack: at 1.9 ms for AAL2/ATM, at 3.5 ms for PPPmux/HDL-C, and from 3.38 ms to 3.48 ms for LIPE. Figure 8 shows the knee of 99.9% data frame delay is at 24 voice users/15data users for AAL2/ATM, at 40 voice users/21 data users for PPPmux/HDL-C,  and at 40 voice users/21 data users for LIPE. Table 16 summarizes the utilisation while the system reaches capacity.

| Protocol stacks | AAL2/ATM | PPPmux/HDL-C | LIPE |
| --- | --- | --- | --- |
| # of voice users | 24 | 40 | 40 |
| # of data users | 15 | 21 | 21 |
| Voice utilization (%) | 17.78 | 26.4 | 25.72 |
| Data utilization (%) | 8.19 | 9.53 | 9.62 |
| Total utilization (%) | 25.97 | 35.93 | 35.34 |

Table 16. The statistics of 20% voice v. s. 80 % data at 64 Kbps

The 99.9% delay of voice frames is consistent for all pairs within eack protocol stack: from 3.9 ms to 4.0 ms for AAL2/ATM, from 5.1 ms to 5.2 ms for PPPmux/HDL-C, and from 5.08 ms to 5.18 ms for LIPE. Figure 9 shows the knee of 99.9% data frame delay is at 10 voice users/80 data users for AAL2/ATM, at 14 voice users/100 data users for PPPmux/HDL-C,  and at 15 voice users/105 data users for LIPE. Table 17 summarizes the utilisation while the system reaches capacity.

| Protocol stacks | AAL2/ATM | PPPmux/HDL-C | LIPE |
| --- | --- | --- | --- |
| # of voice users | 10 | 14 | 15 |
| # of data users | 80 | 100 | 105 |
| Voice utilization (%) | 7.41 | 9.24 | 9.64 |
| Data utilization (%) | 39.62 | 41.62 | 44.04 |
| Total utilization (%) | 46.67 | 50.86 | 53.68 |

Table 17. The statistics of 20% voice v. s. 80 % data at 144 Kbps

| Pair | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| # of voice users | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 |
| # of data users | 50 | 55 | 60 | 65 | 70 | 75 | 80 | 85 | 90 |

Table 18. The combination of 20% voice and 80% data (2)

Table 18 shows the number of voice users and that of data users corresponding Figure 10 Figure 10 does not show obvious knees for 99.9% data delay curves.

# References

[1] TSGR-IPAdHoc(00)2393, A Simulation Model for Iub IP Based User Plane IP Protocol Stacks, (Motorola).

[2] R. Pazhynnur, I. Ali, Craig Fox, PPP Multiplexed Frame Option:, <draft-ietf-pppext-pppmux-00.txt>, January 5, 2000.

[3] TSG-RAN Working Group 3, TSGR3#15(00)2146.

[4] TS 25.321 V3.2.0, MAC Protocol Specification.