3GPP TSG-RAN WG3 Meeting # Ad-hoc	R3-170009

Spokane, Washington, USA, Jan. 17th – 19th, 2017

Agenda item:		10.6.2.1

Source:	LG Electronics Inc.

Title:	Text Proposal for intra-RAT mobility procedures

Document for:		Discussion

1. Text Proposal

----------------Start of the First Change---------------

10.2.1	Intra-system Mobility

Principle: The LTE X2 handover procedure as in TS 36.300 Figure 10.1.2.1.1-1: Intra-MME/Serving Gateway HO is taken as a baseline for intra-system mobility i.e. intra RAT (gNB <-> gNB; eLTE eNB <-> eLTE eNB) and inter-RAT (eLTE-eNB <-> gNB).

10.2.1.1	Intra-system Intra-RAT mobility scenarios

Scenario: Inter gNB mobility with Xn interface

Figure 10.2.1.1-1: Inter gNB mobility scenario

Editor’s note: Intra gNB mobility scenario is subject to discussion with more progress from RAN2 and RAN3 on functional split and RAN architecture.

10.2.1.1.1	Inter-gNB Handover Variant with in-band Path Switch

The following call flow presents a variant of the inter-gNB handover derived from LTE X2-handover where the path switch is triggered over the user plane for latency optimization. This variant is shown below in an example where two PDU sessions are active towards different UPGWs:

Figure 10.2.1.1.1-1: Intra-new RAN Handover using in-band Path Switch over NG-U

Starting point: The TNL ID is the identifier in the tunnel header over NG-U carrying all the packets for a given UE and a given PDU session and the UPGW @ is the address of the UPGW which has been assigned at PDU session creation.

-	At step 1 during handover preparation gNB2 receives the context information for the UE and the sessions including the tunnel information (UPGW @, TNL ID) for each relocated session then performs handover execution.

-	At step 2, at the end of handover execution gNB2 sends over NG-U for each session a special packet including the TNL ID to inform the UPGW that it now handles the traffic for that session. The UPGW identifies from the TNL ID in the tunnel header the UE and associated PDU session. Reception of this first packet automatically switches the DL path between the gNB2 and the UPGW for this (UE, PDU session).

-	At step 3, The UPGW sends an End Marker packet to gNB1. Receiving the End Marker packet in gNB1 including TNL ID in the tunnel header automatically releases the tunnel between the gNB1 and the UPGW for this (UE, PDU session).

-	At step 4 the gNB may send a gNB Change Request message to inform NG CP of the new gNB ID.

This above described procedure is applicable to both eLTE eNB and gNB.

10.2.1.1.X	Evaluation on CP based path switch solution and in-band path switch solution

In order to better understand the two solutions for intra-RAT mobility, the comparison table is summarized in table x.

Table x. Comparison table between CP based path switch solution and in-band path switch solution

|  | In-band Path Switch Solution | CP based Path Switch Solution |
| --- | --- | --- |
| Latency Optimization | About one message save compared with CP solution | Normal |
| Number of path switch in case of U-GW Relocation needed: target gNB2 point of view | Twice | Once |
| Security | Not guaranteed for the early switched packets | Guaranteed for the switched packets |
| Signaling Reducing | Not clear | Normal |

----------------End of the First Change---------------