3GPP TSG-RAN WG3 Meeting #Ad Hoc	R3-170183

Spokane, Washington, USA, 17 – 19, January, 2017

Agenda Item:	   10.3.2

Source: 	   Ericsson

Title: 	   TP for Clarification on Initial UE Access

Document for:	Discussion and Decision

# Introduction

In R3-170182 it was explained that the description of the Initial UE Access procedure in TR38.801 should be corrected to say that the procedure is used to establish a signalling connection between RAN and CN. This TP for TR38.801 captures the aspects described in that paper.

# Text Proposal to TR38.801

------------------------------------------First Change------------------------------------------

### 10.2.4	Initial UE Access

Following a UE connection request, this is initiated by the New RAN to establish a signalling connection between New RAN and NGC in case of an Idle-to Active transition. The first message from the NGC, following the INITIAL UE MESSAGE, completes the establishment of a signalling connection. With the first reply message the NGC may initiate a UE context setup.

Figure 10.2.4-1: Initial UE Access over NG

-	At step 1 following the reception of the first UL NAS message to be forwarded to NGC, the selection of CN entity is performed.

Editor’s note: The details of the selection of the CN entity is pending SA2 decision

-	At step 2, the New RAN uses the INITIAL UE MESSAGE to deliver the NAS PDU to the CN entity selected in the previous step

Editor’s note: It is still FFS whether INITIAL UE MESSAGE is used for this.

-	At step 3, the NGC completes the UE associated signaling connection setup. The New RAN configures the UE accordingly.

------------------------------------------End of Changes------------------------------------------