3GPP TSG-RAN WG3 NR Adhoc 	R3-170185

# Spokane, Washington, USA, 17th – 19th January 2017

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.801 | CR | CRNum | rev | - | Current version: | 1.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | TP on Mobility Management Procedures |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_NR_newRAT |  |  |  |  |  |  | Date: |  | 2017-01-09 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-14 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) |  |
| Reason for change: |  |  |  |  |  |  |  |  |  |  |
| Summary of change: |  |  |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.2.4, 7.2.4.1.x (New) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

-----------------------------Start of Changes-----------------------------

### 7.2.4	NG Interface Procedures

To support the functions listed in Section 7.2.3 the NG interface should support the following procedures. The procedures are classified in different categories.

Interface management procedures:

-	NG Setup: To establish an NG interface between the New RAN and NGC nodes

-	Configuration Updates from New RAN and NGC nodes: To update configuration of the interface from the New RAN or NGC (FFS)

-	NG Reset: To reset the NG interface

-	Error Indication: To report detected errors in one incoming message

UE Context Management Procedures:

-	Initial Context Setup: To establish the initial UE context both in New RAN and in NGC

-	UE Context Release: To remove UE context information

-	UE Context Modification: To modify an already established UE context (FFS)

UE Mobility Management Procedures:

-	Handover Preparation: Needed to prepare resource allocation from the source RAN to the NGC for a UE handing over

-	Handover Resource Allocation: Needed to reserve resources at the target RAN for a UE handing over

-	Path Switch Request: Needed to request the switch of a UP connection to a different UP GW at the NGC (FFS)

-	Handover Cancel: Needed to cancel an ongoing handover preparation or an already prepared handover (FFS)

-	Handover Notification: Needed to indicate to the NGC that handover has been successfully completed (FFS)

Transport of NAS Messages Procedures:

-	Initial UE Message: To transport the first NAS PDU sent when no UE signaling connection is established from the UE to the NGC (FFS)

-	DL NAS Transport: To transport NAS PDUs in DL

-	UL NAS Transport: To transport NAS PDUs in UL

-	Non Delivery NAS Indication: To indicate that the New RAN node did not deliver a NAS PDU to the UE (FFS)

-	NAS Rerouting: In order to route a NAS request to a different CN node (FFS)

Paging Procedures:

-	Paging: To provide a paging instruction from the NGC to the New RAN

PDU Session Management Procedures:

-	PDU Session Setup: Needed to establish a PDU Session and assign resources to it according to the QoS and other configuration parameters assigned to the flow(s) configured within the session

-	PDU Session Modify: Needed to modify a previously configured PDU session, related QoS flows and respective New RAN resources

-	PDU Session Release: Needed to release a previously configured PDU Session and related New RAN resources

-	PDU Session Modification Indication: Needed to switch a PDU Session from one termination point at the New RAN to a different termination point. (FFS)

-----------------------------Next Change-----------------------------

##### 7.2.4.1.x	UE Mobility Management Procedures

Handover Preparation

Figure 7.2.4.1.x-1: Example of Handover Preparation procedure

The Handover Preparation procedure enables initiation of a handover involving the NGCN. In this procedure the source RAN signals to the NGCN the intention to handover a UE to a given target RAN. The source RAN produces and signals information that should be passed transparently to the target RAN.

In return, the source RAN receives by the NGCN information such as PDU Sessions that cannot be handed over to the target RAN and that should be released. Also, the source RAN receives information that were sent by the target RAN transparently to the NGCN.

Handover Resource Allocation

Figure 7.2.4.1.x-2: Example of Handover Resource Allocation procedure

This procedure is mainly needed to communicate to the target RAN which PDU Sessions should have resources allocated as part of the handover. The target RAN needs this information to run access control and to decide whether resources can be allocated to the PDU Sessions that are requested to be handed over. The other fundamental role of this procedure is to create a UE context at the target RAN for the UE that is handed over. Therefore, the NGCN should signal to the target RAN information that are relevant to establish a UE context.

In return, the target RAN signals to the NGCN information about the PDU Sessions that were admitted and those that failed to be admitted. Additionally, the target RAN signals to the NGCN information that should be transparently conveyed to the source RAN.

-----------------------------End of Changes-----------------------------