3GPP TSG-RAN WG3 Meeting #Ad Hoc	R3-170187

Spokane, Washington, USA, 17 – 19, January, 2017

Agenda Item:	   10.3.2

Source: 	   Ericsson

Title: 	   TP for Clarification on Path Switch

Document for:	Discussion and Decision

# Introduction

In R3-170186 it was explained that the conclusions taken by SA2 in TR23.799 acknowledge the presence of a Path Switch procedure over the NG-C interface between the RAN and the NGC. This TP proposes changes to TR38.801 in order to confirm the support of such procedure

# Text Proposal to TR38.801

------------------------------------------First Change------------------------------------------

### 7.2.4	NG Interface Procedures

------------------------------------------Unchanged Text Skipped------------------------------------------

UE Mobility Management Procedures:

-	Handover Preparation: Needed to prepare resource allocation from the source RAN to the NGC for a UE handing over (FFS)

-	Handover Resource Allocation: Needed to reserve resources at the target RAN for a UE handing over (FFS)

-	Path Switch Request: Needed to request the switch of a UP connection to a different UP GW at the NGC

-	Handover Cancel: Needed to cancel an ongoing handover preparation or an already prepared handover (FFS)

-	Handover Notification: Needed to indicate to the NGC that handover has been successfully completed (FFS)

------------------------------------------Second Change------------------------------------------

#### 7.2.4.1	NG Interface Procedures Descriptions

This section describes NG procedures. The NG interface can terminate in an eLTE eNB or in a gNB. Therefore, NG procedures apply to both an eLTE eNB and a gNB.

------------------------------------------Unchanged Text Skipped------------------------------------------

##### 7.2.4.1.X	UE Mobility Management Procedures

Path Switch

Figure 7.2.4.1.x-8: Example of Path Switch procedure

After an Xn based handover, target RAN and CN will need to exchange signalling over the NG to at least allow the RAN to communicate to NGC DL information for UP data of handed over PDU Sessions.

The RAN and CN may exchange information about the PDU Sessions that were admitted and those that have failed to be admitted at target RAN and that should instead be removed.

------------------------------------------End of Changes------------------------------------------