# 3GPP TSG RAN WG3 Meeting #NR ADHoC	R3-172182

# Qingdao, China, 27th -29th June 2017

Agenda item:		10.10.3

Source:	Huawei

Title:					TP on Mobility procedures to 38.401

Document for:		pCR

1. Introduction

In last RAN3 meeting, it was agreed to capture (with FFS) functions for F1AP. This paper propose to capture texts on mobility procedures to 38.401.

2. Conclusion

This paper proposes capture texts on mobility procedures to 38.401.

Annex – Text proposal 38.401

8.2	Inter-DU Mobility

Editor Note: This subclause shows overall procedures for inter-DU mobility.

Editor Note: The name of this subclaues is FFS.

Basically, UE Context related procedures are used for inter-DU mobility over F1, the following figure illustrates the basic signaling flow.

Figure 8.2-x. Inter-DU mobility within gNB-CU

Below is a more detailed description of the Inter-DU mobility within gNB-CU procedure:

        - CU makes a handover decision to the target cell under another DU under current serving CU, based on measurement results.

        - CU initiates UE Context Setup Request over F1 AP towards the target DU, during which target DU should prepare the resource in the target cell for the incoming UE, admission control in DU might be performed during this process.

        - After resource preparation in target cell is finished, target DU confirms back to CU with UE Context Setup Complete message.

        - After getting confirmation from target DU, the CU sends the handover command to UE using RRC Connection Reconfiguration message.

        - UE detaches from the source cell and accesses to target cell, then confirms the successful completion handover procedure using RRC Connection Reconfiguration Complete message from the target cell.

        - Upon reception of the UE Context Release message, the source DU can release related resources associated to that UE.

During this procedure, lossless handover could be achieved taking advantage of flow control mechanisms or other means between CU and DU.