# 3GPP TSG RAN WG3 Meeting #NR AD-HoC2	R3-172201

# Qingdao, China, 27th -29th June 2017

Agenda item:		10.10.2

Source:	Huawei

Title:					TP on Moving GTP-U management functionality as part of bearer management to 38.401

Document for:		pCR

1. Introduction

In last RAN3 meeting, bearer management was agreed as part of F1 AP functionality, which is on top of GTP-U protocol, thus it is proposed to move GTP-U management functionality as part of bearer management functionality to 38.401.

2. Conclusion

This paper propose to move GTP-U management functionality as part of bearer management functionality in 38.401.

Annex – Text proposal 38.401

10.3.2.2	F1-C functions

10.3.2.2.2	F1 interface management function

The error indication function is used by the gNB-DU or gNB-CU to indicate to the gNB-CU or gNB-DU that an  error has occurred.

The reset function is used to initialize the peer entity after node setup and after a failure event occurred. This procedure can be used by both the gNB-DU and gNB-CU.

The F1 setup (respectively the gNB-DU and gNB-CU configuration update) function allows to exchange (respectively update) application level data needed for the gNB-DU and gNB-CU to interoperate correctly on the F1 interface. The F1 setup is initiated by the gNB-DU (FFS).

Editor’s note: The space of NR Cell ID, and how/whether to map them in the gNB-CU are FFS.

10.3.2.2.3	gNB-DU management function (FFS)

Editor’s note: Whether gNB-DU is managed by O&M or by gNB-CU is FFS. The impact on F1 interface is FFS.

10.3.2.2.4	System Information management function

Scheduling of system broadcast information is carried out in the gNB-DU. The gNB-DU is responsible for transmitting the system information according to the scheduling parameters available.

Editor’s note: How to support this function is FFS.

10.3.2.2.5	gNB-DU and gNB-CU measurement reporting function (FFS)

The gNB-DU and gNB-CU measurement reporting functions are used to report the measurements of gNB-DU and and gNB-CU, respectively.

10.3.2.2.6	Load management function (FFS)

The load management function allows the gNB-CU and gNB-DU to request the reporting of load information to the gNB-DU and gNB-CU, respectively.

10.3.2.2.7	Paging function (FFS)

The gNB-DU is responsible for transmitting the paging information according to the scheduling parameters available.

10.3.2.2.8	F1 UE context management function

The F1 UE context management function supports the establishment of the necessary overall initial UE context.

The establishment of the F1 UE context is initiated by the gNB-CU and accepted or rejected by the gNB-DU based on admission control criteria (e.g., resource not available).

The F1 UE context management function also supports the release of the context previously established in the gNB-DU. The release of the context is triggered by the gNB-CU either directly or following a request received from the gNB-DU.

Editor’s note: What is UE context needs to be clarified.

10.3.2.2.9	Bearer management function

The bearer management function is responsible for triggering establishing, modifying and releasing radio bearer resources for user data transport once an F1 UE context is available in the gNB-DU. The establishment and modification of radio bearer resources are triggered by the gNB-CU and accepted/rejected by the gNB-DU based on  resource reservation information and QoS information to be provided to the gNB-DU.

The mapping between QoS flows and radio bearers is performed by CU and the granularity of bearer management over F1 is radio bearer level.

A bearer service request will trigger the establishment and release of a GTP-U (3GPP TS 29.281 [x3]) tunnel between the gNB-CU and the gNB-DU, during which a tunnel identifier for each direction is assigned.

Editor’s note: Whether mobility functions such as intra-CU/inter-DU handover can be supported by bearer management function is FFS.

Editor’s note: The relationship of bearer management and simultaneous transmission from two DUs is FFS.

Editor’s note: How to split and configure RRM functions between gNB-CU and gNB-DU is FFS.

Editor’s note: How to provide QoS information to gNB-DU is FFS.