3GPP TSG-RAN WG3 Meeting #ad-hoc2	R3-172499

Qingdao, China, 27th – 29th June 2017

Agenda Item:	10.1

Source:	Ericsson

Title:	TP for QoS Parameters

Document for:	Discussions & Approval

# 1	Text Proposal for NGAP TS 38.423

<<<<<<<<<<<<<<<<<<<< Begin Text Proposal>>>>>>>>>>>>>>>>>>>>

---------------------------------

Skip unchanged text

----------------------------------

### 9.2.1	QoS Flow Level Parameters

This IE defines the QoS to be applied to a QoS flow.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| 5QI | M (FFS) |  | INTEGER (0..255) | 5G QoS Indicator defined in TS 23.501 [7]. Logical range and coding specified in TS 23.501 [7]. Editor’s Note: Coding is FFS. Presence is FFS (probably not needed if the Non standardised QoS Flow descriptor is present. Could also be realised as a choice with the Non standardised QoS Flow descriptor.) | – |  |
| Allocation and Retention Priority | M |  | 9.2.y | Note: presence needs to be checked with latest SA2 status. | – |  |
| Non-standardised QoS Flow descriptor | O |  | 9.2.x | Note: presence needs to be checked with latest stage 2 status. | - |  |
| GBR QoS Flow Information | O |  | 9.2.2 | This IE applies to GBR bearers only and shall be ignored otherwise. | – |  |

---------------------------------

Skip unchanged text

----------------------------------

### 9.2.2	GBR QoS Flow Information

This IE indicates the maximum and guaranteed bit rates of a GBR QoS flows for downlink and uplink.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Maximum Flow Bit Rate Downlink | M |  | 9.2.3 | Maximum Bit Rate in DL. Details in TS 23.501 [7]. | – | – |
| Maximum Flow Bit Rate Uplink | M |  | 9.2.3 | Maximum Bit Rate in UL. Details in TS 23.501 [7]. | – | – |
| Guaranteed Flow Bit Rate Downlink | M |  | 9.2.3 | Guaranteed Bit Rate (provided that there is data to deliver) in DL. Details in TS 23.501 [7]. | – | – |
| Guaranteed Flow Bit Rate Uplink | M |  | 9.2.3 | Guaranteed Bit Rate (provided that there is data to deliver). Details in TS 23.501 [7]. | – | – |
| Notification Control | O |  | ENUMERATED {notification requested, ...} | Editor’s Note: Details to be checked with SA2 progress. | - |  |

### 9.2.3	Bit Rate

This IE indicates the number of bits delivered by NG RAN in UL or to NG RAN in DL within a period of time, divided by the duration of the period. It is used, for example, to indicate the maximum or guaranteed bit rate for a GBR QoS flow, or an aggregated maximum bit rate.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Bit Rate | M |  | INTEGER (0..<tbd>) | The unit is: bit/s Editor’s Note: Range is FFS |

---------------------------------

Skip unchanged text

----------------------------------

#### 9.2.x	Non-standardised QoS Flow descriptor

This IE indicates the QoS parameters for a Non-standardised QoS flows for downlink and uplink.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Allocation/Retention Priority |  |  |  |  |
| > Resource Type | M |  | ENUMERATED(GBR, Non-GBR) |  |
| > Priority level | M |  | <reference> | Note: Details are FFS |
| > Packet Delay Budget | M |  | <reference> | Note: Details are FFS |
| > Packet Error Rate | M |  | <reference> | Note: Details are FFS |
| > Averaging window | M |  | <reference> | Note: Details are FFS |

#### 9.2.y	Allocation and Retention Priority

This IE specifies the relative importance of a QoS flow compared to other QoS flows E-RABs for allocation and retention of NG RAN resources.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Allocation/Retention Priority |  |  |  |  |
| >Priority Level | M |  | INTEGER (1..15) | Desc.: This IE defines the defines the relative importance of a resource request (see TS 23.501 [7]). Usage: Values are ordered in decreasing order of priority, i.e., with 1 as the highest priority and 15 as the lowest priority. Further usage is defined in TS 23.501 [7]. |
| >Pre-emption Capability | M |  | ENUMERATED(shall not trigger pre-emption, may trigger pre-emption) | Desc.: This IE indicates the pre-emption capability of the request on other QoS flows Usage: The QoS flow shall not pre-empt other QoS flows or, the QoS flow may pre-empt other QoS flows. The Pre-emption Capability indicator applies to the allocation of resources for a QoS flow and as such it provides the trigger to the pre-emption procedures/processes of the NG RAN node. |
| >Pre-emption Vulnerability | M |  | ENUMERATED(not pre-emptable, pre-emptable) | Desc.: This IE indicates the vulnerability of the QoS flow to preemption of other QoS flows. Usage: The QoS flow shall not be pre-empted by other QoS flows or the QoS flow may be pre-empted by other QoS flows. Pre-emption Vulnerability indicator applies for the entire duration of the QoS flow, unless modified and as such indicates whether the QoS flow is a target of the pre-emption procedures/processes of the NG RAN node. |

<<<<<<<<<<<<<<<<<<<< End Text Proposal  >>>>>>>>>>>>>>>>>>>>