3GPP TSG-RAN WG3 Meeting #ad-hoc2	R3-172504

Qingdao, P.R. China, 27th – 29th June 2017

Agenda Item:	10.1

Source:	Ericsson

Title:	PDU session vs PDU session resource

Document for:	Discussions & Approval

# 1	Background

At the last RAN3#96 meeting, we have discussed if to use PDU session or PDU session resource in PDU session management procedure without reaching any conclusion. The TS 38.413 at the moment states this matter is subject for further study:

## 8.2	PDU Session Management Procedures

Editor’s Note:	The message names for PDU Session Management may need further review.

Editor’s Note:	It is FFS whether to name the PDU session/QoS flow as PDU session resource/QoS flow resource or other names in the descriptions and IE definitions.

## 8.2	PDU Session Management Procedures

Editor’s Note:	The message names for PDU Session Management may need further review.

Editor’s Note:	It is FFS whether to name the PDU session/QoS flow as PDU session resource/QoS flow resource or other names in the descriptions and IE definitions.

# 2	Discussion

First some basics: TS 23.501 has defined that:

PDU Session: Association between the UE and a Data Network that provides a PDU connectivity service. The type of association can be IP, Ethernet or unstructured.

Further Table 5.6.1-1 defines the Attributes of a PDU session

| PDU session attribute | May be modified later during the lifetime of the PDU session | Notes |
| --- | --- | --- |
| Slicing information | No | (Note 1)(Note 2) |
| DNN (Data Network Name) | No | (Note 1)(Note 2) |
| PDU session Type | No | (Note 1) |
| SSC mode | No | (Note 1) The semantics of Service and Session Continuity mode is defined in clause 5.6.9.2 |
| PDU session Id | No |  |
| NOTE 1: If it is not provided by the UE, the network determines the parameter based on default information received in user subscription. Subscription to different DNN(s) may correspond to different default SSC modes and different default PDU session Types NOTE 2: Slicing information and DNN are used by AMF to select a SMF to handle a new session. Refer to clause 5.2. |  |  |

Obviously PDU session is a logic connection between UE and the Data Network and it is in the NAS scope.

PDU Session Resource on the other hand, is the resource that RAN allocates to implement/represent the PDU session. It is in AS scope.

At the last RAN3 meeting we have discussed if it is enough to use “PDU Session Resource” in the procedure and message names, but to use PDU session in the descriptions and IE definitions.

Note in NR the PDU session is established between UE and SMF. It is not between UE and AMF. Thus in NGAP, we will see IEs related to PDU session resource (between AMF and gNB), NAS PDU and PDU sessions (between SMF and gNB).

In our opinion, in the PDU session resource related procedures and messages, when the description and IE definition are related to “AMF and gNB”, we should use “PDU session resource”, when the description and IE definition are related to “SMF and gNB”, we should use “PDU session”.

However there is an exception when it comes to PDU Session ID IE in e.g. the PDU Session Resource Setup List IE in PDU SESSION RESOURCE SETUP REQUEST.

In TS 38.413, chapter 9.3.1.5	PDU Session Resource Setup List

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| PDU Session Resource Setup List |  | 1 |  |  | YES | reject |
| >PDU Session Resource Setup Item IEs |  | 1 .. <maxnoof PDUSessionResources> |  |  | EACH | reject |
| >>PDU Session ID [FFS] | M |  | <ref> |  | - |  |
| >>S-NSSAI | O |  | <ref> |  | - |  |
| >>PDU Session Setup Request Transfer | O |  | <ref> | This IE includes a container from the SMF to the RAN node transparently to the AMF as per section <ref>. | - |  |

Here we use “PDU Session ID” not “PDU Session Resource ID” as it is the AMF who selects the SMF functions for PDU sessions and the “PDU session ID” is known/assigned by AMF, as per TS 23.501. Thus there is no need to introduce a “PDU Session Resource ID”.

Proposal 1: RAN3 to keep the current wording of PDU session resource in the related NGAP description and IE definition and remove the corresponding FFS.

Proposal 2: RAN3 to remove the FFS related to PDU Session ID.

# 3	Proposals

Proposal 1: RAN3 to keep the current wording of PDU session resource in the related NGAP description and IE definition and remove the corresponding FFS.

Proposal 2: RAN3 to remove the FFS related to PDU Session ID.

Text proposal in [1] and [2].

# 4	Reference

- [1]	R3-172505	TP of PDU session resource, Ericsson

- [2]	R3-172506	TP of PDU session resource, Ericsson